3GPP TSG-RAN WG4 Meeting #110	R4-2402808

Athens, GR, Feb 26th – Mar 01st, 2024

Agenda item:	8.8.2

Source: 	Qualcomm Incorporated

Title: 	RRM performance test cases for FR2 HST enhancements

Document for:	Discussion

# Introduction

RAN4 completed the RRM core requirements for NR_HST_FR2_enh WI in the last meeting and had a preliminary discussion on the performance part [1].

In this contribution, we provide our views on the necessity of some of the test cases for RRM.

# RRM performance test cases

Here’s a indicative list of the test cases to be considered for this WI:

| TC index | Corresponding Core Requirement | Necessity of New Test Case |
| --- | --- | --- |
| TC1 | Requirements for Cell Re-selection | New test case needed. (New test case for A.7.1.1.X Cell reselection to FR2 inter-frequency NR case for Rel-18 FR2 HST intra-band CA) FFS, how to define the new TC |
| TC2 | Requirements for SCell Activation Delay for Deactivated SCell | FFS, whether the new test case is needed/reuse the existed one |
| TC3-TC6 | Requirements for Inter-frequency measurement with measurement gaps | FFS, whether the new test case is needed. (If new TC needed. New test case for A.7.6.2.X1 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA with SSB time index detection when DRX is used (Pcell in FR2)) |
|  |  | FFS, whether the new test case is needed. (If new TC needed. New test case for A.7.6.2.X2 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA without SSB time index detection when DRX is used (Pcell in FR2)) |
|  |  | New test case needed. (New test case for A.7.6.2.X3 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA with SSB time index detection when DRX is not used (Pcell in FR2)) |
|  |  | New test case needed. (New test case for A.7.6.2.X4 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA without SSB time index detection when DRX is not used (Pcell in FR2)) |
| TC7 | Requirements for Inter-frequency measurement without measurement gaps | No new test case needed. |
| TC8 | Requirements for One shot large UL timing adjustment for FR2 Power Class 6 UE | New test case(s) needed. FFS, how to define the new TC(s) FFS, MRTD requirement verification |
|  | Requirements for MAC-CE based TCI state switch delay in HST FR2 scenarios |  |
|  | New requirements for MRTD with simultaneous two-panel reception for FR2 Power Class 6 UE |  |
| TC9 | Requirements for SSB based radio link monitoring | No new test case needed. |
| TC10 | Requirements for SSB based beam failure detection | No new test case needed. |
| TC11 | Requirements for SSB based L1-RSRP | New test case needed. (New test case for A.7.6.3.X SSB based L1-RSRP measurement when DRX is used for FR2-1 PC6 UEs supporting SimultaneousReceptionFR2HST-r18) |
| TC12 | Requirements for Intra-frequency measurements with/without measurement gaps | FFS, whether the new test case is needed. (If new TC needed. New test case for A.7.6.1.X SA event triggered reporting test without gap under non-DRX for UE supporting [measurementEnhancementCAInterFreqFR2-r18]) |

Test Cases for SCell activation delay

We think that deactivating an SCell is not going to be a typical scenario in FR2 HST. Furthermore, we don’t have any SCell activation test cases for FR1 HST, so we think we can skip SCell activation tests for FR2 HST as well.

Proposal 1: Do not introduce new test on SCell activation delay for R18 FR2 HST UEs.

Test cases on Uplink timing adjustment

Two main issues on uplink timing adjustment tests that were discussed during the last meeting were:

- Whether to introduce simultaneous multi-panel operation with UL timing adjustment in R18 FR2 HST

- Whether to define the test case combining the UL timing adjustment and TCI state switch

On 1), we think that simultaneous multi-panel operation and UL timing adjustment are two separate features for R18 FR2 HST UEs and a UE may not support both the features. In general, we believe it’s not a good idea to combine multiple features in the test cases to keep the test complexity low. So we prefer to not introduce simultaneous multi-panel operation in the UL timing adjustment test cases.

Proposal 2: Do not introduce simultaneous multi-panel operation in the UL timing adjustment test cases.

On 2), the new MAC-CE indication affects two UE requirements – one shot timing adjustment and TCI state switching delay. Although both the requirements are tied to a single indication, it would be better to not complicate test cases by testing both the behaviours in a single test case. We prefer to have separate test cases to test the UE behaviour for one-shot timing adjustment and TCI state switching delay upon receiving the MAC-CE indication.

Proposal 3: Introduce separate test cases to test the UE behaviour for one-shot timing adjustment and TCI state switching delay upon receiving the MAC-CE indicating 1 or 0.

# Conclusion

Proposal 1: Do not introduce new test on SCell activation delay for R18 FR2 HST UEs.

Proposal 2: Do not introduce simultaneous multi-panel operation in the UL timing adjustment test cases.

Proposal 3: Introduce separate test cases to test the UE behaviour for one-shot timing adjustment and TCI state switching delay upon receiving the MAC-CE indicating 1 or 0.

# References

[1] R4-2321349, “WF on FR2 HST RRM performance requirements”, Samsung, RAN4#109