3GPP TSG-RAN WG4 Meeting RAN4#110      				R4-2402823 Athens, Greece

February 26 – March 1, 2024

Agenda Item:	8.12.2

Source:	Lenovo, Motorola Mobility

Title:	Effect of Random Phase Offset on UL MIMO

Document for:  Approval

- Introduction

In previous meetings, there have many contributions on how to evaluate the UE TRP for two antenna port UL-MIMO including [1-5].  The proposals have included the following approaches for measuring the transmit power

- Measuring the power when transmitting with each of the four precoders and taking the average[1].

- Measuring the power when transmitting with each of the four precoders and taking the maximum [2-4].

One of the issues that has been addressed is the phase variation between the two antennas in [5].  However, there are two aspects to this issue, only one of which has been addressed.

- The phase variation in the time interval between the most recent SRS transmission and the time the received power is measured.

- The value of the phase offset between the two antennas.

In this contribution, we consider the second issue, and in particular, the effect of this random phase offset on the performance of the codebook for both two and four transmit antennas. Due to the random phase offset between the antennas, the power measured in a particular direction using the best precoder is a random variable that depends on the value of the random phase offset.

- Impact of Random Phase Offset

The requirement for coherent uplink MIMO is given by the following from [6]:

For coherent UL MIMO, Table 6.4D.4-1 lists the maximum allowable difference between the measured relative power and phase errors between different antenna connectors in any slot within the specified time window from the last transmitted SRS on the same antenna connectors, for the purpose of uplink transmission (codebook or non-codebook usage) and those measured at that last SRS. The requirements in Table 6.4D.4-1 apply when the UL transmission power at each antenna connector is larger than 0 dBm for SRS transmission and for the duration of time window.

Table 6.4D.4-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted

| Difference of relative phase error | Difference of relative power error | Time window |
| --- | --- | --- |
| 40 degrees | 4 dB | 20 msec |

  - Two Transmit Antennas

It can be noted that while the relative phase change is limited, there is no requirement on the relative phase offset so that this phase is unknown and can be any value θ. As a result, the application of the precoders can be modelled as in Figure 1 below.

Figure 1:  Random phase offset between the two UE transmit antennas

Due to the random phase offset, the single layer MIMO precoders with TPMI indices 2-5 in Table 6.3.1.5-1 of [7] are transformed into the following

1211 ejθ,  121-1 ejθ,  121j ejθ,  121-j ejθ                                     (1)

where the second weight w1 of each precoder is multiplied by ejθ.

While the coherence requirement limits the rate of change of θ to 40 degrees every 20 msec, the rate of change can be much slower.  Since this phase offset is unspecified and its value and its rate of change are unknown, it would be difficult to ensure that the effect of the random phase offset is properly captured by measurement.

Due to the random phase offsets between the two transmit antennas, the power transmitted in a particular azimuth and elevation using the best precoder is a random variable with a distribution depending on the distribution of the random phase offset.

Observation 1:	The power Pφ,ϕ transmitted in a particular azimuth and elevation with a best precoder within a set of precoders is a random variable that depends on the distribution of the phase offset θ between the two transmit antennas.

To evaluate the performance of the precoders, the radiated power can be averaged over the distribution of the phase θ.  Alternatively, the performance of the precoders can be evaluated for the worst-case phase θ. Neither of these approaches can be implemented via measurement as there is no way to measure or control the phase offset between the antennas in a radiated test. However, the average and worst-case performance of the precoders over the phase θ can be evaluated analytically using the per antenna power measurements.

Let P1 and P2 denote the received power Pφ,ϕ for elevation φ and azimuth ϕ received from the first and second antennas when transmitting separately.  These two received signals have phases ψ1 and ψ2 (ψ2 is the phase without the offset θ).  To determine the power received both antennas transmit simultaneously with a particular precoder, the phases ψ1 and ψ2 and the phase offset θ must be known.  For example, with the use of the precoder with TPMI index 2 given by 1211 ejθT, the power of the received signal would be given by

P=P1ej ψ1+P2ej ψ2+θ2,

and this depends on ψ1 and ψ2, and the relative phase offset θ through the phase difference

ψ2-ψ1+θ.

Since the power depends on the relative phase offset θ, the optimal precoder will also depend on θ.  Since θ is unspecified and unknown, the radiated power can be determined in one of two ways:

- As the power of the best precoder for each θ, averaged over θ over the interval [0, 2π).

- As the power of the best precoder for each θ, for the worst-case phase θ over the interval [0, 2π)

Because the first method averages the power of the best precoder as a function of the relative phase θ on the interval [0, 2π), the result is independent of the initial phase difference ψ2-ψ1.  For the second approach, assume that θmin is the phase that results in the minimum power for the best precoder when ψ2-ψ1=0. If ψ2-ψ1≠0, then the phase θmin-ψ2-ψ1 will yield the same power for the same precoder that produced the minimum power for ψ2-ψ1=0.  As a result, it can be assumed that ψ2-ψ1=0 for both approaches i) and ii).

The average of the maximum power over the set of precoders in (1) is given by the expression

Pave, max=12π02πmaxP1+P2ejθ2, P1-P2ejθ2, P1+j P2ejθ2,P1-j P2ejθ2 dθ.                                                                                                         (2)

In (2), it can be noted that the measurements P1 and P2 already reflect the scaling factor of 12 corresponding to the precoders with TPMI indices 0 and 1. It is shown in the Appendix that

Pave, max=P1+P2+4 2 π P1P2 .

The maximum power over θ over the set of precoders in (1) is given trivially by setting θ=0, so that

Pmax, max=maxθmaxP1+P2ejθ2, P1-P2ejθ2, P1+j P2ejθ2,P1-j P2ejθ2

=P1+P22=P1+P2+2P1P2

and this corresponds to perfect phase alignment of the two signals. The difference between this maximum power for the phase-aligned case and the average of the maximum is given by

P1+P2+2P1P2-P1+P2+4 2 π P1P2

=21-2 2 π P1P2≈0.2 P1P2 .

In the case that P1=P2=P, the power with exact phase alignment is 4 P while the power of the best precoder averaged over phase θ is given by 3.8 P.  So, for two antenna elements with P1=P2=P the difference due to averaging over the relative phase θ is 0.22 dB.

The power of the best precoder for the worst-case phase is given by

Pmin, max=minθ maxP1+P2ejθ2, P1-P2ejθ2, P1+j P2ejθ2,P1-j P2ejθ2 .

It is shown in the Appendix that

Pmin, max =P1+P2+2 P1P2

For the case that P1=P2=P, the power with exact phase alignment is 4 P while the power of the best precoder for the worst-case relative phase θ is given by 3.4 P, which is a difference of 0.7 dB.

In summary, we have

Pmax, max=P1+P2+2 P1P2

Pave, max=P1+P2+4 2 π P1P2

and

Pmin, max=P1+P2+2 P1P2 .

The complementary cumulative distribution function of the phase alignment error is shown in Figure 2 for the case that power received from the two antennas transmitting separately is equal so that P1=P2.  From this figure, it can be observed that the power corresponding to the best precoder is quite insensitive to the phase offset θ.  Furthermore, it should be noted that the phase alignment loss is greatest when P1=P2 for the given azimuth and elevation, but will generally be significantly less.

Observation 2:	The power of the best precoder averaged over the random phase offset is given by

Pave, max=P1+P2+4 2 π P1P2 ,

where P1 and P2 denote the received power measured when the antennas transmit separately. Pave, max is at most 0.22 dB worse than perfect phase alignment.

Observation 3:	The power of the best precoder for the worst-case phase is given by

Pmin, max=P1+P2+2 P1P2 ,

where P1 and P2 denote the received power measured when the antennas transmit separately. Pmin, max is at most 0.7 dB worse than perfect phase alignment.

Proposal 1:  	For single-layer uplink MIMO with two antennas, the radiated power Pφ,ϕ for elevation φ and azimuth ϕ can be defined as the average power of the best precoder given by

Pave, max=P1+P2+4 2 π P1P2 ,

where P1 and P2 denote the received power measured when the antennas transmit separately.

Figure 2:  Distribution of Loss Due to Phase Alignment Error with the Coherent 2 Antenna Port Precoders in Table 6.3.1.5-1 in [7].

  - Four Transmit Antennas

# If the two-antenna coherent precoders are used are used to define TRP for devices with two transmit antennas, it seems reasonable to consider using the four-antenna coherent precoders to define TRP for devices with four transmit antennas. For four transmit antennas, the second, third and fourth antennas each have a random phase offset with respect to the first antenna, as shown in Figure 3.

Figure 3:  Three random phase offsets between the four transmit antennas

As the number of antennas increases from 2 to 4, the number of single-layer coherent precoders increases from 4 to 16. As a result, measuring the power for all 16 precoders at each azimuth and elevation is still possible but may be significantly more time consuming.  Similarly, as in the case of two transmit antennas, the measurements will not address the randomness of the phase offsets between the antennas, but will instead only be a snapshot of these phase offsets at one point in time. A similar approach can be used as for the case of two antennas, but it is more complicated given that there are more precoders and also that there are now three random phase offsets rather than one.

The set of sixteen single-layer coherent precoders for four antenna ports is given in in Table 6.3.1.5-2 of [7]. With random phase offsets, the precoders with TPMI indices 12-15 in Table 6.3.1.5-2 become

121ejθ1ejθ2-ejθ3 , 121ejθ1j ejθ2j ejθ3 , 121ejθ1-ejθ2ejθ3 , 121ejθ1-j ejθ2-j ejθ3 .

In the same manner as with the two transmit antennas, the power can be measured for the four transmit antennas transmitting separately.  Let P1, P2, P3, and P4 note the received power Pφ,ϕ for elevation φ and azimuth ϕ received from each of the antennas transmitting one at a time. It can be noted that the single antenna power measurements already reflect the scaling factor of 12 corresponding to the precoders with TPMI indices 0, 1, 2, and 3, so this scaling factor is not included in the expressions below.  In the same manner as for two transmit antennas, Pmax, max, Pave, max, and Pmin, max can be defined for the set of 16 coherent precoders in Table 6.3.1.5-2 [7] with TPMI indices 12-27.

Applying this same random phasing to all of the precoders, the average of the maximum power over the set of precoders can be expressed as

Pave, max=12π302π02π02πmax12≤i≤27ai,1P1+ai,2P2ejθ1+ai,3P3ejθ2        3+ai,4P4ejθ32dθ1dθ2dθ3 ,

where the precoder coefficients  ai,j:12≤i≤27,  1≤i≤4 for precoders with TPMI indices 12-27 from Table 6.3.1.5-2 in [7] are in Table 1 below:

Table 1:  Precoder coefficients (without scaling factor of12) from Table 6.3.1.5-2 in [7] for TPMI indices 12-27

The power of the best precoder over the set of precoders for the worst-case phase can be expressed as

Pmin, max=minθ1,θ2,θ3max12≤i≤27ai,1P1+ai,2P2ejθ1+ai,3P3ejθ2+ai,4P4ejθ32 .     (4)

The maximum power of the best precoder over the set of precoders is given by

Pmax, max=maxθ1,θ2,θ3max12≤i≤27ai,1P1+ai,2P2ejθ1+ai,3P3ejθ2+ai,4P4ejθ32

and is shown in the Appendix to be given by

Pmax, max=P1+P2+P3+P42

The expressions in (2) and (3) can both be evaluated numerically.  In the special case in which

P1=P2=P3=P4=P,

we have

Pmax, max=16 P2,

Pave, max=12.7 P2 ,

and

Pmin, max=8 P2 .

From the above, it can be observed that depending on the state of the random phase offsets between the antennas, the power of the best precoder can vary by 3 dB over the set of precoders, depending on the random phase offsets between the antennas.  This can be compared with the case of two antennas where the power of the best precoder varies by at most 0.7 dB as a function of the random phase offset. The complementary cumulative distribution of the beamforming gain relative to the maximum achievable gain is shown in Figure 4.

Figure 4:  Distribution of Loss Due to Phase Alignment Error with Coherent 4 Antenna Port Precoders in TS 38.211 - Table 6.3.1.5-2

Observation 4:	For four antennas, the power Pφ,ϕ transmitted in a particular azimuth and elevation with a best precoder within a set of precoders is a random variable that depends on the distribution of the phase offsets θ1, θ2, and θ3 between the transmit antennas.

Observation 5:	For four antennas, the power for the best precoder over the set of precoders can vary by 3 dB and depends on the values of the random phase offsets θ1, θ2, and θ3 between the antennas.

Observation 6:	Because the power variation due to the variations of the random phase offsets of the best precoder over the set of precoders can be so large, there is some risk in defining the power as the power for the best precoder at a single snapshot in time.

Proposal 2:	It is proposed that the power P1 and P2 for the first and second antennas be measured when transmitting separately, and that the radiated power Pθ,ϕ for elevation θ and azimuth ϕ be defined as the average power for the best precoder given by

Pave, max=12π302π02π02πmax12≤i≤27ai,1P1+ai,2P2ejθ1+ai,3P3ejθ2        +ai,4P4ejθ32dθ1dθ2dθ3,

which can be evaluated numerically.

- Summary

In this contribution, we have considered the impact of the random phase offsets between the antennas on transmitted power for single layer MIMO.  In particular, we have shown that the power Pφ,ϕ transmitted in a particular azimuth and elevation with a particular precoder is a random variable which depends on the values of the random phase offsets of the transmit antennas with respect to each other.

For two antennas, it was shown that the variation of the power of the best precoder over the random phase offset was approximately 0.7 dB. For four antennas, it was shown that the variation of the power of the best precoder over the random phase offsets was 3 dB.  As a result of this variation, it is somewhat risky to define radiated power as the measured power for the best precoder in a particular azimuth and elevation since this is only a snapshot in time.  In this contribution, we propose that the power be defined in terms of the power of the best precoder averaged over the random phase offsets.

For two transmit antennas:

Observation 1:	The power Pθ,ϕ transmitted in a particular azimuth and elevation with a best preocder within a set of precoders is a random variable that depends on the distribution of the phase offset θ between the two transmit antennas.

Observation 2:	The power of the best precoder averaged over the random phase offset is given by

Pave, max=P1+P2+4 2 π P1P2 ,

where P1 and P2 denote the received power measured when the antennas transmit separately. Pave, max is at most 0.22 dB worse than perfect phase alignment.

Observation 3:	The power of the best precoder for the worst-case phase is given by

Pmin, max=P1+P2+2 P1P2 ,

where P1 and P2 denote the received power measured when the antennas transmit separately. Pmin, max is at most 0.7 dB worse than perfect phase alignment.

Proposal 1:  	For single-layer uplink MIMO with two antennas, the radiated power Pθ,ϕ for elevation θ and azimuth ϕ can be defined as the average power of the best precoder given by

Pave, max=P1+P2+4 2 π P1P2 ,

where P1 and P2 denote the received power measured when the antennas transmit separately.

For four transmit antennas:

Observation 4:	For four antennas, the power Pθ,ϕ transmitted in a particular azimuth and elevation with a best precoder within a set of precoders is a random variable that depends on the distribution of the phase offsets θ1, θ2, and θ3 between the transmit antennas.

Observation 5:	For four antennas, the power for the best precoder over the set of precoders can vary by 3 dB and depends on the values of the random phase offsets θ1, θ2, and θ3 between the antennas.

Observation 6:	Because the power variation due to the variations of the random phase offsets of the best precoder over the set of precoders can be so large, there is some risk in defining the power as the power for the best precoder at a single snapshot in time.

Proposal 2:	It is proposed that the power P1 and P2 for the first and second antennas be measured when transmitting separately, and that the radiated power Pθ,ϕ for elevation θ and azimuth ϕ be defined as the average power for the best precoder given by

Pave, max=12π302π02π02πmax12≤i≤27ai,1P1+ai,2P2ejθ1+ai,3P3ejθ2        +ai,4P4ejθ32dθ1dθ2dθ3,

which can be evaluated numerically.

# References

- R4-2317004, “WF on NR_FR1_TRP_TRS_enh”, vivo, 3GPP RAN4 #108bis

- R4-2315382, “On multi-Tx radiated output power methodologies”, Apple, 3GPP RAN4 #108bis

- R4-2311227, “UL MIMO radiated output power metric and test methodology,” Apple, 3GPP RAN4 #108, August 2023

- R4-2318432, “On UL MIMO test methodology,” Apple, Vodafone, T-Mobile USA, 3GPP RAN4#109

- R4-2318105, “On TxD phase drift and number of receive chains,” Huawei, HiSilicon, 3GPP RAN4#109.

- 3GPP TS 38.101-1 User Equipment radio transmission and reception; Part 1: Range1 Standalone

- 3GPP TS 38.214 NR; Physical layer procedures for data

Appendix:  Received Power for Best Precoder as a Function of Phase Offset Between the Transmit Antennas

A1:  Two Transmit Antennas

Average of the Maximum Pave, max

Pave, max=12π02πmaxP1+P2ejθ2, P1-P2ejθ2, P1+j P2ejθ2,P1-j P2ejθ2 dθ.                                                                                                         (A1)

Without loss of generality, assume that P1≥P2 and let γ=P2P1, then

Pave, max=P12π02πmax1+γejθ2, 1-γejθ2, 1+j γejθ2,1-j γejθ2 dθ.

=P12π02πmax1+γ2+2γ cosθ, 1+γ2-2γ cosθ,1+γ2-2γ sinθ,1+γ2+2γ sinθ dθ

=P11+γ2+P1 γ π02πmaxcosθ,-cosθ,-sinθ,sinθ dθ

=P11+γ2+P1 γ π-π4π4cosθ dθ-3π45π4cosθ dθ-5π47π4sinθ dθ+π43π4sinθ dθ

=P11+γ2+P1 γ π 42

=P1+P2+4 2 π P1P2 .

Maximum of the Maximum Pmax, max (perfect phase alignment)

The max power of the best precoder over the set of precoders in (1) is given by

Pmax, max=maxθ maxP1+P2ejθ2, P1-P2ejθ2, P1+j P2ejθ2,P1-j P2ejθ2 .

This expression can be maximized by setting θ=0, so that

Pmax, max=P1+P22=P1+P2+2P1 P2

Minimum of the Maximum Pmin, max

The minimum power of the best precoder over the set of precoders is given by

Pmin, max=minθ maxP1+P2ejθ2, P1-P2ejθ2, P1+j P2ejθ2,P1-j P2ejθ2

=P1minθmax1+γ2+2γ cosθ, 1+γ2-2γ cosθ,1+γ2-2γ sinθ,1+γ2+2γ sinθ

=P11+γ2minθmax2γ cosθ, -2γ cosθ,-2γ sinθ,2γ sinθ

=P11+γ2+2 γ

=P1+P2+2 P1P2 .

A2:   Four Transmit Antennas

Average of the Maximum Pave, max

Applying this same random phasing to all of the precoders, the average of the maximum can be expressed as

Pave, max=12π302π02π02πmax12≤i≤27ai,1P1+ai,2P2ejθ1+ai,3P3ejθ2        (A2)+ai,4P4ejθ32dθ1dθ2dθ3

where the precoder coefficients  ai,j:12≤i≤27,  1≤i≤4 for precoders with TPMI indices 12-27 from Table 6.3.1.5-2 in [7] are in Table A1 below:

Table A1:  Precoder coefficients from Table 6.3.1.5-2 in [7] for TPMI indices 12-27

The squared term in the integrand of (3) can be expressed as

ai,1P1+ai,2P2ejθ1+ai,3P3ejθ2+ai,4P4ejθ32=P1+P2+P3+P4+2 P1P2 Reai,1ai,2*e-jθ1+2 P1P3 Reai,1ai,3*e-jθ2+2 P1P4 Reai,1ai,4*e-jθ3+2 P2P3 Reai,2ai,3*ejθ2-θ3+2 P2P4 Reai,2ai,4*ejθ2-θ4+2 P3P4 Reai,3ai,4*ejθ3-θ4

It can be noted that

Reai,jai,k*ejθ=cosθai,jai,k*=1-sinθai,jai,k*=j-cosθsinθai,jai,k*=-1ai,jai,k*=-j

In the simplest case in which P1=P2=P3=P4=P, the squared term simplifies as

ai,1P1+ai,2P2ejθ1+ai,3P3ejθ2+ai,4P4ejθ32=4 P+2 P2Reai,1ai,2*e-jθ1+Reai,1ai,3*e-jθ2+Reai,1ai,4*e-jθ3+Reai,2ai,3*ejθ2-θ3+Reai,2ai,4*ejθ2-θ4+Reai,3ai,4*ejθ3-θ4

In general, the expression in (A2) can be evaluated numerically.

Maximum of the Maximum Pmax, max (perfect phase alignment)

The maximum power of the best precoder over the set of precoders is given by

Pmax, max=maxθ1,θ2,θ3max12≤i≤27ai,1P1+ai,2P2ejθ1+ai,3P3ejθ2+ai,4P4ejθ32

This expression can be maximized by setting i=12 so that a1,1=a1,2=a1,3=1 and a1,4=-1 and setting θ1=θ2=0 and θ3=π, so that

ai,1P1+ai,2P2ejθ1+ai,3P3ejθ2+ai,4P4ejθ32=P1+P2+P3+P42

Minimum of the Maximum Pmin, max

The minimum power of the best precoder over the set of precoders in (1) is given by

Pmin, max=minθ1,θ2,θ3max12≤i≤27ai,1P1+ai,2P2ejθ1+ai,3P3ejθ2+ai,4P4ejθ32     (3)

which must be evaluated numerically.