3GPP TSG-RAN WG4 Meeting #110	R4-2402857

Athens, Greece, Feb 26th – Mar 1st, 2024

Title: 	WF for UE 8Rx performance requirements

Agenda Item:	8.1.4

Source: 	Huawei, HiSilicon

Document for:	Approval

1 Single carrier requirements

Issue 1-1: UE feature definition

- Agreement

  - Define two optional without UE capability signalling UE feature for the support of different 8Rx receiver types

    - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector

    - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors

Issue 1-2: Rank2 test cases

- Agreement

  - Define two sets of performance requirements as per UE support of different 8Rx receiver type

    - Set#1 requirement: Baseline SU-MIMO 8Rx receiver

    - Set#2 requirement: Simplified SU-MIMO 8Rx receiver

Issue 1-3: Rank4 test cases

- Agreement

  - Define two sets of performance requirements as per UE support of different 8Rx receiver type

    - Set#1 requirement: Baseline SU-MIMO 8Rx receiver

    - Set#2 requirement: Simplified SU-MIMO 8Rx receiver

Issue 1-4: Rank8 test cases

- Agreement

  - Define one set of performance requirements as per UE support of baseline SU-MIMO 8Rx receiver

Issue 1-5: How to define the requirements for Rank 2,4,8

- Agreement

  - Use the derivation methodology used from NR UE Rel-15 requirements for the final SNR requirements, i.e. max span 2.5dB and with additional 0.8dB margin

  - Capture the SNR values with square bracket as per the simulation summary for this meeting, interesting companies can bring simulation results for both 8Rx receiver types for next meeting

2 CA requirements

Issue 2-1: How to define requirements for CC with Rank2

- Agreement

  - Define two set of performance requirements as per UE support of different 8Rx receiver type

    - Set#1 requirement: Baseline SU-MIMO 8Rx receiver

    - Set#2 requirement: Simplified SU-MIMO 8Rx receiver

Issue 2-2: How to define requirements for CC with Rank8

- Agreement

  - Define one set of performance requirements as per UE support of baseline SU-MIMO 8Rx receiver

Issue 2-3: Antenna correlation configuration for Rank2 CA test

- Agreement

  - Keep TDLC300-100 ULA Medium B ( = 0.3,  = 0.005154) that is same as Rank 2 single carrier test

Issue 2-4: MCS for Rank2 CA test

- Agreement

  - Keep MCS 19 that is same as Rank 2 single carrier test

Issue 2-5: How to define the requirements for 8Rx CA

- Agreement

  - Use the derivation methodology used from NR UE Rel-15 requirements for the final SNR requirements, i.e. max span 2.5dB and with additional 0.8dB margin

  - Capture the SNR values with square bracket as per the simulation summary for this meeting, interesting companies can bring simulation results for both 8Rx receiver types for Rank2 test cases for next meeting

Issue 2-6: Number of HARQ processes

- Agreement

| - HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | For CC with Rank 2: 10 For CC with Rank 8: 8 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

Issue 2-7: K1 value

- Agreement

| - The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} | {7,5,4,11,9} |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | {2} | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} | N/A |

Annex A: Simulation assumptions (for information)

A.1 PDSCH

Other common parameters, refer to TS 38.101-4: Table 5.2-1: Common test parameters for PDSCH

Table 1: Simulation assumptions for PDSCH performance

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD, FDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | TDD: 30/40 FDD: 15/10 |
| MIMO layer and Antenna configuration |  |  | Rank 2: 2x8 Rank 4: 4x8 Rank 8: 8x8 |
| Propagation condition and antenna correlation |  |  | Rank 2: TDLC300-100 ULA Medium B ( = 0.3, = 0.005154) Rank 4: TDLA30-10 Low Rank 8: TDLA30-10 Low |
| MCS |  |  | Rank2: MCS19 Rank 4: MCS17 Rank 8: MCS17 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL |  | Rank2/4: Single-symbol Rank 8: Double-symbol |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| NZP-CSI-RS configuration |  |  | Rank 8: row 6 |
| Overhead |  |  | 0 |
| N1 and N2 configurations for 8Tx cases |  |  | Use (N1,N2) = (4,1), (O1, O2) = (4,1) |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 8 layers cases |
| Number of HARQ Processes |  |  | TDD: 8 FDD: 4 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Tx EVM (Explicitly modeled in the simulation) |  |  | 64QAM:6% 256QAM:3% |
| Test metric |  |  | SNR@70% max TP |

A.2 SDR

Other common parameters, refer to TS 38.101-4:

- Table 5.5A-1: Common test parameters for FDD and TDD component carriers

- Table 5.5A-2: Additional test parameters for FDD CC

- Table 5.5A-3: Additional test parameters for TDD CC

Table 2: Simulation assumptions for SDR performance

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) DL |  | Rank2/4: Single-symbol Rank 8: Double-symbol |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000 – 1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 8 1024QAM: 2 |
| Antenna configuration | 2 layers |  | 2x8 |
|  | 4 layers |  | 4x8 |
|  | 8 layers |  | 8x8 |

A.3 CQI test

Other common parameters, refer to TS 38.101-4:

- Table 6.2.2.1.1.1-1: CQI reporting definition test for FDD

- Table 6.2.2.2.1.1-1: CQI reporting definition test for TDD

Table 3: Simulation assumptions for CQI test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Duplex Mode |  |  |  | TDD, FDD |  |  |  |
| Bandwidth/SCS |  |  | MHz | TDD: 40MHz/30kHz FDD: 10MHz/15kHz |  |  |  |
| TDD UL-DL pattern |  |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  |  | dB | 4 | 5 | 10 | 11 |
| Propagation channel |  |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | TDD: 16 FDD: 8 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | TDD: 10/9 FDD: 5/0 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | two-one-TypeI-SinglePanel-Restriction |  | 00000001 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | TDD: 9.5 FDD: 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Number of HARQ Processes |  |  |  | FDD: 4 TDD: 8 |  |  |  |
| Measurement channel |  |  |  | TDD: As specified in Table A.4-3, TBS.3-4 FDD: As specified in Table A.4-1, TBS.1-3 |  |  |  |

Annex B: CR split

B.1 CR work split for 8Rx single carrier test in

| Section | Test cases | Companies |
| --- | --- | --- |
| 5.1.1.2 Applicability of requirements for different number of RX antenna ports | Table PDSCH/PDCCH/PBCH test applicability rules for 8Rx | Nokia |
| 5.2.4 8RX requirements (New) | 5.2.4.1 FDD PDSCH performance requirements | Samsung |
|  | 5.2.4.2 TDD PDSCH performance requirements |  |
| 5.5A Sustained downlink data rate provided by lower layers | SDR tests | Ericsson |
| 6.1.1.2 Applicability of requirements for different number of RX antenna ports | Table 6.1.1.2-1: Requirements applicability for CSI applicabaility rules | Huawei |
| 6.2.4 8RX requirements (New) | 6.2.4.1 FDD CQI requirements | CTC |
|  | 6.2.4.2 TDD CQI requirements |  |
| A.3.2 Reference measurement channels for PDSCH performance requirements | A.3.2.1.1 Reference measurement channels for SCS 15 kHz FR1 | MediaTek |
|  | A.3.2.2.2 Reference measurement channels for SCS 30 kHz FR1 |  |
| B.1 Static propagation condition | B.1.2 UE Receiver with 8Rx (New) | ZTE |
| B.2.3 MIMO Channel Correlation Matrices | MIMO Correlation Matrices using Uniform Linear Array (ULA) | Apple |
| 5.2A.4 8Rx requirements | Demodulation requirements for 8Rx PDSCH CA | Huawei |

B.2 CR work split for 8Rx CA test

| Section | Test case | Company |
| --- | --- | --- |
| 5.1.1.7 | CA applicability rules | Huawei |
| 5.2A.4 (New) | 8Rx CA performance requirements | Nokia, Nokia Shanghai Bell |
| A.3.2.1.1 Reference measurement channels for SCS 15 kHz FR1 | FRC for FDD 15kHz SCS 2 layers (5MHz,10MHz,15MHz,20MHz,25MHz) : MCS19 | Ericsson |
|  | FRC for FDD 15kHz SCS 2 layers (30MHz,35MHz,40MHz,45MHz,50MHz): MCS 19 | China Telecom |
| A.3.2.2.2 Reference measurement channels for SCS 30 kHz FR1 | FRC for TDD 30kHz SCS 2 layers (5MHz,10MHz,15MHz,20MHz,25MHz,30MHz): MCS 19 | Apple |
|  | FRC for TDD 30kHz SCS 2 layers (40MHz,50MHz,60MHz,80MHz,90MHz,100MHz): MCS 19 | Apple |
| A.3.2.1.1 Reference measurement channels for SCS 15 kHz FR1 | FRC for FDD 15kHz SCS 8 layers (5MHz,10MHz,15MHz,20MHz,25MHz) : MCS17 | MTK |
|  | FRC for FDD 15kHz SCS 8 layers (30MHz,35MHz,40MHz,45MHz,50MHz): MCS 17 | Ericsson |
| A.3.2.2.2 Reference measurement channels for SCS 30 kHz FR1 | FRC for TDD 30kHz SCS 8 layers (5MHz,10MHz,15MHz,20MHz,25MHz,30MHz): MCS 17 | Ericsson |
|  | FRC for TDD 30kHz SCS 8 layers (40MHz,50MHz,60MHz,80MHz,90MHz,100MHz): MCS 17 | Samsung |

Annex C: Reference

[1] R4-2220613, WF on 8RX UE demodulation and CSI requirements, RAN4#105, Huawei, HiSilicon

[2] R4-2302942, WF for 8Rx UE performance requirements, RAN4#106, Huawei, HiSilicon

[3] R4-2305888, WF for 8Rx UE performance requirements, RAN4#106bis-e, Huawei, HiSilicon

[4] R4-2309806, WF for 8Rx UE performance requirements final, RAN4#107, Huawei, HiSilicon

[5] R4-2313877, WF for 8Rx UE performance requirements final, RAN4#108, Huawei, HiSilicon

[6] R4-2316914, WF for 8Rx UE performance requirements final, RAN4#108bis, Huawei, HiSilicon

[7] R4-2321199 WF for 8Rx UE performance requirements, RAN4#110, Huawei, HiSilicon, MediaTek