3GPP TSG-RAN WG4 Meeting # 110 	R4-2402887

Athens, Greece, Feb. 26th – March 1st, 2024

Agenda Item: 8.5.3

Source: MediaTek Inc.

Title: Discussion on RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG

Document for: Discussion

# Introduction

The work item description (WID) [1] indicates that the performance part should start from fourth quarter of 2023. Therefore, in this paper we discuss the principles to define the test cases.

# Discussion

From Rel-17 Concurrent MG test cases discussion [2] and defined test cases, the following principles can be noted:

- Only define test case in NR SA in both FR1 and FR2

- Do not introduce the test for L1 impact

- Do not introduce test cases for intra-freq measurement without gap

- Define a minimum set of test cases for SSB-based measurement

- Only define test case under non-DRX

- Define test case without SBI reporting.

- On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations

- Do not define test cases with simultaneously FR1 and FR2 gaps configured.

- Test cases are limited to single serving carrier

- Only use mandatory gap patterns to define test cases

- Focus on only fully non-overlp and partially partial overlap in the test case design

- Verify gap dropping behaviour without introducing additional test cases

Therefore, RAN4 can take the above principles from Rel-17 at starting point and further study what functionalities to be tested for Concurrent MG with Pre-MG and Concurrent MG with NCSG.

- RAN4 shall take the following Rel-17 principles from Concurrent MG as starting point to define test cases for Rel-18 Concurrent MG with Pre-MG/NCSG.

- Only define test case in NR SA in both FR1 and FR2 - Do not introduce the test for L1 impact - Do not introduce test cases for intra-freq measurement without gap - Define a minimum set of test cases for SSB-based measurement - Only define test case under non-DRX - Define test case without SBI reporting. - On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations - Do not define test cases with simultaneously FR1 and FR2 gaps configured. - Test cases are limited to single serving carrier - Only use mandatory gap patterns to define test cases - Focus on only fully non-overlp and partially partial overlap in the test case design - Verify gap dropping behaviour without introducing additional test cases

Regarding the test case list for Case 1, we provide out tentative list as below:

Table 1: tentative test cases for Case 1

| No. | Test case | Test setup and scenario | Purpose of test |
| --- | --- | --- | --- |
| 1-1-1 | Event triggered reporting test on intra-frequency in FR1 with concurrent gap and autonomous activation/deactivation of Pre-MG for | - Pcell (Cell1) on F1 - Unknown neighbor cell (Cell2) on F2 - Unknown neighbor cell (Cell3) on F3 - F1, F2 and F3 in FR1 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test - MG (#1) with lower priority covers Cell2 - Pre-MG (#2) with higher priority covers Cell3 - MG Overlapping: Partially-fully overlapping | Configuration: - UE can be configured with Pre-MG but being deactivated and the other Type2 MG within concurrent MGs. The configuration of them can be: - The periodicity of Pre-MG and Type2 MG can be Ttype2MG =2* TpreMG - The occasion of Type2 MG can be fully overlapped with these of Pre-MG - Pre-MG activated has higher priority than that of Type2 MG Testing metric: [Figure 1 is used for illustration] - During T1/2, the Pre-MG is deactivated. (Deactivated doesn't cause collision to Con-MG) - T1: Cell2 doesn't transmit anything. - T2: Cell2 start transmitting: - When pre-MG being deactivated at the beginning of testing, UE can report the results of Cell2 within the required period. (+ detection delay and measurements) - We need an event triggering mechanism. - During T3, the Pre-MG being activated (activation delay of Pre-MG overlaps with MG and Pre-MG has higher priority). [dynamic collision testing] - the UE shall NOT report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Type2 MG occasions. - the UE shall be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasion overlapped in dynamic collision - During T4, starts with activated Pre-MG [starting from the 1st complete Pre-MG occasion after the beginning of PCell’s DL slot (i+Latest end time (TBWPswitchDelay, MGL)) + 5ms] - After pre-MG being activated by UE autonomously, UE can report the results of Cell2 and Cell3 within the required period |
| 1-1-2 | Event triggered reporting test on intra-frequency in FR1 with concurrent gap and network-controlled activation/deactivation of Pre-MG for | - Pcell (Cell1) on F1 - Unknown neighbor cell (Cell2) on F2 - Unknown neighbor cell (Cell3) on F3 - F1, F2 and F3 in FR1 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test - MG (#1) with lower priority covers Cell2 - Pre-MG (#2) with higher priority covers Cell3 - MG Overlapping: Partially-fully overlapping | Configuration and testing metric: - Same as of TC 1-1-1. |
| 1-1-3 | Event triggered reporting test on intra-frequency in FR2 with concurrent gap and autonomous activation/deactivation of Pre-MG | - Pcell (Cell1) on F1 - Unknown neighbor cell (Cell2) on F2 - Unknown neighbor cell (Cell3) on F3 - F1, F2 and F3 in FR2 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test - MG (#1) with lower priority covers Cell2 - Pre-MG (#2) with higher priority covers Cell3 - MG Overlapping: Partially-fully overlapping | Configuration and testing metric: - Same as of TC 1-1-1. |
| 1-1-4 | Event triggered reporting test on intra-frequency in FR2 with concurrent gap and network-controlled activation/deactivation of Pre-MG | - Pcell (Cell1) on F1 - Unknown neighbor cell (Cell2) on F2 - Unknown neighbor cell (Cell3) on F3 - F1, F2 and F3 in FR2 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test - MG (#1) with lower priority covers Cell2 - Pre-MG (#2) with higher priority covers Cell3 - MG Overlapping: Partially-fully overlapping | Configuration and testing metric: - Same as of TC 1-1-1. |
| 1-1-5 | Event triggered reporting test on intra-frequency in FR1 with concurrent gap with Pre-MG and autonomous activation/deactivation of two Pre-MG for FR1 | - Pcell (Cell1) on F1 - Unknown neighbor cell (Cell2) on F2 - Unknown neighbor cell (Cell3) on F3 - F1, F2 and F3 in FR1 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test - MG (#1) with lower priority covers Cell2 - Pre-MG (#2) with higher priority covers Cell3 - Activation Overlapping: fully overlapping | - Verify that the UE correctly activates and deactivates the pre-MG and makes correct measurement and reporting of an event with activated and deactivated pre-MG - Multiple Pre-MG activation/deactivation delay - Verify that the UE makes correct reporting of an event: - After pre-MG being activated by UE autonomously, UE can report the results of Cell2 and Cell3 within the required period - Alternatively: test with ACK/NACK |
| 1-1-6 | Event triggered reporting test on intra-frequency in FR1 with concurrent gap with Pre-MG and network-controlled activation/deactivation of two Pre-MG for FR1 | - Pcell (Cell1) on F1 - Unknown neighbor cell (Cell2) on F2 - Unknown neighbor cell (Cell3) on F3 - F1, F2 and F3 in FR1 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test - MG (#1) with lower priority covers Cell2 - Pre-MG (#2) with higher priority covers Cell3 - Activation Overlapping: fully overlapping | - Verify that the UE correctly activates and deactivates the pre-MG and makes correct measurement and reporting of an event with activated and deactivated pre-MG - Multiple Pre-MG activation/deactivation delay - Verify that the UE makes correct reporting of an event: - After pre-MG being activated by UE autonomously, UE can report the results of Cell2 and Cell3 within the required period - Alternately: test with ACK/NACK |
| 1-1-7 | Event triggered reporting test on intra-frequency in FR1 with concurrent gap with Pre-MG and autonomous activation/deactivation of two Pre-MG for FR2 | - Pcell (Cell1) on F1 - Unknown neighbor cell (Cell2) on F2 - Unknown neighbor cell (Cell3) on F3 - F1, F2 and F3 in FR2 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test - MG (#1) with lower priority covers Cell2 - Pre-MG (#2) with higher priority covers Cell3 - Activation Overlapping: fully overlapping | Same as of TC 1-1-5. |
| 1-1-8 | Event triggered reporting test on intra-frequency in FR1 with concurrent gap with Pre-MG and network-controlled activation/deactivation of two Pre-MG for FR2 | - Pcell (Cell1) on F1 - Unknown neighbor cell (Cell2) on F2 - Unknown neighbor cell (Cell3) on F3 - F1, F2 and F3 in FR2 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test - MG (#1) with lower priority covers Cell2 - Pre-MG (#2) with higher priority covers Cell3 - Activation Overlapping: fully overlapping | Same as of TC 1-1-6. |

Figure 1: Illustration for test case time periods for TC 1-1-1/2/3/4

Regarding the test case list for Case 2, we provide out tentative list as below:

Table 2: tentative test cases for Case 2

| No. | Test case | Test setup and scenario | Purpose of test |
| --- | --- | --- | --- |
| 1-2-1 | Event triggered reporting test on intra-frequency in FR1 with concurrent gap and NCSG | - Pcell (Cell1) on F1 - Unknown neighbor cell (Cell2) on F2 - Unknown neighbor cell (Cell3) on F3 - F1, F2 and F3 in FR1 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test - MG (#1) with lower priority covers Cell2 - NCSG (#2) with higher priority covers Cell3 - MG Overlapping: Partially-fully overlapping | - UE can report the results of Cell2 and Cell3 within the required period |
| 1-2-2 | Event triggered reporting test on intra-frequency in FR2 with concurrent gap and NCSG | - Pcell (Cell1) on F1 - Unknown neighbor cell (Cell2) on F2 - Unknown neighbor cell (Cell3) on F3 - F1, F2 and F3 in FR2 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test - MG (#1) with lower priority covers Cell2 - NCSG (#2) with higher priority covers Cell3 - MG Overlapping: Partially-fully overlapping | - UE can report the results of Cell2 and Cell3 within the required period |

- RAN4 should consider defining test cases listed in Table 1 and 2 of this contribution.

# Summary

In this contribution, we have the following proposal:

Proposal 1: RAN4 shall take the following Rel-17 principles from Concurrent MG as starting point to define test cases for Rel-18 Concurrent MG with Pre-MG/NCSG.

- Only define test case in NR SA in both FR1 and FR2

- Do not introduce the test for L1 impact

- Do not introduce test cases for intra-freq measurement without gap

- Define a minimum set of test cases for SSB-based measurement

- Only define test case under non-DRX

- Define test case without SBI reporting.

- On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations

- Do not define test cases with simultaneously FR1 and FR2 gaps configured.

- Test cases are limited to single serving carrier

- Only use mandatory gap patterns to define test cases

- Focus on only fully non-overlp and partially partial overlap in the test case design

- Verify gap dropping behaviour without introducing additional test cases

Proposal 2: RAN4 should consider defining test cases listed in Table 1 and 2 of this contribution.

# References

- RP-231477, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation, June 2023, Taipei Taiwan.

- R4-2210585, “WF on R17 NR MG enhancements – multiple concurrent MGs”, MediaTek Inc.