3GPP TSG-RAN WG4 Meeting #110	R4-2402890

Athens, Greece, Feb. 26th – March 1st, 2024

Agenda Item: 8.10.5

Source: MediaTek inc.

Title: Discussion on performance part for FR1 less than 5MHz

Document for: Discussion and decision

# Introduction

The agreed work plan for FR1 less than 5 MHz indicates that the performance parts start from the October 2023 meeting [1]. In addition, the discussion for the performance part is captured in WF [2]. Hence, this contribution paper provides the views on which test shall be specified.

# Discussion on performance requirements

Based on the WF from the previous meeting, the discussion can be divided to multiple sections:

- Discussion on tests for different CBW

From the WF, the open issue is given as:

Sub-topic 2-1 Tests for different CBW Way forward: Further discuss whether a UEs supporting 12 RB bandwidth also shall support 15 RB bandwidth configuration, at least in rel-18.

This issue is not clear enough. Further details for discussion are needed.

- Discussion on L3 and L1 accuracy requirements

From the WF, the open issue is given as:

## Sub-topic 2-2 L3 and L1 accuracy requirements Way forward: Further discuss if existing SSB based L3 and L1 accuracy requirements applies for less than 5MHz BW UE.

So far, the discussion didn’t impact the accuracy requirements, hence, it is not clear whether a test case will be needed or not. Further details for discussion are needed.

- Discussion on test cases for UE operating in 5 MHz and use of DRX

From the WF, the open issue is given as:

## Sub-topic 2-3 Test cases for UE operating in 5 MHz Way forward: RAN4 further discuss following two types of UEs for specifying test cases for UE operating in 5 MHz. UE type 1: RAN4 specify test case for UE supporting other Channel Band width (CBW) along with less than 5 MHz CBW UE Type 2: RAN4 specify test case for UE supporting only less than 5 MHz. ## Sub-topic 2-4 Use of DRX and non-DRX in test cases Way forward: For RLM/BFD, link recovery and SSB index reading test cases, should RAN4 specify the test cases of non-DRX with 12 PRBs, while test cases with DRX are specified with 15 PRBs?

On one hand, for the UE operating with 5 MHz bandwidth, the defined requirements are the same as that for the existing NR UE. Thus, there is no need to define additional test cases to cover the UE operating with 5 MHz. On the other hand, from the core requirements changes for the 3 MHz UE, the main changes are captured in the radio link monitoring (RLM), link recovery procedure including beam failure detection (BFD), and SSB index reading. Therefore, the test cases should be specified to cover these new requirements. In addition, RLM and link recovery procedure requirements has two sets of requirements, which are based on 12 PRBs and 15 PRBs. In order to minimise the number of test cases for RLM and BFD, RAN4 should specify the test cases of non-DRX with 12 PRBs, while test cases with DRX are specified with 15 PRBs.

- For UE with 5 MHz, given that the requirements are the same as the existing NR UE, hence, there is no need to specify additional test cases for 5 MHz UE.

- RAN4 shall specify test cases to cover the new requirements of RLM, BFD, link recovery and SSB index reading for 3MHz channel BW.

- For the RLM/BFD test cases, RAN4 should specify the test cases of non-DRX with 12 PRBs, while test cases with DRX are specified with 15 PRBs.

- Discussion on HO test cases

From the WF, the open issue is given as:

## Sub-topic 2-5 HO test cases Way forward: Further discuss and decide the introduction of HO test cases for UE type 1 and 2 as mentioned in subtopic 2-3.

If RAN4 agree to capture new HO delay values in the requirements, then, a new test case will be needed. Further details for discussion are needed.

- If RAN4 agree to capture new HO delay values in the requirements, then, a new test case will be needed.

- Discussion on RLM, BFD and CBD test cases plus SSB index reading

From the WF, the open issue is given as:

## Sub-topic 2-6 RLM test cases Way forward: Further discuss and decide the introduction of RLM test cases for UE type 1 and 2 as mentioned in subtopic 2-3. ## Sub-topic 2-7 BFD and CBD test cases Way forward: Further discuss/decide the introduction of BFD and CBD test cases for UE type 1 and 2 as mentioned in subtopic 2-3 ## Sub-topic 2-8 Intra-frequency measurement delay with index reading test cases Way forward: Further discuss and decide the introduction of measurement delay with index reading test cases for UE type 1 and 2 as mentioned in subtopic 2-3

The suggested test cases for RLM to be specified are provided below:

Radio link Monitoring: - Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode with PDCCH 12 PRBs UE. - Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode PDCCH 12 PRBs UE. - Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode with PDCCH 15 PRBs UE. - Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode with PDCCH 15 PRBs UE.

The suggested test cases for BFD to be specified are provided below:

Beam Failure Detection and Link recovery procedures: - Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode with PDCCH 12 PRBs UE. - Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode with PDCCH 15 PRBs UE.

The suggested test cases for SSB index reading to be specified are provided below:

SSB index reading: - SA event triggered reporting tests without gap under non-DRX with SSB index reading for 12 PRBs of PBCH UE. - SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 12 PRBs PBCH UE.

- RAN4 should specify the following test cases for Radio link Monitoring:

- Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode with PDCCH 12 PRBs UE.

- Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode PDCCH 12 PRBs UE.

- Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode with PDCCH 15 PRBs UE.

- Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode with PDCCH 15 PRBs UE.

- RAN4 should specify the following test cases for BFD and link recovery:

- Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode with PDCCH 12 PRBs UE.

- Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode with PDCCH 15 PRBs UE.

- RAN4 should specify the following test cases for SSB index reading:

- SA event triggered reporting tests without gap under non-DRX with SSB index reading for 12 PRBs of PBCH UE.

- SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 12 PRBs PBCH UE.

# Summary

In this contribution, we have the following proposals:

Proposal 1:For UE with 5 MHz, given that the requirements are the same as the existing NR UE, hence, there is no need to specify additional test cases for 5 MHz UE.

Proposal 2:RAN4 shall specify test cases to cover the new requirements of RLM, BFD, link recovery and SSB index reading for 3MHz channel BW.

Proposal 3: For the RLM/BFD test cases, RAN4 should specify the test cases of non-DRX with 12 PRBs, while test cases with DRX are specified with 15 PRBs.

Proposal 4: If RAN4 agree to capture new HO delay values in the requirements, then, a new test case will be needed.

Proposal 5: RAN4 should specify the following test cases for Radio link Monitoring:

- Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode with PDCCH 12 PRBs UE. - Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode PDCCH 12 PRBs UE. - Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode with PDCCH 15 PRBs UE. - Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode with PDCCH 15 PRBs UE.

Proposal 6: RAN4 should specify the following test cases for BFD and link recovery:

- Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode with PDCCH 12 PRBs UE. - Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode with PDCCH 15 PRBs UE.

Proposal 7: RAN4 should specify the following test cases for SSB index reading:

- SA event triggered reporting tests without gap under non-DRX with SSB index reading for 12 PRBs of PBCH UE. - SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 12 PRBs PBCH UE.

# References

- R4-2300197, “Work plan for NR support for dedicated spectrum less than 5MHz for FR1”, Nokia, Athens, Greece, Feb 2023.

- R4-2321629, “WF on NR_FR1_lessthan_5MHz_BW WI”, Nokia, Chicago, USA, Nov 2023.