3GPP TSG-RAN WG4 Meeting #110	R4-2402911

# Athens, Greece, February 26 – March 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM performance requirements for NR Carrier Phase Positioning

Agenda item:	8.14.3.5

Document for:	Discussion

Introduction

The specification of NR carrier phase positioning (NR CPP) is one of the objectives in the Rel-18 WID on expanded and improved positioning [1].

This contribution summarizes RAN4 #109 agreements and lists discussion on some of the open issues with regard to impacts to RRM performance requirements for NR CPP.

Discussion

RAN4 agreements and open issues

RAN4 agreements and list of open issues from RAN4 #109 are captured below [2]. More details are available in [3].

## Sub-topic 2-1 Measurement period requirements - Issue 2-1-1: PRS measurement period requirements for DL RSCP/DL RSCPD: Agreements: - When LMF requests the UE to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) for a PFL, and UE supports FG 41-2-3, the Twindow needs to be considered in the measurement period: - For periodic time window, - Adopt the following updates to the existing measurement period requirements for the PFL - Tavailable is defined as LCM(Tprs, MGRP, Twindow), where Twindow is the maximum periodicity of the indicated time window(s) - When calculating Lprs and Tprs, only the PRS resources in the indicated resources sets and overlapped with both the MG and the indicated time window(s) are considered. - For one-shot time window case, - Lavailable_PRS,i only counts PRS resource instances within the indicated time window(s). - the start of the measurement period is the start of the time window. - Issue 2-1-2: CPP measurement in RRC_IDLE state: Agreements: - RAN4 to define the CPP measurements requirements in RRC_IDLE state.

- Issue 2-1-3: The impact of carrier frequency offset: Agreements: - No consensus in RAN4 on whether or not to specify UE behavior or requirements related to measurement of carrier frequency offset in Rel-18 core part. RAN4 can close the WI core part without further agreement on this issue. - FFS whether and how to account for carrier frequency offset in the accuracy requirements in performance part. - Issue 2-1-4: Requirements applicability regarding MG reconfiguration Agreements: - If during the measurement period, the MG pattern is reconfigured or time window for carrier phase measurement is reconfigured, the measurement period for RSCPD with RSTD and RSCP with UE Rx-Tx can be longer. - Issue 2-1-5: Impact of UE mobility Agreements: - Uphold the previous agreement at RAN4#108. ## Sub-topic 2-2 CPP measurement reporting requirements - Issue 2-2-1: Reporting requirements for DL CPP measurement Agreements: - Uphold the previous Reporting requirements for DL CPP measurement at RAN4#108. ## Sub-topic 2-3 Accuracy requirements for DL RSCPD - Issue 2-3-1: Channel model Proposals - Option 1: - Define accuracy requirements for DL RSCPD measurement at least for AWGN channel. - Define accuracy requirements for DL RSCPD measurement for Two-Tap channel based on the simulation results. For RSTD measurement which is reported together, the existing accuracy requirements for fading channel are reused. - Issue 2-3-2: Side condition Agreements: - Define DL RSCPD measurement accuracy requirements for side condition [-3, -6] dB if RSTD measurement is done with reduced number of samples. - FFS: Define DL RSCPD measurement accuracy requirements for side condition [-6, -13] dB if RSTD measurement is done over 4 samples. - Issue 2-3-3: Number of Samples Agreements: - Define the DL RSCPD accuracy measurement requirements with single sample. - Issue 2-3-4: PRS configuration Proposals - Option 1: - RAN4 to align the targeted PRS BW range for NR DL CPP in RRC_CONNECTED to that for positioning techniques specified in Rel-16 and in RRC_INACTIVE to that for positioning techniques specified in Rel-17 in regard to BW related RRM performance requirements. ## Sub-topic 2-4 Accuracy requirements for DL RSCP - Issue 2-4-1: whether to define accuracy requirements for DL RSCP Agreements: - Define relative accuracy requirements for DL RSCP measurement. - Issue 2-4-2: Side condition Agreements: - Update the simulation assumption of DL RSCPD measurement to accommodate the simulation for relative accuracy of DL RSCP measurement. - In R4-2321459, to add SINR side condition (-3, -13, -13)dB for NSample = 1 and (0, -6, -6)dB for NSample = 1. ## Sub-topic 2-5 Accuracy requirements for UL RSCP - Issue 2-5-1: Accuracy requirements for UL carrier phase measurement: Proposals - Option 1: - Define the gNB accuracy measurement requirements of UL RSCP NG-RAN node assisted positioning. - Option 2: - RAN4 to postpone the decision on whether to define UL RSCP measurement accuracy requirements to performance part. - RAN4 to consider whether the Rel-16 approach for gNB Rx-Tx time difference accuracy performance with specified BB performance and manufacturer declared impairments margin can be reused for defining UL RSCP accuracy performance in Rel-18. - Option 3: - RAN4 to investigate RRM impacts due to SRS configuration change and UL transmission timing change due to TA adjustment or UE autonomous timing adjustment. - RAN4 to investigate whether UE can keep using fixed Tx timing although the DL reception reference timing is changed in order to guarantee the measurement accuracy for UL RSCP.

  - UE measurement accuracy requirements

Targeted PRS bandwidth range for accuracy requirements

The issue was discussed at RAN4 #109 without consensus.

- Issue 2-3-4: PRS configuration Proposals - Option 1: - RAN4 to align the targeted PRS BW range for NR DL CPP in RRC_CONNECTED to that for positioning techniques specified in Rel-16 and in RRC_INACTIVE to that for positioning techniques specified in Rel-17 in regard to BW related RRM performance requirements.

Regarding issue 2-3-4, as NR CPP will use MG assisted measurements performed on a single PFL/carrier, the targeted PRS BW range for NR DL CPP in FR1 and FR2 should be aligned to Rel-16 for RRC_CONNECTED and to Rel-17 for RRC_INACTIVE. Previously, it has already been agreed that the measurement period requirements (formula) for DL measurements are agnostic to the PRS BW. Hence the same PRS BW range as for Rel-16 and Rel-17 should be considered for BW related RRM performance requirements for DL CPP in TS 38.133.

- RAN4 to align the targeted PRS BW range for NR DL CPP in RRC_CONNECTED to that for positioning techniques specified in Rel-16 and in RRC_INACTIVE to that for positioning techniques specified in Rel-17 in regard to BW related RRM performance requirements.

Channel model

The issue was discussed at RAN4 #109 without consensus.

- Issue 2-3-1: Channel model Proposals - Option 1: - Define accuracy requirements for DL RSCPD measurement at least for AWGN channel. - Define accuracy requirements for DL RSCPD measurement for Two-Tap channel based on the simulation results. For RSTD measurement which is reported together, the existing accuracy requirements for fading channel are reused.

Regarding issue 2-3-1, option 1 can serve as baseline for defining measurements accuracy requirements for RSCPD and RSTD. In addition, the same channel models should apply for DL RSCP and UE Rx-Tx time difference measurements.

1) Define accuracy requirements for DL RSCP and RSCPD at least for AWGN channel.

- 2) Define accuracy requirements for DL RSCP and RSCPD for Two-Tap channel based on the 	simulation results.

- 3) For the companion measurements, i.e. UE Rx-Tx time difference and RSTD, the existing 	accuracy requirements for fading channel are reused.

Side condition for RSCPD absolute measurement accuracy

- Issue 2-3-2: Side condition Agreements: - Define DL RSCPD measurement accuracy requirements for side condition [-3, -6] dB if RSTD measurement is done with reduced number of samples. - FFS: Define DL RSCPD measurement accuracy requirements for side condition [-6, -13] dB if RSTD measurement is done over 4 samples.

The issue was discussed at RAN4 #109 without consensus.

Regarding above issue 2-3-2, we agree to align the side condition for RSCPD to that for RSTD in case RSTD is measured over 4 samples.

- RAN4 to align the side condition for RSCPD absolute accuracy to that for RSTD, in case RSTD is measured over 4 samples, i.e. [-6, -13] dB.

It is noted simulation results for RSCPD are provided in [4].

Side condition for DL RSCP relative measurement accuracy

The issue was not discussed at RAN4 #109. For side condition for DL RSCP accuracy we propose to reuse the side conditions for UE Rx-Tx time difference for 4 samples from Rel-16, i.e. [-3,-13] dB and for 2 samples from Rel-17 latency reduction, i.e.[0,-6] dB.

RAN4 to align the side condition for DL RSCP relative accuracy to that for UE Rx-Tx time difference, in case UE Rx-Tx time difference is measured over 2 samples, i.e. [0, -6] dB, or over 4 samples, i.e.        [-3, -13] dB.

TRP measurement accuracy requirements

UL RSCP accuracy requirements

- Issue 2-5-1: Accuracy requirements for UL carrier phase measurement: Proposals - Option 1: - Define the gNB accuracy measurement requirements of UL RSCP NG-RAN node assisted positioning. - Option 2: - RAN4 to postpone the decision on whether to define UL RSCP measurement accuracy requirements to performance part. - RAN4 to consider whether the Rel-16 approach for gNB Rx-Tx time difference accuracy performance with specified BB performance and manufacturer declared impairments margin can be reused for defining UL RSCP accuracy performance in Rel-18. - Option 3: - RAN4 to investigate RRM impacts due to SRS configuration change and UL transmission timing change due to TA adjustment or UE autonomous timing adjustment. - RAN4 to investigate whether UE can keep using fixed Tx timing although the DL reception reference timing is changed in order to guarantee the measurement accuracy for UL RSCP.

Regarding issue 2-5-1, we support options 2 and 3. Option 3 has been dealt with in section 2.2.3 on impact of UE mobility and is rather related to core requirement part. Thus, we refer to Option 2 in the following.

In fact, as no decision has been taken in the core part, whether to define measurement accuracy requirements for UL RSCP needs to be done in the performance part. In our view, measurement accuracy requirements for UL RSCP could be specified following the approach for gNB Rx-Tx time difference in Rel-16, where BB performance was specified, and a manufacturer declared impairments margin added on top. This approach can be reused for UL RSCP in Rel-18.

RAN4 to consider whether the Rel-16 approach for gNB Rx-Tx time difference accuracy performance with specified BB performance and manufacturer declared impairments margin can be reused for defining UL RSCP accuracy performance in Rel-18.

Conclusion

Impacts to RRM performance requirements for NR carrier phase positioning (NR CPP) being one of the objectives in the Rel-18 WID on expanded and improved positioning [1], are investigated in this contribution.

The following proposals are made.

- UE measurement accuracy requirements

- RAN4 to align the targeted PRS BW range for NR DL CPP in RRC_CONNECTED to that for positioning techniques specified in Rel-16 and in RRC_INACTIVE to that for positioning techniques specified in Rel-17 in regard to BW related RRM performance requirements.

1) Define accuracy requirements for DL RSCP and RSCPD at least for AWGN channel.

- 2) Define accuracy requirements for DL RSCP and RSCPD for Two-Tap channel based on the 	simulation results.

- 3) For the companion measurements, i.e. UE Rx-Tx time difference and RSTD, the existing 	accuracy requirements for fading channel are reused.

- RAN4 to align the side condition for RSCPD absolute accuracy to that for RSTD, in case RSTD is measured over 4 samples, i.e. [-6, -13] dB.

RAN4 to align the side condition for DL RSCP relative accuracy to that for UE Rx-Tx time difference, in case UE Rx-Tx time difference is measured over 2 samples, i.e. [0, -6] dB, or over 4 samples, i.e.        [-3, -13] dB.

- TRP measurement accuracy requirements

RAN4 to consider whether the Rel-16 approach for gNB Rx-Tx time difference accuracy performance with specified BB performance and manufacturer declared impairments margin can be reused for defining UL RSCP accuracy performance in Rel-18.

- References

- RP-233382, “Revised WID on Expanded and Improved NR positioning”, 3GPP TSG RAN#102

- R4-2321524, “WF on R18 NR positioning – SL positioning and Carrier Phase Positioning”, CATT

- R4-2318176, “Topic summary for [109][220] NR_pos_enh2_part2”, Moderator (CATT)

- R4-2401875, “Simulation results for DL RSCPD”, Nokia, Nokia Shanghai Bell