3GPP TSG-RAN WG4 Meeting #110	R4-2402927

# Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | 0013 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR proposal to add Doppler and Delay variation examples as a function of time for NGSO and GSO in a new Annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Doppler shift and Delay variation versus time is not explicitly defined in the document. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Plots for NGSO (LEO at 600 km, LEO at 1200 km) with orbit inclination of 88 degrees and GSO with orbit inclination of 7 degrees is given as part of a new Annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of clarification of a classic Doppler and Delay variation with respect to different representative orbits. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | None |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

Annex D:
Summary of simulation results for the IoT derivation

In this annex, simulations results used to derive the IoT value were collected.

[To be updated]

Annex E:
Examples of Delay and Doppler shift variation as a function of time for different satellite orbit types

The purpose of Annex E is only informative and is related to the distance variation between NTN UE and Satellite as captured in Section 5.3 of TR 38.811 [5] which presents methodology to characterise Doppler and propagation Delay.

Therefore, the information in current section shall not be interpreted as testing configuration. The purpose is only to provide a more realistic representation of Doppler and Delay with respect to a few constellations given as examples.

As described below, Delay and Doppler shift variation with time are presented as examples for different defined NGSO (LEO@600km, LEO@1200km) and GSO satellite representative orbits. The NTN UE location is assumed static for all graphs as follows:

- Latitude: 25.08439333 deg;

- Longitude: 121.56076999 deg;

- Altitude: 0 m.

Three representative cases are further defined in following sub-sections E.1, E.2 and E.3 respectively:

- Low Earth Orbit (LEO) at 600 km altitude case;

- Low Earth Orbit (LEO) at 1200 km altitude case;

- GSO orbit case with a satellite inclination of 7 degrees relative to the Equator.

## E.1 LEO 600 km case

For this example, a satellite orbit inclination with respect of the Equator plane of 88° is considered.

Figure E.1-1: LEO600km with 88° orbit inclination

## E.2 LEO 1200 km case

For this example, a satellite orbit inclination with respect of the Equator plane of 88° is considered.

Figure E.2-1: LEO1200km with 88° orbit inclination

## E.3 GSO orbit case

For this example, a satellite orbit inclination with respect of the Equator plane of 7° is considered.

Figure E.3-1: GSO with 7° orbit inclination

## E.4 Examples of Doppler and Delay for different elevation angles and for different satellite orbit types

One can represent the following few “selected” example scenarios computed for a min elevation angle target of 30°:

Table E.4-1: Example scenarios with values computed for 30° min elevation angle target

| Frequency | Constellation | Elevation | Doppler shift (kHz) | OneWayDelay_ms |
| --- | --- | --- | --- | --- |
| 2,00 GHz | LEO@600 km | 30,0 | 39,734 | 3,6 |
| 2,00 GHz | LEO@600 km | 149,9 | -39,899 | 3,7 |
| 2,00 GHz | LEO@600 km | 89,6 | 0,026 | 2,0 |
| 2,00 GHz | LEO@1200 km | 29,9 | 35,127 | 6,7 |
| 2,00 GHz | LEO@1200 km | 149,9 | -35,273 | 6,8 |
| 2,00 GHz | LEO@1200 km | 89,5 | 0,007 | 4,0 |
| 30,00 GHz | LEO@600 km | 30,0 | 596,011 | 3,6 |
| 30,00 GHz | LEO@600 km | 149,9 | -598,486 | 3,7 |
| 30,00 GHz | LEO@600 km | 89,6 | 0,384 | 2,0 |
| 30,00 GHz | LEO@1200 km | 29,9 | 526,908 | 6,7 |
| 30,00 GHz | LEO@1200 km | 149,9 | -529,095 | 6,8 |
| 30,00 GHz | LEO@1200 km | 89,5 | 0,110 | 4,0 |
| 2,00 GHz | GEO | 90,14 | 0,000 | 119,37 |
| 2,00 GHz | GSO | 58,85 | 0,19 | 121,99 |
| 2,00 GHz | GSO | 58,85 | -0,19 | 121,99 |
| 2,00 GHz | GSO | 68,87 | 0,000 | 120,59 |
| 30,00 GHz | GEO | 90,14 | 0,000 | 119,37 |
| 30,00 GHz | GSO | 58,85 | 2,87 | 121,99 |
| 30,00 GHz | GSO | 58,85 | -2,87 | 121,99 |
| 30,00 GHz | GSO | 68,87 | 0,000 | 120,59 |

Similarly, one can represent the following few “selected” example scenarios computed for a min elevation angle target of 10°:

Table E.4-2: Example scenarios with values computed for 10° min elevation angle target

| Frequency | Constellation | Elevation | Doppler shift (kHz) | OneWayDelay_ms |
| --- | --- | --- | --- | --- |
| 2,00 GHz | LEO@600 km | 9,9 | 45,304 | 6,5 |
| 2,00 GHz | LEO@600 km | 170,0 | -45,239 | 6,6 |
| 2,00 GHz | LEO@600 km | 89,6 | 0,026 | 2,0 |
| 2,00 GHz | LEO@1200 km | 10,0 | 40,062 | 10,4 |
| 2,00 GHz | LEO@1200 km | 170,0 | -39,918 | 10,7 |
| 2,00 GHz | LEO@1200 km | 89,5 | 0,007 | 4,0 |
| 30,00 GHz | LEO@600 km | 9,9 | 679,567 | 6,458 |
| 30,00 GHz | LEO@600 km | 170,0 | -678,582 | 6,605 |
| 30,00 GHz | LEO@600 km | 89,6 | 0,384 | 2,031 |
| 30,00 GHz | LEO@1200 km | 10,0 | 600,927 | 10,441 |
| 30,00 GHz | LEO@1200 km | 170,0 | -598,774 | 10,656 |
| 30,00 GHz | LEO@1200 km | 89,5 | 0,110 | 4,039 |
| 2,00 GHz | GEO | 90,14 | 0,000 | 119,37 |
| 2,00 GHz | GSO | 58,85 | 0,19 | 121,99 |
| 2,00 GHz | GSO | 58,85 | -0,19 | 121,99 |
| 2,00 GHz | GSO | 68,87 | 0,000 | 120,59 |
| 30,00 GHz | GEO | 90,14 | 0,000 | 119,37 |
| 30,00 GHz | GSO | 58,85 | 2,87 | 121,99 |
| 30,00 GHz | GSO | 58,85 | -2,87 | 121,99 |
| 30,00 GHz | GSO | 68,87 | 0,000 | 120,59 |

Note: UE position was considered as stationary and configured with Latitude: 25.08439333, Longitude: 121.56076999, Altitude: 0.

Annex F
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-11 | RAN4# 101-e | R4-2120776 |  |  |  | Added approved TPs in RAN4#101-e including: R4-2120759, draft TP to TR 38.863: Operating bands and channel arrangements R4-2120760, TP for 38.863 on system parameters on satellite bands R4-2120761,TP to TR 38.863 – Regulatory aspects R4-2120762, TP to TR 38.863: node class, RF RX (6.2) R4-2120763, TP for 38.863 on NTN UE transmission characteristics R4-2120772, Draft text proposal to update TR 38.863 NTN related RF and co-existence aspects | 0.1.0 |
| 2022-01 | RAN4#101-bis-e |  |  |  |  | Added approved TPs in RAN4#101-bis-e including: R4-2201257, TP for 38.863 on system parameters to clarify “NTN satellite bands” R4-2201838, Draft proposal to update TR 38.863 NTN related RF and co-existence aspects R4-2202988, Draft Text Proposal for TR 38.863 R4-2203037, TP for 38.863 on UE transmission characteristics for satellite access R4-2203038, TP for 38.863 on maximum input level for NTN UE R4-2203039, TP for 38.863 on UE Receiver characteristics for satellite access R4-2203040, TP to TR 38.863 on transmitter characteristics for satellite access node R4-2203081, Draft text proposal to update TR 38.863 Chapter 3 R4-2203082, TP to TR 38.863 on channel raster and sync raster R4-2203084, TP to TR 38.863 Regulatory aspects for HAPS R4-2203085, TP to TR 38.863 on general aspects R4-2203129, TP to TR 38.863 – Regulatory aspects | 0.2.0 |
| 2022-03 | RAN4#102-e |  |  |  |  | Added approved TPs in RAN4#102--e including: R4-2205557, R4-2207330, TP TR 38.863 7.4.1 NTN UE Requirement (General) R4-2207333, TP to TR 38.863 on Section 5.2 NTN Satellite band R4-2207338, TP for TR 38.863: Regulatory aspects for NTN satellite access nodes and Ues operating in UL1626.5-1660.5 MHz and DL 1525-1559 MHz frequencies ranges R4-2207339, TP to TR 38.863 Regulatory aspects for HAPS R4-2207345, Draft text proposal for Clauses 7, 7.1, 7.2, 7.3 in TR 38.863 R4-2207351, Draft text proposal to update TR 38.863 Chapter 6 R4-2207353, Draft text proposal for Clauses 6.4 and 6.5 in TR 38.863 to include simulation results based on Non-AAS antenna assumption R4-2207360, Draft text proposal for Clause 7.3.4.7.3 OTA ACLR in TR 38.863 R4-2207367, Draft text proposal for Clause 7.3.5.6 OTA Out-of-band blocking in TR 38.863 R4-2207379, Draft text proposal for Clause 7.3.3.2.4 Out-of-band blocking in TR 38.863 R4-2207381, Draft text proposal for Clauses 7.3.3.2.3.1 Adjacent Channel Selectivity (ACS) and 7.3.3.2.3.2 In-band blocking in TR 38.863 R4-2207397, TP on TR 38.863 for NTN UE Tx requirements R4-2207398, Draft TP to update TR 38.863 clause 7.4.3.2 on NTN UE ACS R4-2207399, Draft TP to update TR 38.863 clause 7.4.3.2 on Blocking characteristics R4-2207401, TP for TR38.863 on Intermodulation characteristics for NTN UE R4-2207402, TP for 38.863 on spurious response for NTN UE R4-2207403, TP for TR 38.863: Unwanted emissions for NTN satellite Ues transmitting in 1626.5 to 1660.5 MHz R4-2207406, TP for TR 38.863: Updates to UE Maximum Output Power for n255 R4-2207414, TP for 38.863 on UE Receiver characteristics for satellite access R4-2207460, Draft text proposal for Clause 7.3.2.2.4.1 ACLR in TR 38.863 | 0.3.0 |
| 2022-05 | RAN4#103-e |  |  |  |  | Added approved TPs in RAN4#103e including: R4-2209089, TP for 38.863 on general part for NTN UE conducted receiver characteristics R4-2209364, TP for 38.863 on UE Rx spurious emission requirements for satellite access R4-2209365, TP for 38.863 on UE antenna characteristics for satellite access R4-2210228, Draft text proposal for Clauses 6.4 and 6.5 Corrections Typos - TR 38.863 R4-2210636, TP to TR 38.863: coexistence issues between NTN and TN from Rel-17 RAN4 study. R4-2210847, TP to TR 38.863 - Updates R4-2210848, Draft text proposal for Clause 6.1 Coexistence Figures - TR 38.863 R4-2210853, TP for 38.863: clause 7.3.2 Conducted transmission characteristics R4-2210858, TP to TR 38.863: Conducted reference sensitivity R4-2210859, TP to TR 38.863: Conducted Rx dynamic range R4-2210860, Draft text proposal for Clause 7.3.2.2.5 Transmitter spurious emissions - TR38.863 R4-2210865, Tentative draft pCR for Clause 7.3.2.2.4.2 Operating band unwanted emissions - TR 38.863 R4-2210866, Draft text proposal for Clause 7.3.2.2.1 SAN output power - TR 38.863 R4-2210867, Draft text proposal for Clause 7.3.3.3.1 OTA sensitivity - TR 38.863 R4-2210868, Draft text proposal for Clause 7.3.3.3.2 OTA reference sensitivity - TR 38.863 R4-2210869, Draft text proposal for Clause 7.3.3.3.3 OTA dynamic range - TR 38.863 R4-2210870, Draft text proposal for Clause 7.3.3.3.7 OTA receiver intermodulation - TR 38.863 R4-2210871, Draft text proposal for Clause 7.3.3.3.8 OTA in-channel selectivity - TR 38.863 R4-2210872, Draft text proposal for Clause 7.3.3.3.4 OTA in-band selectivity and blocking - TR 38.863 R4-2210875, TP for TR 38.863: Updates to n255 A-MPR Clause R4-2211133, Draft Text Proposal to Update TR 38.863 Chapter 3,6 and 8 R4-2211134, Draft Text Proposal to Update TR 38.863 structure R4-2211146, Draft text proposal for Clause 7.3.3.3.2 OTA reference sensitivity - TR 38.863 | 0.4.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-06 | RAN#96 |  |  |  |  | Approved by plenary – Rel-17 spec under change control | 17.0.0 |
| 2022-09 | RAN#97 | RP-222035 | 0001 |  | F | CR for 38.863 to maintain UE RF parts | 17.1.0 |
| 2022-09 | RAN#97 | RP-222035 | 0002 | 1 | F | Correction to TR 38.863 on Regulatory aspects for HAPS | 17.1.0 |
| 2023-03 | RAN#99 | RP-230516 | 0005 |  | F | CR for TR 38.863 to maintain SAN parts | 17.2.0 |

<End of Change 1>