3GPP TSG-RAN WG4 Meeting # 110					      R4-2402961

# Athens, GR, 26 Feb – 01 Mar, 2024

Source: 	ZTE Corporation

Title: 	WF on NCR EMC Conformance

Agenda Item:	8.20.3

Document for:	Approval

- Introduction

During RAN4#110 meeting, the issues from moderator summary[4] have been well discussed. Therefore based on online and offline discussion, the following WF was formulated.

- Way Forward

The open issues, agreements and tentative agreements reached in RAN4#110 meeting are listed below.

Agreements:

Proposal 1: The wanted RF input signal nominal frequency for NCR-Fwd can reuse BS’s NR-ARFCN.

Proposal 2: The wanted RF input signal nominal frequency for NCR-MT can reuse UE’s NR-ARFCN.

Proposal 3: Table 1 below can be the baseline for future discussion and work split.

Open issues:

Companies are encouraged to further check the open issues below.

Issue 1: Further check the RSE limits of NCR-Fwd, wide area NCR-MT and local area NCR-MT, especially on different type of NCR (1-C,1-H and 2-O).

Issue 2: Further check the testability of monitoring NCR-Fwd and NCR-MT simultaneously during immunity test.

Issue 3: Further investigate the performance metric for NCR-Fwd and NCR-MT, whether power accuracy, gain, throughput and data rate are suitable or not.

Table 1: Guidance on NCR EMC Perf

| 4. Test conditions |  |
| --- | --- |
| 4.1 General | Some editorial changes corresponding to NCR are required. |
| 4.2 Arrangements for establishing a communication link | For NCR-Fwd reuse the NR-ARFCN for BS. For NCR-MT reuse the NR-ARFCN for UE. |
| 4.3 Narrow band responses | Some editorial changes corresponding to NCR are required. |
| 4.4 Exclusion bands | Reuse the exclusion band of NR repeater but introduce the content of NCR |
| 4.5 NR repeaters test configurations | Wait until the discussion of RF conformance test |
| 5. Performance assessment |  |
| 5.1 General | Some editorial changes corresponding to NCR are required. |
| 5.2 NR repeaters | Wait until the decision on open issue. |
| 5.3 Ancillary equipment | No change |
| 6. Performance criteria |  |
| 6.1 Performance criteria for continuous phenomena for NR repeaters | No change |
| 6.2 Performance criteria for transient phenomena for NR repeaters | No change |
| 6.3 Performance criteria for continuous phenomena for NCR | Wait until the decision on open issue. |
| 6.4 Performance criteria for transient phenomena for NCR |  |
| 6.5 Performance criteria for continuous phenomena for Ancillary equipment | No change |
| 6.6 Performance criteria for transient phenomena for Ancillary equipment | No change |
| 8. Emission |  |
| 8.1 Test configurations | Some editorial changes corresponding to NCR are required. |
| 8.2 Radiated emission | Wait until the decision on open issue. |
| - Immunity |  |
| 9.1 Test configurations | Some editorial changes corresponding to NCR |

- References

- R4-2401144	Discussion on NCR EMC Perf implementation guidance

- R4-2401148	Discussion on NCR EMC Perf detailed analysis

- R4-2402246	Discussion on NCR EMC Immunity setup

- R4-2402649	Summary for [110][309] NCR RF conformance