3GPP TSG-RAN4 Meeting#110	R4-2402974

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | ❌ | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on 38.104 FRC for FR2-1 UL 256QAM demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FR2-1 UL 256QAM is introduced in Rel-18, the FRCs need to be defined for demodulation requirement. |  |  |  |  |  |  |  |  |
| , |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defining FRC for 60kHz SCS 50MHz BW; Defining FRC for 120kHz SCS 50MHz, 100MHz BW and 200MHz BW. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete demodulation requirements the corresponding FRCs are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2402028 |  |  |  |  |  |  |  |  |

<Start of change>

A.X	Fixed Reference Channels for performance requirements (256QAM, R=682.5/1024)

The parameters for the reference measurement channels are specified in table A.X-1and A.X-2 for FR2-1 PUSCH performance requirements:

-	FRC parameters are specified in table A.X-1 for FR2-1 PUSCH with transform precoding disabled, Additional DM-RS position = pos0 and 1 transmission laye.

-	FRC parameters are specified in table A.X-2 for FR2-1 PUSCH with transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer.

Table A.X-1: FRC parameters for FR2-1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos0 and 1 transmission layer (256QAM, R=682.5/1024)

| Reference channel | G-FR2-AX-1 | G-FR2-AX-2 | G-FR2-AX-3 |
| --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 | 60 |
| Allocated resource blocks | 32 | 132 | 66 |
| Data bearing CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 |
| Modulation | 256QAM | 256QAM | 256QAM |
| Code rate (Note 2) | 682.5/1024 | 682.5/1024 | 682.5/1024 |
| Payload size (bits) | 18432 | 75792 | 37896 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 9 | 5 |
| Code block size including CRC (bits) (Note 2) | 6176 | 8448 | 7608 |
| Total number of bits per slot without PT-RS | 27648 | 114048 | 57024 |
| Total number of bits per slot with PT-RS (Note 3) | 26496 | 109296 | 54648 |
| Total resource elements per slot without PT-RS | 3456 | 14256 | 7128 |
| Total resource elements per slot with PT-RS (Note 3) | 3312 | 13662 | 6831 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos0 with l0= 0 as per Table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |  |

Table A.X-2: FRC parameters for FR2-1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (256QAM, R=682.5/1024)

| Reference channel | G-FR2-AX-4 | G-FR2-AX-5 | G-FR2-AX-6 |
| --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 120 | 120 | 60 |
| Allocated resource blocks | 32 | 132 | 66 |
| Data bearing CP-OFDM Symbols per slot (Note 1) | 8 | 8 | 8 |
| Modulation | 256QAM | 256QAM | 256QAM |
| Code rate (Note 2) | 682.5/1024 | 682.5/1024 | 682.5/1024 |
| Payload size (bits) | 16392 | 67584 | 33816 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 |
| Number of code blocks - C | 2 | 9 | 5 |
| Code block size including CRC (bits) (Note 2) | 8232 | 7536 | 6792 |
| Total number of bits per slot without PT-RS | 24576 | 101376 | 50688 |
| Total number of bits per slot with PT-RS (Note 3) | 23552 | 97152 | 48576 |
| Total resource elements per slot without PT-RS | 3072 | 12672 | 6336 |
| Total resource elements per slot with PT-RS (Note 3) | 2944 | 12144 | 6072 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1 with l0= 0 and l =8 as per Table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 3: PT-RS configuration KPT-RS =2, LPT-RS =1. |  |  |  |

<End of change>