3GPP TSG-RAN WG4 Meeting # 110	R4-2402993

# Athens, Greece, February 26 – March 1

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-4: PMI reporting requirements for FR2 multipanel reception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The PMI reporting requirements for FR2 multipanel reception agreed to be defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new test - New Chapter 8.3.3.2.2 - 2 new reference channels added in A.3.2.2.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements for FR2 multipanel reception will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.2.2 (new), A.3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

##### 8.3.3.2.2	Single PMI with 2 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme with simultaneous reception

For the parameters specified in Table 8.3.3.2.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 8.3.3.2.2-2.

Table 8.3.3.2.2-1: Test parameters (dual-layer)

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| SSB |  |  |  |  |  | SSB#0 |  | SSB#1 |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #2 |  | N/A |
|  |  |  | CORESETPoolIndex |  |  | 0 |  | N/A |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 4 for CSI-RS resources 1 and 3 l0 = 8 for CSI-RS resources 2 and 4 |  | l0 = 4 for CSI-RS resources 5 and 7 l0 = 8 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 160 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 80 for CSI-RS resources 1 and 2 81 for CSI-RS resources 3 and 4 |  | 80 for CSI-RS resources 5 and 6 81 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  | TCI state #1 |
| CSI-RS for beam refinement |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1 and 2 |  | k0=1 for CSI-RS resources 3 and 4 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |  | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resources 1,2,3,4 |  |  |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resources 1,2,3,4 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 160 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 0 for CSI-RS resources 1,2,3,4 |  |  |
|  |  |  | QCL info |  |  | TCI state #2 |  | TCI state #3 |
| Duplex mode |  |  |  |  |  | TDD |  |  |
| Bandwidth |  |  |  |  | MHz | 100 |  |  |
| Subcarrier spacing |  |  |  |  | kHz | 120 |  |  |
| TDD DL-UL configurations |  |  |  |  |  | FR1.120-1 as specified in Annex A.1.3 |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-35 |  |  |
| Correlation matrix and antenna configuration (Note 6) |  |  |  |  |  | 4x4 FR2- mTRxP-mRX ρ = -12dB |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #0 |  | TCI State #1 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| PTRS configuration (Note 5) | Frequency density (KPT-RS) |  |  |  |  | 2 |  | Test 1: N/A Test 2: 2 |
|  | Time density (LPT-RS) |  |  |  |  | 1 |  | Test 1: N/A Test 2: 1 |
|  | Resource Element Offset |  |  |  |  | 2 |  | Test 1: N/A Test 2: 3 |
| TCI State #0 | Type 1 QCL information |  |  | SSB index |  | SSB #0 |  | N/A |
|  |  |  |  | QCL Type |  | Type C |  | N/A |
|  | Type 2 QCL information |  |  | SSB index |  | SSB #0 |  | N/A |
|  |  |  |  | QCL Type |  | Type D |  | N/A |
| TCI State #1 | Type 1 QCL information |  |  | SSB index |  | N/A |  | SSB #1 |
|  |  |  |  | QCL Type |  | N/A |  | Type C |
|  | Type 2 QCL information |  |  | SSB index |  | N/A |  | SSB #1 |
|  |  |  |  | QCL Type |  | N/A |  | Type D |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type D |  | N/A |
| TCI State #3 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type D |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.3 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 4, (8,-) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (13) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #9 | Resource #10 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 |  |
|  |  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3, (8,-) | Row 3, (10,-) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (12) | (12) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 1 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (8,13) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #9}, with K1=1 CMR group #2: {NZP CSI-RS resource #10}, with K2=1 CMR paring: {NZP CSI-RS resource #9, NZP CSI-RS resource #10} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 8 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | CodebookType |  |  |  | typeI-SinglePanel |  |  |
|  |  | CodebookMode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (1,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (1,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 001111 |  |  |
|  |  | RI Restriction |  |  |  | N/A |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 1.75 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | Test 1: R.PDSCH.5-7.2 TDD Test 2: R.PDSCH.5-7.3 TDD |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.125 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. Note 5: PT-RS configuration in Test 1 uses single port (one TRxP) and Test 2 used dual port (both TRxPs) Note 6: Correlation matrix according to the [FR2-mTxRP-mRX] in B.x.x.x. TRxP#1 uses TX antenna indices (1,2) and TRxP#2 uses TX antenna indices (3,4) corresponding to the respective antenna configuration matrix rows |  |  |  |  |  |  |  |  |

Table 8.3.3.2.2-2: Minimum requirement

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
|  | [1.15] | [1.15] |

END OF CHANGE 1

START OF CHANGE 2

Table A.3.2.2.5-7: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-7.1 TDD | R.PDSCH.5-7.2 TDD | R.PDSCH.5-7.3 TDD |  |  |
| Channel bandwidth | MHz | 100 | 100 | 100 |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |  |
| Allocated resource blocks | PRBs | 66 | 66 | 66 |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames |  | 63 | 63 | 63 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 13 |  |  |
| Modulation |  | 16QAM | 16QAM | 16QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 |  |  |
| Number of MIMO layers |  | 1 | 2 | 2 |  |  |
| Number of DMRS REs (Note 3) |  | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  | 6 | 6 | 12 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i = 80 | Bits | 14344 | 28680 | 27144 |  |  |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 14344 | 28680 | 27144 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i = 80 | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |  |
| For Slot i = 80 | CBs | 2 | 4 | 4 |  |  |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | CBs | 2 | 4 | 4 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i = 80 | Bits | 28824 | 57552 | 54912 |  |  |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 30360 | 60720 | 58080 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 45.1836 | 90.342 | 85.504 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

END OF CHANGE 2