3GPP TSG-RAN4 Meeting #110	R4-2403053

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0501 | rev | 1 | Current version: | 15.20.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf) CR to TS38.101-4: Corrections on FRC definition (Rel-15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc., Huawei, HiSilicon, Ericsson, Anritsu, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS38.101-4, the Number of PRB for CSI-RS in Frequency Occupation is “ceil(BWP size/4)*4”. Also, in TS38.331, it is mentioned that “If the configured value is larger than the width of the corresponding BWP, the UE shall assume that the actual CSI-RS bandwidth is equal to the width of the BWP”. Therefore, the number of CSI-RS RB in FRC is equal to the number of allocated resource blocks (may not be mutiples of 4) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update with correct values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Values will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2402549 |  |  |  |  |  |  |  |  |

START OF CHANGE 1

## A.3.2	Reference measurement channels for PDSCH performance requirements

<Unchanged part skipped>

#### A.3.2.1.2	Reference measurement channels for SCS 30 kHz FR1

Table A.3.2.1.2-1: PDSCH Reference Channel for FDD (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-1.1 FDD |  |  |  |  |
| Channel bandwidth | MHz | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 51 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 39 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 19 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.51 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 39 | Bits | 40976 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 39 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A |  |  |  |  |
| For Slots i = 1,…, 39 | CBs | 5 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 20, 21 | Bits | 77112 |  |  |  |  |
| For Slots i = 1,…, 19, 22, …, 39 | Bits | 80784 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 79.903 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

A.3.2.2	TDD

A.3.2.2.1	Reference measurement channels for SCS 15 kHz FR1

A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

Table A.3.2.2.2-1: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and FR1.30-1A (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-1.1 TDD | R.PDSCH.2-1.2 TDD | R.PDSCH.2-1.3 TDD |  |  |
| Channel bandwidth | MHz | 40 | 40 | 40 |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |
| Allocated resource blocks | PRBs | 106 | 6 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 7 |  |  |
| Allocated slots per 2 frames |  | 31 | 31 | 27 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 4 | 4 | 4 |  |  |
| Modulation |  | QPSK | QPSK | QPSK |  |  |
| Target Coding Rate |  | 0.30 | 0.30 | 0.30 |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 18 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 2664 | 144 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 8064 | 480 | 4608 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 16 | 16 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 16 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 1 | 1 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 1 | 1 | 1 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 20, 21 | Bits | 25440 | 1512 | 13392 |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 8904 | 504 | N/A |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 26712 | 1584 | 15264 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 11.419 | 0.677 | 6.221 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-2: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-2.1 TDD | R.PDSCH.2-2.2 TDD | R.PDSCH.2-2.3 TDD | R.PDSCH.2-2.4 TDD |  |
| Channel bandwidth | MHz | 40 | 40 | 40 | 40 |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |  |
| Allocated resource blocks | PRBs | 106 | 106 | 106 | 106 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 |  |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 13 | 13 | 13 | 13 |  |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 |  |
| Number of MIMO layers |  | 1 | 2 | 3 | 4 |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 12 | 12 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 24 | 24 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 8456 | 16896 | 22032 | 29192 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 26632 | 53288 | 73776 | 98376 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 2 | 3 | 3 | 4 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 4 | 7 | 9 | 12 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 20, 21 | Bits | 53424 | 106848 | 145008 | 193344 |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 17808 | 35616 | 45792 | 61056 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 55968 | 111936 | 152640 | 203520 |  |
| Max. Throughput averaged over 2 frames | Mbps | 37.644 | 75.318 | 104.004 | 138.646 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-3: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-3.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 19 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.51 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 27144 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 83976 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 10 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slots i = 20, 21 | Bits | 160272 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 53424 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 167904 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 118.796 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-4: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (256QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-4.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 256QAM |  |  |  |  |
| MCS index |  | 24 |  |  |  |  |
| Modulation |  | 256QAM |  |  |  |  |
| Target Coding Rate |  | 0.82 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 29192 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 92200 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 11 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slots i = 20, 21 | Bits | 106848 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 35616 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 111936 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 130.308 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-5: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-2

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-5.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} |  | 8 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | Bits | 5376 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} | Bits | 8456 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,39} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i = 20, 21 | Bits | 26712 |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | Bits | 17808 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19,22,…,39} | Bits | 27984 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 11.875 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-6: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-3

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-6.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} |  | 8 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 27 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | Bits | 5376 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,39} | Bits | 8456 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,39} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i = 20, 21 | Bits | 26712 |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | Bits | 17808 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,19,22,…,39} | Bits | 27984 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 10.184 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-7: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and CSI-RS overlapped with PDSCH

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-7.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 16896 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 53288 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 3 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 7 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,5} for i from {1,…,19,22,…,39} | Bits | 103456 |  |  |  |  |
| For Slots i = 20 | Bits | 98368 |  |  |  |  |
| For Slots i = 21 | Bits | 106848 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 35616 |  |  |  |  |
| For Slot i, if mod(i, 10) = {1,2,3,4,6} for i from {1,…,19,22,…,39} | Bits | 111936 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 75.318 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-8: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR1.30-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-8.1 TDD | R.PDSCH.2-8.2 TDD |  |  |  |
| Channel bandwidth | MHz | 40 | 40 |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 |  |  |  |
| Allocated resource blocks | PRBs | 106 | 106 |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames |  | 23 | 23 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 13 | 13 |  |  |  |
| Modulation |  | 16QAM | 16QAM |  |  |  |
| Target Coding Rate |  | 0.48 | 0.48 |  |  |  |
| Number of MIMO layers |  | 1 | 2 |  |  |  |
| Number of DMRS REs (Note 3) |  | 24 | 24 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i = 20 | Bits | 24576 | 49176 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 24576 | 49176 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i = 20 | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | CBs | N/A | N/A |  |  |  |
| For Slot i = 20 | CBs | 3 | 6 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | CBs | 3 | 6 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i = 20 | Bits | 48336 | 96672 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 50880 | 101760 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 28.2624 | 56.5524 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

Table A.3.2.2.2-9: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-4 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-9.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 51 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {4,5} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 3 for i from {0,…,39} |  | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,6,7,8,9} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 19 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.51 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {4,5} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 3 for i from {0,…,39} |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,6,7,8,9} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {4,5} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 3 for i from {0,…,39} | Bits | 13064 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,6,7,8,9} for i from {1,…,39} | Bits | 40976 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {4,5} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 3 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,6,7,8,9} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {4,5} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 3 for i from {0,…,39} | CBs | 2 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,6,7,8,9} for i from {1,…,39} | CBs | 5 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {4,5} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slots i = 20, 21 | Bits | 77112 |  |  |  |  |
| For Slot i, if mod(i, 10) = 3 for i from {0,…,39} | Bits | 25704 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,6,7,8,9} for i from {1,…,19,22,…,39} | Bits | 80784 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 57.930 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<Unchanged part skipped>

Table A.3.2.2.2-11: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-5

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-11.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,39} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {0,…,39} |  | 10 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,39} |  | 18 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {0,…,39} |  | 18 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,39} | Bits | 8064 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {0,…,39} | Bits | 6528 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,39} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {0,…,39} | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i = 20 | Bits | 25440 |  |  |  |  |
| For Slot i = 21 | Bits | 20352 |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,19,22,…,39} | Bits | 26712 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {0,…,19,22,…,39} | Bits | 21624 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 6.893 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-12: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-6

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-12.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,39} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {0,…,39} |  | 8 |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {0,…,39} |  | 10 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,39} |  | 18 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {0,…,39} |  | 18 |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {0,…,39} |  | 18 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,39} | Bits | 8064 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {0,…,39} | Bits | 4992 |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {0,…,39} | Bits | 6528 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,39} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {0,…,39} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {0,…,39} | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i = 20 | Bits | 25440 |  |  |  |  |
| For Slot i = 21 | Bits | 15264 |  |  |  |  |
| For Slot i, if mod(i, 4) = 0 for i from {1,…,19,22,…,39} | Bits | 26712 |  |  |  |  |
| For Slot i, if mod(i, 4) = 1 for i from {1,…,19,22,…,39} | Bits | 16536 |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {0,…,39} | Bits | 21624 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 9.389 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

A.3.2.2.3	Reference measurement channels for SCS 60 kHz FR1

A.3.2.2.4	Reference measurement channels for SCS 60 kHz FR2

Table A.3.2.2.4-1: PDSCH Reference Channel for TDD UL-DL pattern FR2.60-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.4-1.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 50 |  |  |  |  |
| Subcarrier spacing | kHz | 60 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,79} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 79} |  | 10 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79} |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 59 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,79} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 79} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,79} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 79} | Bits | 25608 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79} | Bits | 34816 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,79} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 79} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,79} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 79} | CBs | 4 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79} | CBs | 5 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,79} | Bits | N/A |  |  |  |  |
| For Slot i = 40, 41 | Bits | 69960 |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {4,…, 79} | Bits | 54912 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,39,42,…,79} | Bits | 73128 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 93.499 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

#### A.3.2.2.5	Reference measurement channels for SCS 120 kHz FR2

Table A.3.2.2.5-1: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 and FR2.120-1A (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-1.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 127 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 3624 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 5504 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 16 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slots i = 80, 81 | Bits | 17490 |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 12210 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79,82,…,159} | Bits | 18282 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 31.942 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.5-2: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-2.1 TDD | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.3 TDD |  |  |
| Channel bandwidth | MHz | 100 | 100 | 200 |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |  |
| Allocated resource blocks | PRBs | 66 | 66 | 132 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 | 13 |  |  |
| Allocated slots per 2 frames |  | 127 | 127 | 127 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 13 |  |  |
| Modulation |  | 16QAM | 16QAM | 16QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 |  |  |
| Number of MIMO layers |  | 1 | 2 | 2 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 | 12 | 12 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 11272 | 22536 | 45096 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 17424 | 34816 | 69672 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 2 | 3 | 6 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 3 | 5 | 9 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 | Bits | 36564 | 69960 | 139920 |  |  |
| For Slots i = 82 | Bits | 34980 | 73128 | 146256 |  |  |
| For Slots i = 83 | Bits | 22836 | 48840 | 97680 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24420 | 48840 | 97680 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79,84,…,159} | Bits | 36564 | 73128 | 146256 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 100.799 | 201.434 | 403.096 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.5-3: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-3.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 127 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 18 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.46 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 16136 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 25104 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 2 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 3 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slots i = 80, 81 | Bits | 52470 |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 36630 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79,82,…,159} | Bits | 54846 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 145.062 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<Unchanged part skipped>

Table A.3.2.2.5-5: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-2 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-5.1 TDD | R.PDSCH.5-5.2 TDD |  |  |  |
| Channel bandwidth | MHz | 100 | 50 |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 |  |  |  |
| Allocated resource blocks | PRBs | 66 | 32 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} |  | 10 | 10 |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,159} |  | 13 | 13 |  |  |  |
| Allocated slots per 2 frames |  | 119 | 119 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 13 | 13 |  |  |  |
| Modulation |  | 16QAM | 16QAM |  |  |  |
| Target Coding Rate |  | 0.48 | 0.48 |  |  |  |
| Number of MIMO layers |  | 2 | 2 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} |  | 12 | 12 |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,159} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 6 | 6 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | Bits | 25608 | 12552 |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,159} | Bits | 34816 | 16896 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,159} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | CBs | 4 | 2 |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,159} | CBs | 5 | 3 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i = 80, 81 | Bits | 69960 | 33920 |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {4,…, 159} | Bits | 54912 | 26624 |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79,82,…,159} | Bits | 73128 | 35456 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 188.739 | 91.843 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.5-6: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-2 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-6.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} |  | 10 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,159} |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 119 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 17 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.43 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,159} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | Bits | 34816 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,159} | Bits | 47112 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,159} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | CBs | 5 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,159} | CBs | 6 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80, 81 | Bits | 114940 |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {4,…, 159} | Bits | 82368 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {1,…,79,82,…,159} | Bits | 109692 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 255.724 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<Unchanged part skipped>

Table A.3.2.2.5-8: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-2 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-8.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 59 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs (Note 3) |  | 24 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 14344 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | Bits | 14344 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i = 80 | CBs | 2 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 28776 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | Bits | 30360 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 42.3148 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

END OF CHANGE 1