3GPP TSG-RAN WG4 Meeting # 110												R4-2403295

Athens, Greece, 26th Feb to 1st Mar 2024

Agenda item:			8.23.5

Source:	Moderator (Ericsson)

Title:	WF on R18 NR RedCap RRM requirements

Document for:	Approval

Relaxation scenarios for INACTIVE mode neighbour cell measurements

  - RAN4 to agree define relaxed neighbour cell measurement requirements for Rel-18 RedCap UE in INACTIVE mode for the relaxation scenarios listed below:

The following relaxation cases (as R17 redcap) are to be specified for R18 eRedCap INACTIVE mode with inactive eDRX > 10.24s:

- Measurements for UE fulfilling stationary criterion

- Measurements for a UE fulfilling not-at-cell edge while stationary criterion

- Measurements for a UE fulfilling stationary and not-at-cell-edge criteria

- Measurements for a UE fulfilling low mobility and stationary criteria

- Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria

- Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria

- Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria

- Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion

- Measurements for UE fulfilling low mobility criterion

- Measurements for UE fulfilling not-at-cell edge criterion

- Measurements for UE fulfilling low mobility and not-at-cell edge criteria

Relaxation factor for INACTIVE mode neighbour cell measurements

Relaxation factor from idle mode and fixed relaxation period from idle mode are used for inactive mode. The relaxation within single RAN PTW is only for evaluation and measurement, not for searching.

INACTIVE mode tests

  - No tests cases are defined for release 18 RedCap UE with enhanced eDRX.

Test for transition requirements

  - No additional performance requirement needs to be specified for transition.