# 3GPP TSG-RAN WG4 Meeting # 110	R4-2403296

# Athens, Greece, 26th Feb. – 1st March, 2024

Agenda item:			9.4.6

Source:	Moderator (MediaTek inc.)

Title:	WF on R18 IoT NTN RRM requirements

Document for:	Approval

# Introduction

This document is to capture all agreements in email thread [110][233] IoT_NTN_enh.

# Topic #1: RRM core requirements

No agreement in this meeting

# Topic #2: RRM performance requirements

Issue 2-1-1: Test case principle

Agreement:

Discuss the test cases for both IoT NTN and eMTC NTN in RAN4 RRM.

#### Issue 2-1-2: SIB33

Agreement (to check if it is agreeable):

- Introduce the general parameters for SIB33 setup for neighbour satellite assistance information as follows:

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | NSC.1 | NSC.2 |
| Scenario |  | GSO | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| neighValidityDuration-r18 | s | 900 | 5 |
| k-Mac-r18 | slot | Not configured | Not configured |
| nta-Common-r18 |  | 0 | 0 |
| nta-CommonDrift-r18 |  | 0 | 0 |
| nta-CommonDriftVariation-r18 |  | 0 | 0 |
| ephemerisInfo |  | According to Annex B.8 |  |

Issue 2-1-3: propagation channel

Agreement:

Propagation channel for eMTC and NB-IoT NTN:

- Revise the propagation condition to AWGN for the existing tests of intra-frequency event triggered reporting for Cat-M1 UE (A.14.5 in TS36.133).

- For the new test cases which to be introduced, set AWGN as propagation condition.

Issue 2-2-1: For NB/eMTC, inter-frequency tests with neighbour cells

Agreement:

- Define test cases which are suspended in Rel-17 due to lack of neighbour cell assistant information.

- Define the following TCs for NB-IoT inter-frequency tests with neighbour cells

| Cell Re-Selection (A.13.1.1) | HD – FDD Inter frequency case for UE Category NB1 in normal coverage | NB-1-1 |
| --- | --- | --- |
| RRC Re-establishment (A.13.3.1) | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage | NB-3-1 |

- Define the following TC for NB inter-frequency tests with neighbour cells

| RRC Re-establishment (A.13.3.1) | A.13.3.1.2x HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | NB-3-2 |
| --- | --- | --- |

- Discuss the down-selection from the following TCs for eMTC inter-frequency tests with neighbour cells

| Cell re-selection (A.14.1.1) | E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M-1-1 |
| --- | --- | --- |
|  | E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M-1-2 |
| Handover (A.14.2.1) | E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA | M-2-1 |
|  | E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA | M-2-2 |
|  | E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB | M-2-3 |
|  | E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB | M-2-4 |
|  | E-UTRAN FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M-2-5 |
|  | E-UTRAN HD-FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M-2-6 |
| RRC re-establishment (A.14.3.1) | E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M-3-1 |
|  | E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M-3-2 |
| UE measurement procedure in RRC-CONNECTED (A.14.5.2) | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under [fading propagation or AWGN] conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M-5-1 |
|  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under [fading propagation or AWGN]conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M-5-2 |
|  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under [fading propagation or AWGN]conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M-5-3 |
|  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under [fading propagation or AWGN] conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M-5-4 |
|  | E-UTRAN FDD-FDD inter-frequency event triggered reporting under [fading propagation or AWGN]conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB | M-5-5 |
|  | E-UTRAN HD-FDD inter-frequency event triggered reporting under [fading propagation or AWGN] conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB | M-5-6 |
|  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under [fading propagation or AWGN] conditions in asynchronous cells with burst gap | M-5-7 |

| Measurement Performance Requirements (A.14.6.1) | FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M-6-1 |
| --- | --- | --- |
|  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M-6-2 |
|  | FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB | M-6-3 |
|  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB | M-6-4 |

Issue 2-2-3: For NB/eMTC, NGSO test configuration

Agreement:

- Both GSO test configuration and NGSO test configuration should be supported for the test cases.

Issue 2-2-4: For NB-IoT, neighbour cell measurement in CONNNECTED mode

Agreement:

Test both inter-frequency and intra-frequency.

Issue 2-3-1: For NB/eMTC, test cases for time/location-based triggering of cell reselection in IDLE mode

Agreement:

For NB-IoT,

| Cell Re-Selection (A.13.1.1) | HD – FDD Intra frequency case for UE Category NB1 in normal coverage, time-based triggering | NB-IDLE-1T |
| --- | --- | --- |
|  | HD – FDD Intra frequency case for UE Category NB1 in [enhanced] coverage, location-based triggering | NB-IDLE-1D |
|  | HD – FDD Inter frequency case for UE Category NB1 in [enhanced] coverage, time-based triggering | NB-IDLE-2T |
|  | HD – FDD Inter frequency case for UE Category NB1 in normal coverage, location-based triggering | NB-IDLE-2D |

For eMTC, discuss the down-selection from the following list:

| Cell Re-Selection (A.14.1.1) | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage, time-based triggering | M-IDLE-1T |
| --- | --- | --- |
|  | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in [enhanced] coverage, location-based triggering | M-IDLE-1D |
|  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in [enhanced] coverage, time-based triggering | M-IDLE-2T |
|  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage, location-based triggering | M-IDLE-2D |
|  | E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in [enhanced] coverage, time-based triggering | M-IDLE-3T |
|  | E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage, location-based triggering | M-IDLE-3D |
|  | E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage, time-based triggering | M-IDLE-3T |
|  | E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in [enhanced] coverage, location-based triggering | M-IDLE-3D |

Issue 2-3-2: For NB/eMTC, test cases for time/location-based triggering of neighbour cell measurements in CONNECTED mode

Agreement:

For NB/eMTC, test cases for time/location-based triggering of neighbour cell measurements in CONNECTED mode

For NB-IoT,

| Measurement Procedure | HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, time-based triggering | NB-CONN-1T |
| --- | --- | --- |
|  | HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, location-based triggering | NB-CONN-1D |
|  | HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, time-based triggering | NB-CONN-2T |
|  | HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, location-based triggering | NB-CONN-2D |

For eMTC, discuss the down-selection based on the following test, and replace “fading propagation” by “AWGN”

| Measurement Procedure (A.14.5.1) | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA, time-based triggering | M-CONN-1T |
| --- | --- | --- |
|  | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA, location-based triggering | M-CONN-1D |
|  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA, time-based triggering | M-CONN-2T |
|  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA, location-based triggering | M-CONN-2D |
|  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA, time-based triggering | M-CONN-3T |
|  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA, location-based triggering | M-CONN-3D |
|  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA, time-based triggering | M-CONN-4T |
|  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA, location-based triggering | M-CONN-4D |

Issue 2-3-3: For eMTC, test cases for time/location-based CHO

Agreement:

- Further discuss the down-selection from the following TCs for eMTC location/time-based CHO

- Change some of the “Time/location only-based conditional handover” tests to “time/location and RSRP based conditioned HO”

| E-UTRAN FDD-FDD Intra frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-11 |
| --- | --- |
| E-UTRAN FDD-FDD Intra frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-12 |
| E-UTRAN HD-FDD Intra frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-13 |
| E-UTRAN HD-FDD Intra frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-14 |
| E-UTRAN FDD-FDD Inter frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-15 |
| E-UTRAN FDD-FDD Inter frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-16 |
| E-UTRAN HD-FDD Inter frequency Time only-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-17 |
| E-UTRAN HD-FDD Inter frequency Location only-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-18 |

Issue 2-3-4: For NB/eMTC, test cases upon t-ServiceStart

<Wayforward>

- FFS RAN4 to introduce tests as follows for verify the UE can be scheduled on the serving cell depending on whether t-ServiceStart is reached or not:

- Subtest 1: UE is scheduled on the serving cell and t-ServiceStart is not reached .

- Subtest 2: t-ServiceStart is reached and UE performs negibhour cell measurements.

#### Issue 2-3-5: Margin for location-based test cases with earth moving cell

Agreement:

- Regarding whether to consider additional margin for location related test cases for earth moving cell, conclusion in NR NTN can be reused.

Issue 2-3-6: Cell setting

<Wayforward>

FFS

- For NB-IoT/eMTC introduce distance-based measurement triggering test cases for RRC Idle and RRC connected modes in Earth moving cell scenarios.

- For NB-IoT/eMTC introduce time-based measurement triggering test cases for RRC Idle and RRC connected modes in Earth Fixed cell scenarios.

### Sub-Topic 2-4: Test cases with GNSS gap

#### Issue 2-4-1: For eMTC, test case with GNSS gap

<Wayforward>

- FFS RAN4 to introduce tests as follows for verify the GNSS measurement when the GNSS gaps overlaps with the mobility measurement gaps for eMTC, considering selecting a short GNSS gap length to make test practical:

  - Subtest 1: gaps are colliding and GNSS measurements are performed and neighbour cell measurements are not performed.

  - Subtest 2: GNSS measurement is completed and UE performs neighbour cell measurements.

### Sub-Topic 2-5: Test cases Worksplit

Agreement:

For NB-IoT

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| Cell Re-Selection (A.13.1.1) | HD – FDD Inter frequency case for UE Category NB1 in normal coverage | NB-1-1 | MTK | Phase 1 (2024.04) |
|  | HD – FDD Intra frequency case for UE Category NB1 in [enhanced] coverage, location-based triggering | NB-IDLE-1D |  |  |
|  | HD – FDD Inter frequency case for UE Category NB1 in [enhanced] coverage, time-based triggering | NB-IDLE-2T |  |  |
| RRC Re-establishment (A.13.3.1) | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage | NB-3-1 |  |  |
|  | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | NB-3-2 |  |  |

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| Measurement Procedure in CONN (new) | HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage | NB-CONN-1 | Huawei | Phase 2 (2024.05) |
|  | HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage | NB-CONN-2 |  |  |
| Measurement Procedure in CONN with time/location triggering (new) | HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, time-based triggering | NB-CONN-1T |  |  |
|  | HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, location-based triggering | NB-CONN-2D |  |  |

For eMTC

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| Cell re-selection (A.14.1.1) | E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M-1-1 | CMCC | Phase 1 (2024.04) |
|  | E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M-1-2 |  |  |
|  | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage, time-based triggering | M-IDLE-1T |  |  |
|  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in [enhanced] coverage, time-based triggering | M-IDLE-2T |  |  |
|  | E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage, location-based triggering | M-IDLE-3D |  |  |
|  | E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage, location-based triggering | M-IDLE-4D |  |  |
| RRC re-establishment (A.14.3.1) | E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M-3-1 | Ericsson | Phase 1 (2024.04) |
|  | E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M-3-2 |  |  |

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| Handover (A.14.2.1) | E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA | M-2-1 | Nokia | Phase 1 (2024.04) |
|  | E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA | M-2-2 |  |  |
|  | E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB | M-2-3 |  |  |
|  | E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB | M-2-4 |  |  |
|  | E-UTRAN FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M-2-5 |  |  |
|  | E-UTRAN HD-FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M-2-6 |  |  |
|  | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | M-2-7 |  |  |
|  | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | M-2-8 |  |  |
| Time/location based conditional Handover (A.14.2.1) | E-UTRAN FDD-FDD Intra frequency Time-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-11 |  | Phase 2 (2024.05) |
|  | E-UTRAN HD-FDD Intra frequency Location-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-14 |  |  |
|  | E-UTRAN FDD-FDD Inter frequency Location-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-16 |  |  |
|  | E-UTRAN HD-FDD Inter frequency Time-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-17 |  |  |
|  | Note: for Time/location based conditional handover TCs, both time/location only-based conditional handover” tests and “time/location and RSRP based conditioned HO tests” are considered |  |  |  |

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| UE measurement procedure in RRC-CONNECTED (A.14.5.1) | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M-5-1 |  | Phase 1 (2024.04) |
|  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M-5-2 |  |  |
|  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M-5-3 |  |  |
|  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M-5-4 |  |  |
|  | E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB | M-5-5 |  |  |
|  | E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB | M-5-6 |  |  |
|  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells with burst gap | M-5-7 |  |  |

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| UE measurement procedure in RRC-CONNECTED with time/location based triggering (A.14.5.1) | E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA, location-based triggering | M-CONN-1D | Nokia | Phase 2 (2024.05) |
|  | E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA, location-based triggering | M-CONN-2D |  |  |
|  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA, time-based triggering | M-CONN-3T |  |  |
|  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA, time-based triggering | M-CONN-4T |  |  |

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| Measurement Performance Requirements (A.14.6.1) | FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M-6-1 | MTK | Phase 1 (2024.04) |
|  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M-6-2 |  |  |
|  | FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB | M-6-3 |  |  |
|  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB | M-6-4 |  |  |