3GPP TSG-RAN WG4 Meeting # 110                                                              R4-2403319

Athens, Greece, February 26 - March 1, 2024

Title: 	FR2 HST RRM performance requirements

Agenda Item:	8.8.2

Source: 	Samsung

Document for:	Approval

Introduction

For Rel-18 work item on enhanced NR support for high speed train scenario in frequency range 2, there are two threads  [213] and [214] organized for RRM discussion on RAN4#110, and this document is used to capture the WF on topics in thread [214].

Topic #1: RRM Performance Requirements for Rel-18 FR2 HST

Sub-topic 1-1 How to design the TC for SSB based L1-RSRP in Rel-18 FR2 HST

Issue 1-1-1 AoA setup for multi-Rx chain DL reception in Rel-18 FR2 HST

Agreement in Ad-hoc minutes:

Agreement: For FR2 PC6 UE, - 2AoA Setup 3, the Relative angular offset between active probes: - Only keep 150 degree as option

- Whether to use 2AoA setup for SSB based L1-RSRP test case

- Agreement:

  - For the test design, configuration set 2 with 2AoA setup should be applied for SSB based L1-RSRP test case in FR2 HST.

- Whether to define a new 2AoA setup for multi-Rx chain DL reception for SSB based L1-RSRP test case

- Way Forward:

  - Option 1: Yes.

Define a new AoA setup to support the SSB based L1-RSRP measurement test on FR2-1 PC6 UEs supporting SimultaneousReceptionFR2HST-r18 - AoA Setup X: 2 AoAs, both AoAs are in Rx beam peak direction - The details on how to define the AoA Setup X for test case, the conclusion from Rel-18 Multi-RX WI could be considered

  - Option 2: No.

Issue 1-1-2 Whether to configure higher layer parameter timeRestrictionForChannelMeasurements

- Agreement:

  - For the SSB based L1-RSRP test case in Rel-18 FR2 HST with highSpeedMeasFlagFR2-r17 is configured to set2, the higher layer parameter timeRestrictionForChannelMeasurements should be configured.

    - Only one sample (M=1) is defined as the measurement period

Sub-topic 1-2 Scope of RRM Performance Requirements for intra-band CA in Rel-18 FR2 HST

Issue 1-2-1 Whether to define test case for enhancement on SCell measurement

- Agreement:

  - No need to define test case for SCell enhanced measurement in Rel-18 FR2 HST.

Issue 1-2-2 Whether to define test case for enhancement on intra-frequency measurements with/without measurement gaps

- Agreement:

  - RAN4 to define a new test case for A.7.6.1.X SA event triggered reporting test without gap under non-DRX for UE supporting [measurementEnhancementCAInterFreqFR2-r18]

Issue 1-2-3 Whether to design the test case for SCell activation delay

- Agreement:

  - No need to define new test case (s) for SCell activation delay for Rel-18 FR2 HST PC6.

Issue 1-2-4 Whether to define test case for enhancement on inter-frequency measurement with MGs considering DRX in connected mode

- Agreement:

  - No need to define test case (s) for enhancement on inter-frequency measurement with MGs considering DRX in connected mode

Issue 1-2-5 How to design the test case for enhancement on SCell re-selection requirement: Inter-frequency measurement in Idle mode

- Agreement:

  - For the general test parameters in the test case on cell reselection to FR2 inter-frequency NR for Rel-18 FR2 HST, the highSpeedMeasFlagFR2-r17 is set to Set 2.

Sub-topic 1-3 How to design the TC for UL timing adjustment in Rel-18 FR2 HST

Issue 1-3-1 Whether to introduce simultaneous multi-panel operation at UL timing adjustment in Rel-18 FR2 HST

Agreement from Tuesday RRM session:

Issue 1-3-1: The necessity of introducing simultaneous multi-panel operation at UL timing adjustment in Rel-18 FR2 HST

Agreement: No

Issue 1-3-2 Discussion on the new R18 UL timing adjustment TC design for FR2 HST

It is encouraged companies to discuss how to design the MAC-CE indication based TCI state switch delay TC in Rel-18 FR2 HST at CR stage directly.

Sub-topic 1-4 RRM TC configuration

Issue 1-4-1 Channel model and propagation condition

Agreement in Ad-hoc minutes:

Agreement: - RAN4 to adopt bi-directional assumption (i.e., AWGN (serving cell) and AWGN with 19444 Hz frequency offset (neighbour cell)) in each test case discussion.

Issue 1-4-2 Test setup of AoA configuration for all new test cases

Way Forward: Address directly at the CR stage.

Sub-topic 1-5 Test case list agreed for information.

[Moderator] Based on the consensus achieved in the ad-hoc minutes and offline discussion. The test case list for information is updated, below.

Table 2 The RRM performance test cases list (for information)

| TC index | Corresponding Core Requirement | Necessity of New Test Case | Company for CR volunteer |
| --- | --- | --- | --- |
| TC1 | Requirements for Cell Re-selection | New test case needed. (New test case for A.7.1.1.X Cell reselection to FR2 inter-frequency NR case for Rel-18 FR2 HST intra-band CA) | Intel |
| TC2 | Requirements for SCell Activation Delay for Deactivated SCell | No new test case needed. |  |
| TC3-TC6 | Requirements for Inter-frequency measurement with measurement gaps | No new test case needed. |  |
|  |  | No new test case needed. |  |
|  |  | New test case needed. (New test case for A.7.6.2.X3 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA with SSB time index detection when DRX is not used (Pcell in FR2)) | Huawei |
|  |  | New test case needed. (New test case for A.7.6.2.X4 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA without SSB time index detection when DRX is not used (Pcell in FR2)) | Ericsson |
| TC7 | Requirements for Inter-frequency measurement without measurement gaps | No new test case needed. |  |
| TC8 | Requirements for One shot large UL timing adjustment for FR2 Power Class 6 UE | New test case(s) needed. | Nokia |
|  | Requirements for MAC-CE based TCI state switch delay in HST FR2 scenarios |  |  |
|  | New requirements for MRTD with simultaneous two-panel reception for FR2 Power Class 6 UE |  |  |
| TC9 | Requirements for SSB based radio link monitoring | No new test case needed. |  |
| TC10 | Requirements for SSB based beam failure detection | No new test case needed. |  |
| TC11 | Requirements for SSB based L1-RSRP | New test case needed. (New test case for A.7.6.3.X SSB based L1-RSRP measurement when DRX is used for FR2-1 PC6 UEs supporting SimultaneousReceptionFR2HST-r18) | Samsung |
| TC12 | Requirements for Intra-frequency measurements with/without measurement gaps | New test case needed. (New test case for A.7.6.1.X SA event triggered reporting test without gap under non-DRX for UE supporting [measurementEnhancementCAInterFreqFR2-r18]) | Nokia |