3GPP TSG-RAN4 Meeting #110	R4-2403454

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on Improvement for SCell/SCG setup delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The feature is about what measurement results can be reported for UE supporting [existing measurement-based solution]. - There is no new measurement requirement. - UE measurement behavior is not changed. Since Clause 4.7 and 5.8 about reporting requirement, subclause for measurement requirement is not appropriate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete measurement requirement subclause under 4.7 and 5.8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement is not defined properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1, 4.7.2, 4.7.3. 5.8.1, 5.8.2, 5.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of 1st change >>

## 4.7	Measurement report for fast CA/DC setup

### 4.7.1	Introduction

The requirements of measurement report for fast CA/DC setup apply in this clause for UE supporting [solution based on existing measurement] and configured with Rel-18 CA/DC candidate frequencies/cells indicated by higher layers.

A UE supporting [solution based on existing measurement] and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report based on the idle mode measurement which is specified in clause 4.4 with fulfil the requirement specifiy in clause 4.7.3. If UE does not support idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 but UE supports [solution based on existing measurement], UE shall report based on the idle mode measurement which is specified in clause 4.2.2 with fulfill the requirement specified in clause 4.7.3.

### 4.7.2	VOID

### 4.7.3	Measurement Report Requirements

A UE supporting [solution based on existing measurement] capability shall be able to report valid measurement results. The measurement results are considered valid if the following conditions are met:

-	the measurements are performed within the last [X] seconds before msg1 transmission for RRC resume/setup request, where [X] is configured by [TBD], and

-	the measurement results satisfy measurement accuracy requirement at the measurement instance.

Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when X is configured.

If the [X] is not configured, UE is not required to perform validity check and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

## << End of 1st change >>

## << Start of 2nd change >>

## 5.8	Measurement report for fast CA/DC setup

### 5.8.1	Introduction

The requirements of measurement report for fast CA/DC setup apply in this clause for UE supporting [solution based on existing measurement] and configured with Rel-18 CA/DC candidate frequencies/cells indicated by higher layers.

A UE supporting [solution based on existing measurement] and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report based on the inactive mode measurement which is specified in clause 5.4 with fulfil the requirement specifiy in clause 5.8.3. If UE does not support idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 but UE supports [solution based on existing measurement], UE shall report based on the inactive mode measurement which is specified in clause 5.1.2 with fulfill the requirement specified in clause 5.8.3.

### 5.8.2	Measurement Requirements

The requirements in clause 4.7.2 shall apply.

### 5.8.3	Measurement Report Requirements

The requirements in clause 4.7.3 shall apply.

## << End of 2nd change >>