3GPP TSG-RAN4 Meeting #110		R4-2403561

# Athens, Greece, Feb 26 – Mar 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to CSSF for SSB when PRS measurements are configured for RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2/19/2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN4#107 the following agreement was reached (R4-2310072) - Clause 9.1A.5.2 needs to be updated to include the impact of the positioning measurements. Details are FFS. Draft CR R4-2321463 was endorsed in RAN4#109 containing some of the necessary changes. However, the CSSF for SSB in section 5.1A.5.2.1 was not updated. Proposed changes in this CR were taken from corresponding requirements defined for non-RedCap UEs in 38.133, 9.1.5.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add definition of long periodicity measurements and corresponding CSSF definition. - Add conditions for NR positioning frequency layers to be considered as candidate for measurement in a measurement gap instance. - Add procedure to calculate CSFF for SSB measurements when measurement gaps are shared with PRS measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of CSSF for RedCap Ues would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1A.5.2.1, 9.1A.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2401231 |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

##### 9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap_RedCap,i and is derived as described in this clause.

If measurement object i refers to a long-periodicity measurement which is any of:

-	an NR measurement for positioning frequency layer i with Tavailable_PRS,i >160ms, where Tavailable_PRS,i is defined in clauses 9.9A.2.5.1, 9.9A.3.5.1, 9.9A.4.5 and 9.9A.5.5 for RSTD, PRS-RSRP ,UE Rx-Tx time difference and PRS-RSRPP measurements without frequency hopping, respectively.

- 	an NR measurement for positioning frequency layer i with Tavailable_PRS,i >160ms, where Tavailable_PRS,i is defined in clauses 9.9A.2.6.1, 9.9A.3.6.1, 9.9A.4.8 and 9.9A.5.8 for RSTD, PRS-RSRP ,UE Rx-Tx time difference and PRS-RSRPP measurements with frequency hopping, respectively.

then CSSFwithin_gap_RedCap,i =1. Otherwise, the CSSFwithin_gap_RedCap,i for other measurement objects participate in the gap competition and the CSSFwithin_gap_RedCap,i are derived as below.

When multiple positioning frequency layers are configured,

-	for each positioning frequency layer i, CSSFwithin_gap_RedCap,i is derived with the following steps assuming no other positioning frequency layer is configured.

-	for each RRM frequency layer i, CSSFwithin_gap_RedCap,i is derived as follows:

-	an intermediate CSSFwithin_gap_RedCap,i,k is derived with the following steps assuming only positioning frequency layer k is configured, and

-	CSSFwithin_gap_RedCap,i= max(CSSFwithin_gap_RedCap,i,k), where k=0…K-1, and K is the number of configured positioning frequency layers.

For each measurement gap j not used for a long-periodicity measurement defined above, count the total number of intra-frequency measurement objects, inter-frequency/inter-RAT measurement objects and NR positioning frequency layers which are candidates to be measured within the gap j.

-	An NR measurement object with SSB measurement configured is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement object, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An NR positioning frequency layer is counted as candidate for a MG occasion if at least one PRS resource on that positioning frequency layer is fully covered by the MGL excluding RF switching time.

-	For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

-	For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

-	Mintra_RedCap,i,j: Number of intra-frequency measurement objects, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

-	Minter_RedCap,i,j : Number of NR inter-frequency layers and EUTRA inter-RAT, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter_RedCap,i,j  equals 0.

-	Mtot_RedCap,i,j = Mintra_RedCap,i,j + Minter_RedCap,i,j : Total number of intra-frequency, inter-frequency and inter-RAT frequncy layers, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot_RedCap,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra_RedCap,i,j = Minter_RedCap,i,j = Mtot_RedCap,i,j =0.

The carrier specific scaling factor CSSFwithin_gap_RedCap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap_RedCap,i= max(ceil(Ri×Mtot_RedCap,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intra-frequency measurement object, CSSFwithin_gap_RedCap,i is the maximum among

-	ceil(Ri×Kintra×Mintra_RedCap,i,j) in gaps where Minter_RedCap,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra_RedCap,i,j) in gaps where Minter_RedCap,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an inter-frequency or inter-RAT measurement object or NR PRS measurement on any one positioning frequency layer, CSSFwithin_gap_RedCap,i is the maximum among

-	ceil(Ri×Kinter×Minter_RedCap,i,j) in gaps where Mintra_RedCap,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter_RedCap,i,j) in gaps where Mintra_RedCap,i,j=0, where j=0…(160/MGRP)-1

where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

##### 9.1A.5.2.2	SA mode: carrier-specific scaling factor for PRS measurements performed within gaps

The CSSF defined in clause 9.1A.5.2.1 applies to PRS measurements performed by RedCap UEs within measurement gaps with and without FH.

----------------------END OF CHANGE----------------------------