3GPP TSG-RAN WG4 Meeting #110	R4-2403639

# Athens, Greece, February 26 – March 1, 2024								was R4-2402450

Source: 	T-Mobile USA, Dish Network

Title:	TP for TR 38.880: Input for CA_n71A-n77A with 3Tx

Agenda Item:	7.25.2

Document for:	Approval

- Introduction

This contribution provides a text proposal for CA_n71A-n77A 3Tx with 2Tx on n77 for TR 38.880.

This revision recalculates the PC1.5 IMD MSD based on the fact that for the IMD, all the power increase from PC2 to PC1.5 is in n77, and the IMD order for n77 is 1.

# Text Proposal

<< Beginning of Text Proposal >>

## 5.x	CA_n71A-n77A

### 5.x.1	Operating bands for CA

Table 5.x.1-1:  CA band combination of band n71+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n71-n77 | n71, n77 |

### 5.x.2	Maximum output power for inter-band CA

Table 5.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.x.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.x.3	MSD scenario studies

Table 5.5.3-1 and 5.5.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_ n71-n77.

Table 5.5.3-1: MSD scenarios for CA_n71-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic Mixing |  |  | No |
| IMD | n71+n77 | n71 | Yes |
| Cross band leakage | n77 UL | n71 DL | Yes |

Table 5.5.3-2: MSD scenarios for CA_n71-n77 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic Mixing |  |  | No |
| IMD | n71+n77 | n71 | No |
| Cross band leakage | n77 UL | n71 DL | Yes |

### 5.x.4	REFSENS requirements

#### 5.x.4.1	REFSENS requirements for total power class 2

The PC2 IMD MSD can be re-used from 2Tx.

| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 11.4 | FDD | IMD57 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |

For 2 Tx the power is 23 dBm for n71 + 23 dBm for n77 = 26 dBm total.

For 3Tx the power is 23 dBm for n71 + 20 dBm + 20 dBm for n77, 23 dBm on n77 and 26 dBm total.

The PC2 MSD due to cross band isolation can be re-used from PC2 with 2Tx and therefore is not shown here.

#### 5.x.4.2	REFSENS requirements for total power class 1.5

There is no PC1.5 MSD requirement for CA_n71-n77A because PC1.5 requires 2Tx in the TDD band currently.

For 3Tx the power is 23 dBm for n71 + 24.8 dBm + 24.8 dBm for n77 for 27.8 dBm TDD, 29 dBm total.

Therefore, the n77 power is increased by 4.8 for PC1.5 compared to PC2 since the n71 power is unchanged.

And for the IMD combination, it is -4*n71_Tx + 1*n77_Tx = n71_Rx

So, we can expect 4.8dB IMD power increase because the order for n77 is one.

MSD due to interference power I is given by:

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have:

If I is increased by X dB, then MSD(X) is given by

Based on the 4.8 dB dB increase intermod power, the resulting MSD is 16.0.

| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 16.0 | FDD | IMD57 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |

The PC1.5 MSD due to cross band isolation can be re-used from PC1.5 with 2Tx and therefore is not shown here.

<< End of Text Proposal >