3GPP TSG-RAN WG4 Meeting #110	R4-2403661

Athens, Greece, 26 February ‒ 1 March, 2024

Title: 	WF on coverage enhancement for part 1

Agenda Item:	8.19.3

Source: 	Huawei, HiSilicon

Document for:	Approval

Issue 1: Whether to introduce UE capability for dpc-Reporting-FR1

Agreement:

- Introduce UE capability for dpc-Reporting-FR1.

Issue 2: Whether to extend the value range of PowerClassPerBandPerBC in order to cover PC5 for better support high power limit for PC3+PC5 CA/DC

Agreement:

- Inform RAN2 that per-band per BC power class capability should include power class 5 since Rel-18.

  - Refer to the LS in R4-2403659.

Issue 3: UE capability for dpc-Reporting-FR1

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41-1 | Support of ΔPPowerClass reporting mechanism | 1. Support of UE report on the ΔPPowerClass for non-CA and CA operation only when the UE is configured with dpc-Reporting-FR1 and the reporting as defined in TS 38.101-1 and 38.101-3 [Please check the following one instead]: Support of ΔPPowerClass /ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC reporting which is triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance, as specified in TS 38.101-1 and TS 38.101-3 | No | Yes | N/A | UE does not support of report on the ΔPPowerClass | Per UE | No | FR1 only | N/A | Component 1 candidate values: - Type 1: The UE can only report ∆PPowerClass for non-CA operation - Type 2: The UE can report ∆PPowerClass for non-CA operation, and the UE can also report ∆PPowerClass/ ΔPPowerClass,CA/∆PPowerClass,EN-DC/∆PPowerClass,NR-DC for CA operation. | Optional with capability signalling |

QC: We should avoid “power class changing”.

Nokia: I cannot say power class discussion will have conclusion in this meeting. Suggest we stay in what we have in the spec.

QC: We don’t really need to wait for the conclusion in power class thread.

Nokia: We can copy the text from TS 38.321 to here.

QC: We may have power fallback in one band but not another for CA case in the future.

Issue 4: Whether to consider DPC reporting for the UE that only using P-MPR instead of indicating max duty cycle capabilities

[Moderator]: For information, there was an agreement on whether to introduce FR1 P-MPR reporting. (R4-2310484)

- Proposals

  - Option 1: Following DPC reporting with the mapping of P-MPR range can be considered.

  - Option 2: Others.

Recommended Way forward:

  - TBD

Ericsson: Please refer to our latest version in order to address the concern.

OPPO: There was agreement that P-MPR will not be introduced for FR1.

Nokia: We also cannot agree the CR. With this way, this is against the Rel-18 WI agreement. Further, this is conflicting with the existing Rel-18 scheme, since it is only related to max duty cycle excedence.

Ericsson: To OPPO, if this is not enabled for UE that is only applied P-MPR. Basically Rel-18 scheme will not be used. How do you see the feature will be used? This is useful for network scheduling. To Nokia, it is correct that the network will know the UE use P-MPR or not. The signalling is intact if we consider both schemes. We just want this Rel-18 scheme to be useful.

Vivo: We agree with Nokia in general. In the field, it is rare that delta P power class will be reported. Please accept that situation.

QC: We are also OK with this reporting. We don’t see the problem to feed that to the network. P-MPR can be used widely.

Nokia: With current specification, this Rel-18 scheme only related to max duty cycle return/exceedence. P-MPR is another story so this cannot be mixed with it. The network would expect different usage for different information.

QC: We are OK with the improvement.

Ericsson: What is the proper way to Nokia?

Nokia: We need to drastically change the spec. If PHR would allow to report P-MPR for FR1 as FR2, then we can consider.

Ericsson: We actually want to avoid big change to the existing signalling and spec. So we propose this union style way to report delta P power class or P-MPR.

Vivo: This discussion is too far from Rel-18 maintenance phase.

OPPO: Similar view with Vivo and please respect the agreement.

Ericsson: What is the big change that people are so concerned?

China Telecom: We support to optimize the duty cycle solution. What is the different between FR1 and FR2 P-MPR, and why it is so hard to report.