3GPP TSG-RAN WG4 Meeting # 110															R4-2403696

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.853 | CR | 0008 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_RAIL_EU_900MHz-Core) CR to TR 38.853: Corrections on FRMCS n100 deployment aspects and related output power limits, Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RAIL_EU_900MHz-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the regulatory ECC feedback received in R4-2400334, RAN4 was asked to clarify that both uncoordinated, as well as coordinated FRMCS deployments are considered, with related n100 output power decisions from ECC Decision (20)02. Furthermore, R4-2400334 clarified that the additional OBUE limit (derived based on the MSR specification) for n100 is relaxed compared to the single-RAT NR limit, and shall be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of regulatory ECC feedback related to coordinated and uncoordinated FRMCS deployments in band n100. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Relevant regulatory feedback would not be properly captured in related TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 7.1.2.1, 7.1.2.1.2, 9, 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

# 1	Scope

The present document is a technical report deals with the use of the Rail Mobile Radio spectrum in the 900 MHz frequency band in CEPT countries which are subject to the ECC Decision (20)02 [1] for the use by the railways. The purpose is to gather the relevant background information and studies in order to address all the necessary precautions to make the paired spectrum of 874.6-800/919.4-925 MHz usable for 5G NR.

------------------------------ Next modified section -------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ECC Decision (20)02: "Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR) ".

[2]	European Commission implementing Decision (EU) 2021/1730 on the harmonised use of the paired frequency bands 874,4-880,0 MHz and 919,4-925,0 MHz and of the unpaired frequency band 1 900-1 910 MHz for Railway Mobile Radio, 28 September 2021.

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[5]	ECC Report 318, Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band.

[6]	ECC Report 313; Technical study for co-existence between RMR in the 900 MHz range and other applications in adjacent bands; approved 21 May 2020.

[7]	R4-210431; LS to 3GPP RAN WG4 on the interferer signal definition for the RMR900 BS Rx blocking requirement.

[8]	R4-2400334;	LS to 3GPP RAN4 on in-block output power requirements for bands n100 and n101 and on additional unwanted emission limits for band n100, LSin, CEPT ECC WG FM.

------------------------------ Next modified section -------------------------

### 7.1.2	Transmitter characteristics

#### 7.1.2.1	BS maximum output power

Based on ECC Decision (20)02 [1] and clarifications in [8], there were two scenarios considered for FRMCS deployment:

- Uncoordinated FRMCS deployment, with the output power limit specified in ECC Decision (20)02 [1],

- Coordinated FRMCS deployment, where no output power limit was defined for the Wide Area BS class.

For the bullet 1 above, the maximum output power for BS operating in band n100 applicable for uncoordinated deployment, should not exceed the Prated,c,AC derived based on table 4.3.1-1 EIRP limits, with the assumption of maximum gain value of the BS antenna gain GmaxRMR900, and assuming one antenna connector, as:

(64.5 – GmaxRMR900) dBm / 5 MHz + (fDL-922.1) x 40/3 dB = 51.5 dBm/5MHz + (fDL - 922.1) × 40/3 dB,

where GmaxRMR900 is the maximum gain (antenna gain and losses) for the RMR900 BS consideration.

NOTE: 	Based on inputs received in [8], it was further clarified by the CEPT ECC WG FM that both uncoordinated, as well as coordinated deployments shall be considered by RAN4 specification for FRMCS deployment of band n100. Therefore, output power limits higher that the above listed EIRP are also possible for FRMCS in band n100, subject to coordination.

------------------------------ Next modified section -------------------------

##### 7.1.2.1.2	OBUE

Emission mask of the OBUE Cat B Option 2 limits in 3GPP TS 38.104 [4] applicable to Europe was compared to the one derived from ECC Decision (20)02 [1], in order to verify the need for additional RMR-specific OBUE requirement introduction.

The following was observed:

-	Both masks were derived with different measurement bandwidths;

-	Emission limits comparison (after measurement bandwidths scaling) revealed that the existing OBUE Cat B Option 2 mask is more stringent only for part of the f offset range (f < 0.2 MHz).

Based on the above motivation the following OBUE requirement are necessary for band n100 in 3GPP TS 38.104 [4], based on conversion of the EIRP limits from ECC Decision (20)02 [1], as summarized in table 7.1.2.1.2-1. The requirement is derived based on table 4.3.1-4, with the assumption of a maximum gain value of the BS GmaxRMR900:

Table 7.1.2.1.2-1: OBUE requirement derivation for n100

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (NOTE) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.1 MHz f_offset < 0.3 MHz | 19.5 dBm | 200 kHz |
| 0.2 MHz f < 1 MHz | 0.6MHz f_offset < 1.4 MHz | 1 dBm | 800 kHz |
| 1 MHz f 10 MHz | 1.5 MHz f_offset < 10.5 | -8 dBm | 1 MHz |
| NOTE: Assuming 13 dBi maximum gain (antenna gain and losses) for GmaxRMR900, and assuming one antenna connector. |  |  |  |

Based on inputs received in [8], it was clarified by the CEPT ECC WG FM that the above limit was derived based on MSR specification and can be removed from NR BS specification. Therefore, the additional OBUE limit for n100 was removed from Rel-17 specifications TS 38.104 clause 6.6.4.2.5.7, and TS 38.141-1 clause 6.6.4.5.6.7.

------------------------------ Next modified section -------------------------

# 9	Deployment aspects

In general, network deployment aspects are out of scope of the RAN4 work. For the derivation of the BS RF requirements it was assumed that the RMR and MFCN base stations are not co-located. It was identified afterwards that such scenarios may actually happen in the field.

Co-location of a MFCN BS and a RMR BS requires coordination among the involved parties.

In accordance with ECC Decision (20)02 Decides 4 [1], in case of implementation of coordination procedures or other mitigation measures for band n100 deployments it is possible to deploy higher E.I.R.P. than stated in the technical conditions in ECC Decision (20)02 Annex 2[1]. This has been further confirmed by the CEPT ECC WG FM feedback in [7], where RAN4 was asked to consider both coordinated, as well as uncoordinated deployments.

For uncoordinated deployment the technical conditions stated in ECC Decision (20)02 Annex 2 [1] apply for the in-block requirements, i.e. output power limit applies for band n100 operation

The use of spectrum and its related conditions, e.g., E.I.R.P, in accordance to ECC Decision (20)02 Part B [1], are in the responsibility of national regulation and coordination among involved parties.

------------------------------ Next modified section -------------------------

# 10	Conclusion

The study dealt with how the RMR900MHz spectrum (874.4 - 880 MHz/ 919.4 - 925 MHz) can be used for rail communication using 5G NR applicable in Europe. The corresponding system parameters, BS RF parameter and UE RF parameter were examined:

-	General:	The focus was on the use of RMR900 under uncoordinated deployment conditions, assuming that RMR BS operates independently of other operators e.g. MFCN, however based on CEPT ECC WG FM clarification received in [7], also coordinated deployments have to be considered. Co-location with other operators and resulting coordination is subject to national regulation and is outside the scope of this work item.

-	System Parameters: The RMR900 spectrum block can be used by 5MHz CBW reusing Rel-15 sync raster design. For channel bandwidths smaller than 5 MHz, related study objectives were covered in a separate Rel-18 work item [RP-220401].

-	BS RF: Based on the ECC Decision (20)02 [1], the EIRP limits were analysed accordingly and the necessary conclusions for an uncoordinated operational approach were derived to define conducted power requirements. . However, based on CEPT ECC WG FM clarification received in [7], also coordinated deployments have to be considered. These include the BS rated output power using corresponding assumptions, the resulting emission values of the transmitter and the specifications of the receiver. Furthermore, the coexistence with other 3GPP radio technologies has been included.

-	UE RF:	Based on the ECC Decision (20)02 [1], the EIRP limits were analysed accordingly, and the necessary conclusions were derived for the use of Power Class 3 (23dBm).

------------------------------ End of modified section -------------------------