3GPP TSG-RAN WG4 Meeting # 110															R4-2403702

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.852 | CR | 0009 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_RAIL_EU_1900MHz_TDD-Core) CR to TR 38.852: complementary update for the n101 cab-radio aspects, Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RAIL_EU_1900MHz_TDD-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During Rel-17 band n101 was introduced based on ECC Decision (20)02 for CEPT countries. Based on the same regulatory ECC Decision (20)02, HPUE requirements were introduced for the cab-radio in Rel-18. In this CR we introduce cross-reference to the Rel-18 work done for n101 cab-radio, for completeness. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 2: Adding reference to the Rel-18 HPUE TR. - 4.3.1: adding descriptive text for RF requirements derivation approach. - 4.3.2, 7.2.1, 10: adding information on the HPUE cab-radio requirements. - Removal of outstanding []. - Other editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Link with the Rel-18 HPUE cab-radio work for n101 would be missing. Multiple TR inconsistencies would remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 4.1, 4.2, 4.3, 5, 6, 7.1, 7.2, 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

# 1	Scope

The present document is a technical report deals with the use of the Rail Mobile Radio spectrum in the 1900MHz frequency band, which was assigned by the ECC Decision (20)02 [1] for the use by the railways in Europe.The purpose is to gather the relevant background information and studies in order to address all the necessary precautions to make the unpaired spectrum of 1900-1910MHz usable for 5G NR.

------------------------------ Next modified section -------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ECC Decision (20)02: "Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR) ".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	RP-211496	Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR), WID

[4]	3GPP TS 38.104: " NR; Base Station (BS) radio transmission and reception".

[5]	European Commission implementing Decision (EU) 2021/1730 on the harmonised use of the paired frequency bands 874,4-880,0 MHz and 919,4-925,0 MHz and of the unpaired frequency band 1 900-1 910 MHz for Railway Mobile Radio, 28 September 2021.

[6]	ECC Report 318, Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band.

[7]	3GPP TR 37.829: " High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands for Rel-18".

------------------------------ Next modified section -------------------------

# 4	Regulatory background

## 4.1	EC Decision 2021/1703

On September 28th 2021, the European Commission Decision 2021/1703 [5] establishes the harmonised conditions for the availability and efficient use of radio spectrum for the Railway Mobile Radio (RMR) in the band 1900 – 1910 MHz. By 1 January 2022, Member States shall designate and make available on a non-exclusive basis the unpaired frequency band 1900 – 1910 MHz for Railway Mobile Radio, in accordance with the technical conditions listed in clause 4.3.

## 4.2	ECC Decision(20)02

The ECC Decision (20)02 [1] addresses the designation of the unpaired frequency band 1900-1910 MHz to be used for Railway Mobile Radio (RMR) on a CEPT wide basis.

By this Decision, the CEPT administrations shall designate the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR) on a non-exclusive basis, according to the technical conditions listed in clause 4.3.

This Decision entered into force on November 20th, 2020.

## 4.3	Technical conditions

### 4.3.1	Base stations using wideband technologies

Only non-AAS Base Stations were considered, based on study outcomes as captured in ECC Decision (20)02 [1]. Baseline for the BS RF requirements derivation for the RAN4 n101 RF specific requirements were extracted from ECC Decision (20)02 [1] in tables 4.3.1-1 to 4.3.1-3.

Table 4.3.1-1: General in-block requirement mandatory for uncoordinated deployment [1]

| RMR channel BW | Maximum EIRP |
| --- | --- |
| 10 MHz | 65 dBm/10 MHz (NOTE) |
| NOTE: In case an administration wishes to allow higher e.i.r.p., coordination or other mitigation measures are required. |  |

Table 4.3.1-2: Baseline requirement [1]

| Frequency Range | EIRP limit |
| --- | --- |
| 1920 - 1980 MHz | -43 dBm/5 MHz |

Table 4.3.1-3: Requirements on wideband RMR BS receiver characteristics [1]

| Parameter | Value |
| --- | --- |
| Level of the wanted signal | RefSens + 3 dB |
| Maximum 5 MHz LTE interfering signal in 1805 - 1880 MHz | -20 dBm |
| NOTE 1: The antenna connector of the BS receiver is the reference point. The reference sensitivity (RefSens) is the minimum mean power received at the antenna connector at which a specified minimum performance shall be met. NOTE 2: These requirements cover both blocking and third-order intermodulation. |  |

### 4.3.2	Terminals other than cab-radios using wideband technologies

ECC Decision (20)02 [1] has also captures requirements for cab-radio, which were not addressed in Rel-17 WI. Those requirements were captured in TR 37.829 [7].

Table 4.3.2-1: Terminals and cab-radio requirements using wideband technologies

| Maximum output power | 23 dBm |
| --- | --- |
| ACLR | 30 dB minimum |
| Other | Uplink power control is mandatory and shall be activated. |

# 5	Frequency band arrangement

The RMR 1900 MHz band is within the range of FR1 and is proposed as TDD band (Table 5-1).

Table 5-1: New NR RMR band in FR1

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n101 | 1900 MHz – 1910 MHz | 1900 MHz – 1910 MHz | TDD |

# 6	NR system parameters

The following system parameters are defined for RMR 1900 MHz band:

Table 6-1: RMR 1900 - Channel bandwidth

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |
| n101 | 30 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6-2: RMR 1900 - applicable NR-ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n101 | 100 | 380000 – <20> – 382000 | 380000 – <20> – 382000 |

Table 6-3: RMR 1900 – applicable SS raster entries

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n101 | 15 kHz | Case A | 4754 – <1> – 4768 |
|  | 30 kHz | Case C | 4760 – <1> – 4764 |

# 7	RF requirements

## 7.1	BS specific requirements

### 7.1.1	General

According to ECC Decision (20)02 [1], the BS RF work for RMR1900 is limited to the Wide Area Base Stations, determined by the use along railway lines and their corresponding characteristics defined by 3GPP TS 38.104 [4].

In accordance to ECC Decision (20)02 [1], the related studies were done for the non-AAS BS architectures, defined in 3GPP TS 38.104 [4]. Therefore, the RAN4 requirements derivation was limited to the BS type 1-C requirements, only.

As the EIRP limits were defined in the ECC Decision (20)02 [1], consideration of non-AAS BS architecture required to convert those EIRP limits into the conducted requirements assuming a maximum antenna gain that can vary depending on the used antenna and related antenna gain as well as the potential losses, e.g., feeder. In the present consideration an antenna gain of 18 dBi and 4 dB losses in accordance to ECC Report 318 [6] resulting to a maximum gain value of GmaxRMR1900 = 14 dBi. Other assumptions may lead to other maximum gain values and conducted requirements. Also, one antenna connector was assumed. If the BS has more than one antenna connector, the limits shall be re-calculated considering the intended number of antenna connectors serving a cell.

### 7.1.2	Transmitter characteristics

#### 7.1.2.1	BS maximum output power

Based on ECC Decision (20)02 [1], the BS maximum output power for BS operating in band n101 applicable for uncoordinated deployment, should not exceed the Prated,c,AC derived based on table 4.3.1-1 EIRP limits, with the assumption of a maximum value of the BS GmaxRMR1900 , and assuming one antenna connector,as:

(65 – GmaxRMR1900) dBm / 10 MHz = 51 dBm / 10 MHz,

(62 – GantRMR1900) dBm / 5 MHz = 48 dBm / 5 MHz,

where GmaxRMR1900 is the maximum gain (antenna gain and losses) of the RMR 1900 BS consideration.

Note:	5 MHz channel bandwidth support was not considered in ECC Decision (20)02 [1] and was added on request of RMR operators. The above limit for 5 MHz channel bandwidth was then specified based on the limit for 10 MHz, keeping constant PSD.

#### 7.1.2.2	Unwanted emissions

##### 7.1.2.2.1	General

Only Category B emission requirements are applicable to NR operation in RMR1900 band.

##### 7.1.2.2.2	Tx spurious emissions

Additional Tx spurious emission requirements are considered for band n101 in 3GPP TS 38.104 [4], based on conversion of the EIRP limits from ECC Decision (20)02 [1], as summarized in table 7.1.2.2.2-1. The requirement is derived based on table 4.3.1-5, with the assumption of maximum gain value of the BS GmaxRMR1900:

Table 7.1.2.2.2-1: Additional Tx spurious emissions requirement derivation for n101

| Spurious frequency range |  | Basic limit |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 1920 MHz – 1980 MHz |  | - 43 - GmaxRMR1900 = -57 dBm (NOTE) |  | 5 MHz |
| NOTE: Assuming a 14 dBi maximum gain (antenna gain and losses) for GmaxRMR1900, and assuming one antenna connector. |  |  |

### 7.1.3	Receiver characteristics

RAN4 will not specify requirements related to the CEPT assumptions for BS enhanced selectivity operating adjacent to band n1 and deployed close by the corresponding railway infrastructure.

#### 7.1.3.2	Rx blocking

Rx blocking requirements applicable for band n101 in 3GPP TS 38.104 [4], based on conversion of the EIRP limits from ECC Decision (20)02 [1], are summarized in table 7.1.3.2-1. Interfering signal characteristic is 5 MHz LTE signal, as specified in ECC Decision (20)02 [1].

Table 7.1.3.2-1: Rx blocking requirement derivation for n101

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Center Frequency of Interfering Signal (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 10 | PREFSENS + 3 dB | Wide Area BS: -20 | 1807.5 MHz to 1877.5 MHz | 5 MHz LTE signal |

## 7.2	UE specific requirements

### 7.2.1	Transmitter characteristics

Rel-17 UE requirements for RMR 1900 are summarized in the upcoming tables applicable for 3GPP TS 38.101-1 [2].

Table 7.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n101 |  |  |  |  |  |  | 23 | ±2 |

NOTE:	PC1 for cab-radio was introduced in Rel-18. Refer to TR 37.829 [7].

Table 7.2.1-2: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n101 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 41, 42, 43, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |

### 7.2.2	Receiver characteristics

The following 3GPP TS 38.101-1 [2] receiver characteristics changes were introduced for 1900 MHz RMR band:

Table 7.2.2-1: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24) |  |

Table 7.2.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n101 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

------------------------------ Next modified section -------------------------

# 10	Conclusion

The study dealt with how the 1900-1910 MHz spectrum block (RMR1900) can be used for rail communication using 5G NR applicable in CEPT countries. The subject-areas 5G NR system, BS RF and UE RF were examined in detail.

General:	The focus was on the use of RMR1900 under uncoordinated conditions, assuming that RMR BS operates independently of other operators e.g. MFCN. Co-location with other operators and resulting coordination is subject to national regulation and will be not further detailed by 3GPP RAN4.

System Parameter:	The RMR1900 spectrum block can be used by either a 10 MHz CBW or a 5 MHz CBW. The corresponding system parameters are to be transferred to 3GPP TS 38.101-1 [2] and 3GPP TS 38.104 [4].

BS RF:	The ECC Decision (20)02 [1], i.e. EIRP limits were analysed accordingly and the necessary conclusions for an uncoordinated operational approach were derived. These include the BS rated output power using corresponding assumptions, the resulting emission values of the transmitter and the specifications of the receiver. The agreed parameters are to be transferred to 3GPP TS 38.104 [4]. Furthermore, the coexistence with other 3GPP radio technologies has been included. These are to be captured in the corresponding normative 3GPP technical specifications.

UE RF:	The ECC Decision (20)02 [1], i.e. EIRP limits were analysed accordingly, and the necessary conclusions were derived for the use of Power Class 3 (23dBm).

NOTE:	PC1 power class was introduced for the cab-radio after completion of Rel-17, with their derivation captured in in TR 37.829 [8].

------------------------------ End of modified section -------------------------