3GPP TSG-RAN WG4 Meeting #110	R4-2403833

# Athens, Greece,  February 26th - March 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1988 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding support for the VLP mode in US |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Charter Communications Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic_enh-Core |  |  |  |  |  | Date: |  |  | 2024-03-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | On September 2023, FCC decided to permit very low-power (VLP) operation in the following sub-bands: U-NII-5 and U-NII-7. And the corresponding changes in the US regulatory domain have been already captured during the 3GPP RAN#102 meeting by updating TR 37.890. To support the VLP mode in US a new NS flag is needed becase the US regulations limit the VLP mode to U-NII-5 and U-NII-7 sub-bands within the 6GHz unlicensed band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | US VLP is introduced with a new flag NS_71. NS_71 is added to Table 6.2F.1-2 capturing the additional requirements for transmit power density and the frequency ranges. NS_71 is added to Table 6.2F.3.1-1. A-MPR values from clause 6.2F.3.16 are added for NS_71. Emission limits specified in clause 6.5F.3.3.5 are added for NS_71. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VLP mode in the unlicensed 6GHz band will not be supported in US. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2F.1, 6.2F.3, 6.2F.3.1, 6.2F.3.16, 6.5F.3.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 38.849 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Updating NS flag from NS_70 to NS_71 as the value ‘70’ is already used for UAV flag. |  |  |  |  |  |  |  |  |

<< Start of changed section >>

### 6.2F.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of shared spectrum channel access carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2F.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  | 23 | +2/-3 | 20 | +2/-3 |
| n96 |  |  |  |  | 23 | +2/-3 | 20 | +2/-3 |
| n102 |  |  |  |  | 23 | +2/-3 | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

The UE operating shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2F.1-2 when NS is signaled and when transmission overlaps with any portion of the specified frequency range.  In case transmission overlaps multiple frequency ranges, the lowest power density requirement applies.

Table 6.2F.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n46 | NS_28 | 20, 40, 60, 80 | 5150 – 5350 | 10 |
|  |  |  | 5470 – 5725 |  |
|  | NS_29 | 20 | 5170 – 5330 | 10 |
|  |  |  | 5490 – 5730 |  |
|  |  | 40 | 5170 – 5330 | 7 |
|  |  |  | 5490 – 5730 |  |
|  |  | 60, 80 | 5170 – 5330 | 4 |
|  |  |  | 5490 – 5730 |  |
|  | NS_30 | 20, 40, 60, 80 | 5150 – 5350 | 11 |
|  |  |  | 5470 – 5725 |  |
|  | NS_31 | 20 | 5150 - 5230 | 10 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 40 | 5150 - 5230 | 7 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 60, 80 | 5150 - 5230 | 4 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 |  |
| n96 | NS_53 | 20, 40, 60, 80, 100 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |
|  | NS_59 | 20, 40, 60, 80, 100 | 5925 – 7125 | 5 |
|  | NS_60 | 20, 40, 60, 80, 100 | 5925 – 7125 | 2 |
|  | NS_61 | 20, 40, 60, 80 | 5925 - 6425 | 1 |
|  | NS_66 | 20, 40, 60, 80, 100 | 5925 - 7125 | -8 |
|  | NS_67 | 20, 40, 60, 80, 100 | 5925 - 7125 | -5 |
|  | NS_71 | 20, 40, 60, 80, 100 | 5925 – 6425MHz 6525 – 6875MHz | -5 |
| n102 | NS_58 | 20, 40, 60, 80, 100 | 5945 – 6425 | 10 |
|  | NS_64 | 20, 40, 60, 80, 100 | 5945 – 6425 | 1 |
|  | NS_65 | 20, 40, 60, 80, 100 | 5945 – 6425 | 11 |
|  | NS_68 | 20, 40, 60, 80, 100 | 5945 – 6425 | 1 |

<< Unchanged parts are skipped >>

### 6.2F.3	UE additional maximum output power reduction

#### 6.2F.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2F.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2F.2.

Table 6.2F.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2F.3.1-1A and table 6.2F.3.1-1B.

Table 6.2F.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (clause) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | n46, n96 | 20, 40, 60, 80 |  | N/A |
| NS_28 | 6.5F.3.3.1 | n46 | 20, 40, 60, 80 |  | 6.2F.3.2 |
| NS_29 | 6.5F.3.3.2 | n46 | 20, 40, 60, 80 |  | 6.2F.3.3 |
| NS_30 | 6.5F.3.3.3 | n46 | 20, 40, 60, 80 |  | 6.2F.3.4 |
| NS_31 | 6.5F.3.3.4 | n46 | 20, 40, 60, 80 |  | 6.2F.3.5 |
| NS_53 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.6 |
| NS_54 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80 |  | 6.2F.3.7 |
| NS_58 | 6.5F.3.3.6 | n102 | 20, 40, 60, 80 |  | 6.2F.3.8 |
| NS_59 | 6.2F.1 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.9 |
| NS_60 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.10 |
| NS_61 | 6.5F.3.3.7 | n96 | 20, 40, 60, 80 |  | 6.2F.3.11 |
| NS_63 | 6.5F.3.3.8 | n102 | 20, 40, 80 |  | 6.2F.3.12 |
| NS_64 | 6.5F.3.3.9 | n102 | 20, 40, 60, 80, 100 |  | 6.2F.3.13 |
| NS_65 | 6.2F.1 | n102 | 20, 40, 60, 80, 100 |  | 6.2F.3.14 |
| NS_66 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.15 |
| NS_67 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.16 |
| NS_68 | 6.2F.1 | n102 | 20, 40, 60, 80, 100 |  | 6.2F.3.17 |
| NS_69 | 6.5F.3.3.8 | n102 | 20, 40, 80 |  | 6.2F.3.18 |
| NS_71 | 6.5F.3.3.5, 6.2F.1 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.19 |
| NOTE 1: The A-MPR shall apply to all active 20 MHz sub-bands contiguously allocated in the channel. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2F.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_28 | NS_29 | NS_30 | NS_31 |  |  | Reserved |
| n96 | NS_01 | NS_53 | NS_54 | NS_59 | NS_60 | NS_61 |  | Reserved |
| n102 | NS_01 | NS_58 | NS_63 |  |  |  |  | Reserved |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

Table 6.2F.3.1-1B: Mapping of extended network signaling label

| NR band | Value of extendedAdditionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| n96 | NS_66 | NS_67 | NS_71 |  |  |  |  |  |
| n102 | NS_64 | NS_65 | NS_68 | NS_69 |  |  |  |  |
| NOTE: extendedAdditionalSpectrumEmission corresponds to an information element of the name [EXTENDED_ additionalSpectrumEmission] defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

<< Unchanged parts are skipped >>

#### 6.2F.3.16	A-MPR for “NS_67” or “NS_71”

When "NS_67" or “NS_71” is indicated in the cell, the A-MPR is specified in Table 6.2F.3.16-1.

Table 6.2F.3.16-1: A-MPR for “NS_67” and “NS_71” power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | QPSK | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 16 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 64 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 256 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
| CP-OFDM | QPSK | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 16 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 64 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 256 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

<< Unchanged parts are skipped >>

##### 6.5F.3.3.5	Requirements for network signalling value "NS_53" or "NS_54" or "NS_60" or “NS_66” or “NS_67” or “NS_71”

When “NS_53” or “NS_54” or “NS_60” or “NS_66” or “NS_67” or “NS_71” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5F.3.3.5-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.5-1: Additional requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

<< End of changed section >>