3GPP TSG-RAN WG4 Meeting # 110											R4-2403891

Athens, Greece, Feb. 26th – Mar. 1st , 2024

Title: 	WF on NR UE power classes

Agenda Item:	12.2.1

Source: 	Huawei, HiSilicon

Document for:	Approval

# Background

It has been observed that:

Observation 1: RAN2 adopts the principle of capability inheritance. A fallback band combination shall not be reported if it has the same or lower capabilities as a parent band combination.

Observation 2: The UE may report fallback BCs having additional functionality, either by adding entries in FeatureSetCombination of the same BandCombination IE, or by separate BandCombination entries.

Observation 3: RAN2 spec states that:

The Network configures serving cell(s) and BWP(s) configuration to comply with capabilities derived from the combination of FeatureSets at the same position in the FeatureSetsPerBand, regardless of activated/deactivated serving cell(s) and BWP(s).

# Way Forward

## <General Aspects>

For any DL CA with single-carrier UL, shall the UE mandatorily support the power class indicated in ue-PowerClass for the UL band if it’s applicable in the spec for the CA configuration?

down-select to the following two options

- Option 2: No. It is optional, subject to the power class capability reported by the UE.

- Option 3: Yes, except for Power Class 1.5 for which the UE shall at least meet the minimum requirements for Power Class 2.

If a BC is not explicitly reported, how to determine the power class for the BC as well as the power class(es) for the UL component band(s)?

Agreement:

- Follow RAN2’s principle of capability inheritance, and derive the power class capabilities from a parent BC;

The fallback BC is NOT reported due to RAN2 fallback rule:

- Provided that this BC is still supported by the UE based on the reported parent BC, the max Tx power PCMAX,f,c for the UL component band is determined by:

  - the power class derived from a parent BC

## <Online agreements>

Agreement:

- For UE that is configured in the single carrier mode (1 DL + 1 UL on this band), the power class is determined by ue-PowerClass for this NR band.

Agreement:

- The RAN4 common understanding is the ue-PowerClassPerBandPerBC-r17 capability can be used for 3Tx band combinations such as UL CA+TxD and UL CA+UL MIMO

# Reference

[1] R4-2401102 Topic summary for [110][143] NR_power_class	Moderator (Samsung)	RAN4#110