3GPP TSG-RAN WG4 Meeting #110-bis	R4-2404126

Changsha, China, April 15-19, 2024

Source:	Nokia

Title:	Discussion on PDSCH Demodulation Requirements for 8Rx

Agenda item:	6.1.3.1.1

Document for:	Discussion

Introduction

This document extends the discussion on the 8Rx UE demodulation and CSI requirements introduced in RAN4 #105 summarized in [1], discussions were continued through to RAN4 #109 where the latest way forward was summarized in [7][4].

The agreements reached during the previous meetings regarding 8 RX and CSI requirements are captured on the WFs [2], [3], [4], [5], [6] and [7].

The major open topics being:

- PDSCH requirements

This paper presents Nokia’s views on the open issues related to the 8Rx UE PDSCH demodulation.

Discussion

Reference Channels for CA Cases

As part of RAN4#110 the draftCR R4-2403083 was submitted to bring together the PDSCH CA requirements for insertion into TS 38.101-4.

During the development of this draftCR it was identified that there were some errors in the reference channels.

Some endorsed CRs on the 8Rx Work Item have some issues associated with them.

The CRs endorsed at the last meeting have numerous issues, e.g., missing CBWs and irregular numbering, which should be fixed in this meeting.

The remainder of the discussion within this contribution is broken down per ‘table’ within TS 38.101-4, as the tables for baseline and simple receiver are the same (except for the SNR) they have only been used once in this contribution.

FDD 15kHz SCS, Rank 2

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | [R.PDSCH.1-20.1 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 10 | R.PDSCH.1-2.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 15 | [R.PDSCH.1-20.2 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 20 | [R.PDSCH.1-20.3 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 25 | [R.PDSCH.1-20.4 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 30 | [R.PDSCH.1-X.1 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 35 | [R.PDSCH.1-X.2 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 40 | [R.PDSCH.1-X.3 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 45 | [R.PDSCH.1-X.4 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 50 | [R.PDSCH.1-X.5 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |

From CRs:

R4-2319230 (CBW 5MHz – 25MHz)

Issues:

- Same numbering as R4-2318668

R4-2319388 (CBW 30MHz – 50MHz)

Issues:

- None

FRC numbering is identical between R4-2318668 and R4-2319230.

Both R4-2318668 and R4-2319230 could be revised to include FRC numbers with ‘.X’ to be agreed at RAN4#110-bis.

FDD 15kHz SCS, Rank 8

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | [R.PDSCH.1-20.1 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 10 | R.PDSCH.1-3.7 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 15 | [R.PDSCH.1-20.2 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 20 | [R.PDSCH.1-20.3 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 25 | [R.PDSCH.1-20.4 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 30 | [R.PDSCH.1-23.5 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 35 | [R.PDSCH.1-23.3 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 40 | [R.PDSCH.1-23.1 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 45 | [R.PDSCH.1-23.4 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 50 | [R.PDSCH.1-23.2 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |

From CRs:

R4-2318668 (CBW 5MHz – 25MHz)

Issues:

- Same numbering as R4-2319230

R4-2319228

Issues:

- Irregular numbering

R4-2319228 numbering is irregular.

R4-2319228 may be revised to have a more regular numbering before being submitted to MCC.

TDD 30kHz SCS, Rank 2, Baseline Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | [R.PDSCH.2-XX.1 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 10 | [R.PDSCH.2-XX.2 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 15 | [TBD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 20 | [R.PDSCH.2-XX.4 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 25 | [R.PDSCH.2-XX.5 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 30 | [R.PDSCH.2-XX.1 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 50 | [TBD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 60 | [R.PDSCH.2-XX.2 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 80 | [R.PDSCH.2-XX.3 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 90 | [R.PDSCH.2-XX.4 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |
| 100 | [R.PDSCH.2-XX.5 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 |  |

From CRs:

R4-2402968 (CBW 5MHz – 25MHz)

Issues:

- Missing 15 MHz case

R4-2402969 (CBW 40MHz – 100MHz)

Issues:

- Missing 50 MHz case

R4-2402968 is missing the 15 MHz case

R4-2402968 may be revised and include the 15 MHz case

R4-2402969 is missing the 50 MHz case

R4-2402969 may be revised and include the 50 MHz case

TDD 30kHz SCS, Rank 8

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | [R.PDSCH.2-31.1 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 10 | [R.PDSCH.2-31.2 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 15 | [R.PDSCH.2-31.3 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 20 | [R.PDSCH.2-31.4 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 25 | [R.PDSCH.2-31.5 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 30 | [R.PDSCH.2-32.1 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 40 | [R.PDSCH.2 3.7 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 50 | [R.PDSCH.2-XX.1 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 60 | [R.PDSCH.2-XX.2 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 80 | [R.PDSCH.2-XX.3 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 90 | [R.PDSCH.2-XX.4 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |
| 100 | [R.PDSCH.2-XX.5 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 |  |

From CRs:

R4-2319229 (CBW 5MHz – 25MHz)

Issues:

- None

R4-2319331 (CBW 40MHz – 100MHz)

Issues:

- None

Conclusion

In this paper, we have assessed the current direction of the 8Rx WI, specifically PDSCH UE Demodulation performance and the FRCs used for the CA performance requirements. We have provided recommendations where agreements need to be made.

Specifically, in the paper, the following Observations and Proposals were made:

Observation 1: Some endorsed CRs on the 8Rx Work Item have some issues associated with them.

Proposal 1: The CRs endorsed at the last meeting have numerous issues, e.g., missing CBWs and irregular numbering, which should be fixed in this meeting.

Observation 2: FRC numbering is identical between R4-2318668 and R4-2319230.

Observation 3: Both R4-2318668 and R4-2319230 could be revised to include FRC numbers with ‘.X’ to be agreed at RAN4#110-bis.

Observation 4: R4-2319228 numbering is irregular.

Observation 5: R4-2319228 may be revised to have a more regular numbering before being submitted to MCC.

Observation 6: R4-2402968 is missing the 15 MHz case

Observation 7: R4-2402968 may be revised and include the 15 MHz case

Observation 8: R4-2402969 is missing the 50 MHz case

Observation 9: R4-2402969 may be revised and include the 50 MHz case

- References

- RP-221496 “Revised WID Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”

- R4-2220152, “Topic summary for [105][326] RF_FR1_enh2_Demod”, Huawei, HiSilicon, RAN WG4 #105, Toulouse, Nov. 2022

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106-bis-e, Online, April 17 – April 26, 2023

- R4-2309806, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #107, Incheon, Korea, 22 – 26 May 2023

- R4-2313877, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #108, Toulouse

- R4-2316914, “WF for 8Rx and 4Tx UE performance requirements”, RAN4#108-bis, Xiamen, October 2023

- R4-2321199, “WF for 8Rx UE performance requirements”, RAN4#109, Chicago, November 2023

- TS 38.101-4, “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements’ v18.1.0, October 2023