3GPP TSG-RAN WG4 Meeting #110bis	R4-2404148

Changsha, China, 15th – 19th April, 2024

Agenda item:	6.8.7.1

Source:	MediaTek Inc.

Title:	Simulation results on UE demodulation requirements for less than 5MHz

Document for:	Information

- Introduction

In RAN4#110 meeting, it was agreed to define requirements on punctured PBCH and PDCCH for less than 5MHz. In this contribution, we provide our simulation results based on the agreed assumptions [1].

- Simulation Results

      - 2.1 Simulation results for punctured PBCH

Based on agreements in the last meeting, we provide simulation results for PBCH in Figure 1 and 2.

Issue 1-4-1: PBCH requirement in non-HST conditions Agreement: Use the following parameters for PBCH requirement in normal conditions. Interested companies are encouraged to bring simulation results. Duplex; Bandwidth (MHz) / Subcarrier spacing (kHz); SSB/PBCH index; Propagation condition; Antenna configuration and correlation matrix; Reference value. Pm-bch (%); SNR (dB). FDD; 3 / 15; Unknown; TDLC300-100; 1 x 2 Low,; 1; TBD. 1x4 Low; 1; TBD.

| Figure 1. PBCH performance for 1T2R | Figure 2. PBCH performance for 1T4R |
| --- | --- |

The SNR values achieving 1% Pm-bch are summarized in Table 1.

Table 1. SNR values achieving 1% Pm-bch

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | Alignment SNR (dB) | Impairment SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1x2 Low | 1 | -5.4 | -2.9 |
|  |  |  |  | 1x4 Low | 1 | -7.5 | -5.0 |

      - 2.2 Simulation results for punctured PDCCH

RAN4 agreed to define PDCCH requirements for 3MHz with 15 PRBs. For this case, partial CCE will be punctured. Simulation curves for BLER versus SNR are shown in Figure 3~6.

Issue 1-3-2: Requirements for punctured PDCCH (non-HST conditions) Agreement: - Define punctured PDCCH demodulation requirements with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW with the following parameters: - 15PRBs, 3 symbols, non-interleaved, AL4, DCI 1_0 (35 bits for 15 PRBs), 2Rx/4Rx; - Use CCEs #4, #5, #6, and #7 to transmit PDCCH with DCI 1_0 Way forward: - Consider the following requirements’ parameters: - Reuse Table 5.3-1: Common test Parameters and Table 5.3.2.1-1: Test Parameters for FDD for 2RX and Table 5.3.3.1-1: Test Parameters for 4RX for the FR1 less than 5MHz PDCCH requirements. Number of Tx; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 3; 15; 3; 4; [R.PDCCH. 1-2.x FDD]; TDLA30-10; 1x2 Low; 1; [TBA]. 2; 3; 15; 3; 4; [R.PDCCH. 1-2.x FDD]; TDLC300-100; 2x2 Low; 1; [TBA]. 1; 3; 15; 3; 4; [R.PDCCH. 1-2.x FDD]; TDLA30-10; 1x4 Low; 1; [TBA]. 1; 3; 15; 3; 4; [R.PDCCH. 1-2.x FDD]; TDLC300-100; 2x4 Low; 1; [TBA]. - Further discuss whether to define requirements with 1TX and/or 2TX.

| Figure 3. punctured PDCCH performance for 1T2R | Figure 4. punctured PDCCH performance for 1T4R |
| --- | --- |
| Figure 5. punctured PDCCH performance for 2T2R | Figure 6. punctured PDCCH performance for 2T4R |

In Table 2, we summarize the SNR values achieving 1% BLER.

Table 2. SNR values achieving 1% BLER

| CBW (MHz) | CORESET RB | Payload size | AL | Propagation condition | Antenna configuration | SNR @ 1% BLER (alignment) | SNR @ 1% BLER (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | 35 | 4 | TDLA30-10 | 1x2 Low | 4.3 | 6.8 |
| 3 | 15 | 35 | 4 | TDLC300-100 | 2x2 Low | 0.5 | 3.0 |
| 3 | 15 | 35 | 4 | TDLA30-10 | 1x4 Low | -0.9 | 1.6 |
| 3 | 15 | 35 | 4 | TDLC300-100 | 2x4 Low | -4.1 | -1.6 |

- Conclusion

In this contribution, we provide simulation results on punctured PBCH and PDCCH for less than 5MHz WI.

- Reference

- R4-2402863, “WF on [110][318] NR_FR1_lessthan_5MHz_BW_demod”, Nokia, Nokia Shanghai Bell