3GPP TSG RAN WG4 Meeting #110bis		R4-2404161

Changsha, China, 15th April – 19th April 2024

Agenda item:	4.2.1.1

Source:	Apple

Title:	Remaining issues for enhanced channel raster

WI/SI:	NR_channel_raster_enh

Release:	Rel-18

Document for:	Decision

# 1	Introduction

During Rel-16 and Rel-17 discussions, several operators expressed an interest in enabling more efficient utilization of "non-standard" channel bandwidths, i.e., the ones which are not present now in TS 38.101 specifications. Referring to the corresponding operator requests, the following channel bandwidths were suggested by operators: 6, 7, 11, 12, 13, 33MHz. As an outcome a new SI was agreed at the RAN#89 meeting aiming to study further which existing solutions can be used and whether new mechanism should be devised [1]. The SI was concluded at RAN#99 meeting with the general conclusions that two methods – overlapping channels from network perspective and the next larger channel – can be used to support irregular channels without requiring changes at the UE side. In addition to that, companies concluded that the existing NR channel raster design is not flexible and does not allow configuring certain NR channel combinations. In response to that RAN#99 agreed a new WI with the only main objective to enable a more flexible raster design [2].

RAN WG4 concluded to introduce a new 10kHz raster that will be added to the specifications as an explicit channel raster for both UE and gNB. And the corresponding WI has been formally completed during the RAN#102 meeting. Nevertheless, what remained unresolved from RAN4#110 meeting is how to capture which bands should have mandatory enhanced channel raster and whether mandatory support can be applied to earlier releases.

# 2	Specification aspects of enhanced channel raster

## 2.1	Mandatory support of the enhanced raster

Referring to the latest version of TS 38.101-1, RAN WG4 agreed to introduce a new table to indicate clearly which bands can have enhanced channel raster. However, that table does not indicate whether enhanced channel raster is mandatory or not. Thus, to avoid any ambiguity on whether this feature is mandatory or optional for the given band, we suggest extending the corresponding tables in TS 38.101-1 and TS 38.101-5. As presented in Table 2.1-1 below we can add a new column, e.g. "Mandatory support", that will indicate whether a UE has to support the enhanced channel raster. And the corresponding CRs can be found in [3][4].

Table 2.1-1: Exemplary modifications to the channel raster table to indicate mandatory support.

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| n1 | 10 | 384000 – <2> – 396000 | 422000 – <2> – 434000 | Yes |
| REMAINING CONTENT OMITTED |  |  |  |  |

Proposal 1a:	Introduce a new column to the tables in TS 38.101-1 and TS 38.101-5 to indicate whether the enhanced channel raster is mandatory or not for the UE side.

It is worth noting that similar tables with bands supporting the enhanced channel raster were also added to the network side specifications, TS 38.104 and TS 38.108. However, since it is generally up to the network implementation whether to deploy enhanced channel raster or not, there are no compelling reasons to provide further clarifications to the specifications on whether this feature is mandatory for the network side.

Proposal 1b:	For the network side specifications, TS 38.104 and TS 38.108, there are no strong reasons to indicate whether the enhanced channel raster is mandatory or not because it is up to the network whether to deploy that feature.

## 2.2	Mandatory support in earlier releases

As discussed earlier by RAN WG4 and RAN WG2, a new per-band capability is added so that a UE can signal whether it supports enhanced channel raster of 10kHz. And based on the request from RAN WG4, this capability is added also to the earlier releases.

However, what remained unresolved from earlier discussions is whether enhanced channel raster can be mandatory for certain bands in earlier releases. From the perspective of the UE capability, it is of course possible to mandate enhanced channel raster also in earlier releases, but it is not clear how the corresponding information will be captured in the 3GPP specifications.

Proposal 2a:	If enhanced channel raster can be mandatory for the earlier releases, we ask RAN WG4 to decide how it will be captured in earlier releases.

As a somewhat related issues, there were several contributions during the RAN4#110 meeting indicating the need of mandating the enhanced channel raster for the Rel-17 RedCap devices. The rationale behind that proposal is that the RedCap devices support only 20MHz channel bandwidth, which could be incompatible with the system bandwidth of e.g. 25 or 35MHz. So, enabling the enhanced channel raster for the Rel-17 RedCap devices will enable more flexible system deployments. On the contrary to it, there were concerns raised on whether we can mandate enhanced channel raster for earlier releases and whether it is absolutely needed for all bands.

-	As agreed earlier by RAN WG4, the enhanced channel raster is needed only for specific bands. If a particular band does not require enhanced channel raster, then it is not clear why RedCap devices should be supporting this feature for the band. From that perspective RedCap devices can follow the same process that the enhanced channel raster is mandatory only for those bands where operators really need it.

-	Referring back to Proposal 2a and related considerations, it should be discussed separately whether we can mandate this feature for Rel-17 RedCap devices that belong to an earlier release. And, generally speaking, it is not only RedCap, but other also other scenarios and use cases introduced in earlier releases, which might need enhanced channel raster.

Proposal 2b:	Rel-17 RedCap devices can follow the same RAN WG4 process on defining for which bands the enhanced channel raster is mandatory (and potentially starting from which release).

# 3	Conclusions

In this discussion paper we have presented our further considerations on the specification related aspects of the enhanced channel raster. As summarized in our paper, we suggest enhancing the corresponding tables in TS 38.101-1 and TS 38.101-5 to add a new column that will indicate unambiguously whether the enhanced channel raster is mandatory or optional for the given band. The corresponding CR can be found in [3] and [4].

Finally, we ask RAN WG4 to conclude whether enhanced channel raster can be mandatory in earlier releases and, if so, how we can capture the corresponding information. The outcome of that discussion should be applicable to the Rel-17 RedCap devices and potentially other features, which were introduced in earlier releases and which could benefit from the ehanced channel raster.

Proposal 1a:	Introduce a new column to the tables in TS 38.101-1 and TS 38.101-5 to indicate whether the enhanced channel raster is mandatory or not for the UE side.

Proposal 1b:	For the network side specifications, TS 38.104 and TS 38.108, there are no strong reasons to indicate whether the enhanced channel raster is mandatory or not because it is up to the network whether to deploy that feature.

Proposal 2a:	If enhanced channel raster can be mandatory for earlier releases, we ask RAN WG4 to decide how it will be captured in earlier releases.

Proposal 2b:	Rel-17 RedCap devices can follow the same RAN WG4 process on defining for which bands the enhanced channel raster is mandatory (and potentially starting from which release).

# 4	References

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson

RP-230813, "New WID on Channel raster enhancement", Ericsson

R4-2404162, "Clarification for the mandatory support of enhanced channel raster for the TN bands", Apple Inc.

R4-2404163, "Clarification for the mandatory support of enhanced channel raster for the NTN bands", Apple, Ligado Networks, Inmarsat, Viasat, Globalstar, Thales, Hughes/Echostar