3GPP RAN TSG-RAN4 Meeting # 110-Bis		R4-2404178

Changsha, China, April 15 – 19, 2024

Agenda item:	9.1.4

Source:	Apple

Title:	Views on 6Rx for handheld UEs

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

A new WID on RF enhancements for NR FR1/FR2 and EN-DC was recently added on the list of Release 19 work items [1]. This WID covers three main working areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID, are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

In this discussion paper, we share our initial views on 6Rx for handheld UEs. Enabling 6Rx for large form-factor handheld UEs has the potential of improving DL coverage and throughputs. To achieve these potential performance improvements, key receiver performance metrics will have to be clearly understood and defined. We will focus our discussions in this paper on the following three receiver RF performance metrics:

- SRS antenna switching and insertion loss imbalance.

- Number of MIMO layer requirements for 6Rx.

- REFSENS requirements for 6Rx.

# Discussion

  - SRS Insertion Loss Imbalance

As the number of Rx antenna ports grows in handheld UEs, it becomes more challenging to accurately perform SRS antenna switching which relies on making UL channel measurements to predict quality of the DL channels. This is because the RF insertion loss differences between the main path (which is used for UL), and the Rx paths are higher and difficult to model. Several UE integration factors contribute to accentuate those insertion loss differences:

- Depending on the antenna architecture, different RF switch topologies ( SPST, SPDT, SP4T etc.) could be used on different branches. As depicted in Figure 2.2-1 below for example, for 2a, the insertion loss for the main (Tx1 path) and Rx1 paths will be lower than the insertion loss for the remaining 5 Rx paths since losses on a typical SP5T are higher than the losses on a SPST. That difference in insertion loss is improved with 2b, and with 2c there is no difference in theory.

- PCB trace losses will also be different between the main antenna path and the Rx diversity paths, due to their relative positions within the UE.

- Additionally, due to the physical location of each Rx diversity antenna relative to the main antenna path, RF coupling will also contribute to the difference in insertion loss between the main path and the Rx paths. This difference becomes complicated to model accurately as the number of Rx paths in handheld UEs increases from 2Rx/4x to 6Rx.

Figure 2.2-1: Examples of antenna configurations for 2T6R

Because of the differences in insertion loss between the main path and Rx paths as highlighted above, SRS antenna switching may not be accurate since it relies on the UL and DL channel reciprocity.

RAN4 has identified these insertion loss difference issues as SRS IL imbalance [2].

Some modeling has shown some potential impacts of SRS IL imbalance on the UE receiver throughputs. Figure 2.2-2 below shows simulation results for 4Rx with and without imbalance taking into consideration [3]. The IL imbalance matrix used for this simulation is [0, -3, -6, -6], meaning 0dB of imbalance on Rx1 relative to main antenna, 3dB on Rx2, 6dB on Rx3, and 6dB on Rx4. As can be observed, for 70% resource utilization (diamond marker), the throughput can degrade as much as 35Mbps/user when the SRS IL imbalance is considered.

Figure 2.2-2: Simulated MU-MIMO user throughput with/without SRS IL imbalance [3]

To address this imbalance issue, ∆TRxSRS for 4Rx has been introduced in the 3GPP specifications (TS 38.101.1) to allow for some reduction on the maximum configured output power for the UE when transmitting SRS. A summary of the transmitted power relaxation is highlighted below:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

∆TRxSRS is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ when

a)  UE transmits SRS on the second SRS resource in every configured SRS resource set when the SRS-TxSwitch capability is indicated as 't1r2' or 't1r1-t1r2'

b)  UE transmits SRS on the second, third and fourth SRS resources of the total 4 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the SRS-TxSwitch capability is indicated as 't1r4' or, 't1r4-t2r4' or 't1r1-t1r2-t1r4' or, 't1r1-t1r2-t2r2-t1r4-t2r4'

c)  UE transmits SRS from the second SRS port pair on the second SRS resource in every configured SRS resource set consisting of two SRS ports when the SRS-TxSwitch capability is indicated as ' t2r4' or ' t1r4-t2r4', or 't1r1-t1r2-t2r2-t2r4' or 't1r1-t1r2-t2r2-t1r4-t2r4', or

d)  UE transmits SRS to a DL-only carrier

The value of ∆TRxSRS is 4.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16..

The value of ∆TRxSRS is 7.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16.

For other SRS transmissions ∆TRxSRS is zero.

Increasing the number of Rx paths on handheld UEs will potentially increase the SRS IL imbalance due to the integration challenges. As a result, the value of the relaxation ∆TRxSRS will be higher for 6Rx compared to 4Rx.

Observation#1: The value of ∆TRxSRS relaxation will be higher for 6Rx compared to 4Rx due to the increase in UE integration complexity when more Rx paths are added.

The defined relaxation shown above describes the tolerance for SRS IL imbalance. This SRS IL imbalance could potentially degrade the Rx throughput as stated above if it’s not addressed. IL pre-compensation has been proposed as a potential solution to mitigate the undesirable effects of the imbalance [4]. This solution allows the UE to boost (or to compensate) the SRS transmission power on the diversity branches that exhibits SRS IL imbalance.

In our views, IL pre-compensation could be a viable solution to address the SRS IL imbalance issue when the UE is not UL power limited. It seems possible for a UE to compensate for the extra insertion loss on the Rx diversity ports relative to the primary antenna port when it has sufficient power available to do so. But when the UE is operating at close to maximum output power, overcoming additional insertion loss would be more challenging so there could be some residual imbalance that cannot be compensated. In that case, IL pre-compensation may not be very effective.

Observation#2: IL pre-compensation could be effective when the UE is not Tx power limited but could be sub-optimal in power limited situations.

Even if IL pre-compensation is effective on addressing the SRS IL imbalance issue, there are some other aspects to consider when implementing SRS antenna switching in 6Rx capable UEs:

- UE factory test time:

IL pre-compensation relies on the UE to accurately know the IL difference between the main antenna path and the Rx paths. Given the fact that the IL difference is affected by several RF front-end components as stated earlier in this paper, this parameter could vary significantly from UE to UE. Therefore, factory calibration would be needed to minimize the variations from part to part. As an illustration, for 2T4R configuration for 4Rx capable UEs, a total of 10 calibrations will be needed (this include the main paths). For 2T6R, this number will increase from 10 to 14 calibrations. Increasing the number of UE factory calibrations will negatively impact the UE test time and cost.

Observation #3: Implementing IL pre-compensation would require UE factory calibration. As a result, UE test time and cost will increase.

- UE memory allocation:

To implement certain features, the configurations and calibration data may have to be downloaded in advance and stored in a UE memory prior to its usage. Because the UE has a finite memory space, the amount of data to be preloaded needs to be minimize. As an example, to implement 2T4R SRS antenna switching, it would require the UE to store in advance  8 data sets (one data set being defined as one set of Tx-Rx calibration data). For 2T6R, this number will increase to 12 data set. This increase in UE memory allocation will increase the UE cost as well.

Proposal #1: The performance benefits of SRS antenna switching for 6Rx in handled UEs will have to be well understood and quantified. This feature should remain optional if those benefits do not clearly outweigh the implementation challenges and the increase in UE cost.

  - MIMO Layers

One of the main objectives of this WID is 1) to support 4 MIMO layers as a baseline, and 2) study the gain and feasibility of supporting 6 MIMO layers with 6Rx. Again, enabling 6Rx for larger form-factor UEs may have the potential of improving DL coverage and throughput, but having more antennas in the same panel will make it difficult to maintain and scale good MIMO performance.

Observation #4: Due to limited area to accommodate more antennas, achieving low antenna correlation for 6Rx will be challenging.

The crucial part here, however, is determining the antenna correlation parameters under which the feasibility of 6 MIMO layers will be evaluated. For example, during the 8Rx work item, it was decided that demodulation performance criteria would be specified under “low correlation” conditions, which was a reasonable assumption given the FWA and CPE form factors. For 6Rx on smartphones, different antenna correlations must be determined and agreed by stakeholders.

Observation #5: FR1 conducted requirements are performed at the antenna port level, under which a suitable channel environment is applied via test equipment as per demodulation requirements definition.

Proposal #2: The antenna correlation assumptions need to be thoroughly studied and understood since the decision of whether 6 MIMO Layers is beneficial will strongly depend on simulation results based on this decision.

Proposal #3: During the discussion of this work item, interested companies should propose concrete antenna correlation analysis based on antenna spacing and other physical considerations.

In terms of methodology, and after the antenna correlation has been decided upon, at least two factors should be considered to establish the viability of 6 MIMO layers.

- First, at what SNR do we see throughput benefits relative to 4 MIMO layers? According to demodulation performance standard practices, an SNR value of 30dB is often regarded an upper bound for practicality and testability. If we only detect increases in the very high SNR range under the agreed-upon correlation parameters, then the feature's prospects may not be particularly promising. On the other hand, if throughput improvements are seen much below 30dB, the feature may pose value.

Proposal #4: The viability of 6 MIMO Layers should be analyzed and understood during RAN4 demodulation performance discussions, along with the discussion of what demodulation requirements should be determined.

- Secondly, however, the amount of the increase in throughput must also be addressed. We feel that the gain in throughput should much surpass the rise in UE cost for the functionality to be practical. For example, components of the modem that scale with the number of Rx, such as channel/parameter estimation engines, MIMO detectors, etc., will become more complicated. Furthermore, increasing the number of CWs from one to two CWs will result in increased modem complexity.

Proposal #5: The performance benefits of 6 MIMO Layers vs 4 MIMO Layers will have to be well understood and quantified. In general, we propose this feature should remain optional if those benefits do not clearly outweigh the implementation and UE cost challenges.

  - REFSENS Requirements

4Rx for LB (< 1GHz) targeting FWA UEs has already been introduced in the 3GGP specifications through the completed WID RP-223517 [5] and the requirements for REFSENS are highlighted below (from Table 7.3.2.2 of 3GPP TS38.101-1):

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n28, n71, n85, n195 | -2.71 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: 4 Rx operation is targeted for FWA form factor |  |

Table 2.3-1: REFSENS Requirements for 4Rx

Given the throughput gains with 4Rx [6], it seems natural to introduce 6Rx for handheld UEs. However, although the newly approved 6Rx WID targets HB and UHB frequencies and  larger form-factor UEs such as foldable handhelds, there are some implementation challenges that could degrade REFSENS when the number of antennas increases to six:

- Increasing the number of Rx antennas to six will increase the complexity of the RF front-end design and this could negatively impact the reference sensitivity as six Rx chains need to be placed in the limited area of a handheld device. The LNA has a major impact on the receiver sensitivity and its placement relative to the RF front-end components plays a major role in determining the overall receiver noise figure and hence the REFSENS. Implementing 6Rx could make it difficult to guarantee optimal positions of the RF front end components, giving the limited space. This could then lead to degraded REFSENS for certain bands compared to 4Rx or 2Rx.

- RF front layout will also be difficult with 6Rx implementation. At higher frequencies, achieving good RF isolation between the different RF paths is challenging. This will lead to degraded single-carrier REFSENS due to poor RF coupling as well as higher MSD requirements for the carrier aggregation and dual-connectivity combinations that are sensitive to cross-band coupling.

- The antenna performance could be compromised as well: As the number of antennas increases, it becomes challenging to have sufficient volume for each antenna to achieve good efficiency. Antenna radiated performance will therefore be impacted. Additionally, RF isolation between the antenna elements will suffer, especially at higher frequencies, which are the target frequencies for the WID. Good MIMO performance relies on good antenna isolation and correlation. Having more antennas will therefore make it difficult to maintain good MIMO performance.

Observation #6: Due to limited area to accommodate more antennas, achieving good REFSENS for 6Rx will be challenging due to sub-optimal component placement, poor isolation at HB and UHB frequencies, and RF coupling between the Rx paths.

Proposal #6: RF isolation at HB and UHB frequencies and antenna placement should be well studied and understood because of their impact on REFSENS for 6Rx handheld UEs.

# 3	Conclusions

Observation#1: The value of ∆TRxSRS relaxation will be higher for 6Rx compared to 4Rx due to the increase in UE integration complexity when more Rx paths are added.

Observation#2: IL pre-compensation could be effective when the UE is not Tx power limited but could be sub-optimal in power limited situations.

Observation #3: Implementing IL pre-compensation would require UE factory calibration. As a result, UE test time and cost will increase.

Proposal #1: The performance benefits of SRS antenna switching for 6Rx in handled UEs will have to be well understood and quantified. This feature should remain optional if those benefits do not clearly outweigh the implementation challenges and the increase in UE cost.

Observation #4: Due to limited area to accommodate more antennas, achieving low antenna correlation for 6Rx will be challenging.

Observation #5: FR1 conducted requirements are performed at the antenna port level, under which a suitable channel environment is applied via test equipment as per demodulation requirements definition.

Proposal #2: The antenna correlation assumptions need to be thoroughly studied and understood since the decision of whether 6 MIMO Layers is beneficial will strongly depend on simulation results based on this decision.

Proposal #3: During the discussion of this work item, interested companies should propose concrete antenna correlation analysis based on antenna spacing and other physical considerations.

Proposal #4: The viability of 6 MIMO Layers should be analyzed and understood during RAN4 demodulation performance discussions, along with the discussion of what demodulation requirements should be determined.

Proposal #5: The performance benefits of 6 MIMO Layers vs 4 MIMO Layers will have to be well understood and quantified. In general, we propose this feature should remain optional if those benefits do not clearly outweigh the implementation and UE cost challenges.

Observation #6: Due to limited area to accommodate more antennas, achieving good REFSENS for 6Rx will be challenging due to sub-optimal component placement, poor isolation at HB and UHB frequencies, and RF coupling between the Rx paths.

Proposal #6: RF isolation at HB and UHB frequencies and antenna placement should be well studied and understood because of their impact on REFSENS for 6Rx handheld UEs.

# 4	References

R4-2403716 “New WID: UE RF enhancements for FR1/FR2 and EN-DC , Phase 4”, by RAN4 chair (Huawei),  3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

R4-2303519 “LS on the UE SRS IL imbalance issue”, by Huawei, 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, February 27 – March 3, 2023

R1-2308129 “Discussion on RAN4 LS on the UE SRS imbalance issue” by Ericsson, 3GPP TSG-RAN WG1 Meeting #114, Toulouse, France, August 21st– 25th, 2023

R102308458 “FL summary on UE SRS IL imbalance issue”, by Moderator (Huawei), 3GPP TSG RAN WG1 #114, Toulouse, France, August 21st– 25th, 2023

RP-223517  “Low NR band 4Rx for handheld UE and 3Tx for inter-band UL CA and EN-DC”, OPPO, China Telecom, China Unicom, CMCC, NTT DoCoMo, Orange, Telecom Italia, T-Mobile USA, Verizon, Vodafone, RAN#98-e meeting, Dec 2022

RP-223026 “Motivation of low band enhancement for handheld UE”, by Oppo, 3GPP TSG RAN Meeting #98e, Electronic Meeting, December 12-16, 2022