3GPP TSG-RAN WG4 Meeting #110-bis	R4-2404200

# Changsha, China, 15th - 19th, April, 2024

Source: 	KDDI

Title: 	Discussion on non-collocated EN-DC/NR-CA for Type 4a/4b

Agenda Item:	9.7.2

Document for:	Discussion

- 1. Introduction

In RAN#110 meeting, a new WI for intra-band non-collocated EN-DC/NR-CA is approved [1]. The WI’s core part is focusing on specifying UE RF/RRM requirements for Type-4 UE capability for FWA and clarifying the UE behaviour for Type-4 supported UE.

- 2. Discussion

2.1 Background

Firstly, we share the operators’ motivation of non-collocated scenarios here again based on [2].
Regarding co-existence with other systems, different co-existence conditions apply for C-band. As a result, Tx antenna collocation network side is not always available. Regarding timing of spectrum allocation, the three blocks in C-band were allocated at different time in Japan (Figure-1), and also, the two blocks were allocated at different time in Korea (Figure-2). This makes Tx antenna collocation cost-inefficient or sometimes infeasible.

Figure-1. Spectrum point of view in Japan

Figure-2. Spectrum point of view in Korea

Observation 1: There are still needs from operators to deploy non-collocated EN-DC/NR-CA.

2.1 Working assumption

According to WID [1], RAN4 need to specify Type-4a/4b capability for FWA. Based on the working assumption previously agreed as WF in RAN4#106 [3], in RAN#103 we propose the following working assumption in Table-1 same as [2].

Table-1. Working assumption for UE architecture

Observation 2: The working assumption of RP-240101 can be reused in RAN4.

2.2 Reference UE architecture

In RAN4#xxx, UE reference architecture for Type 4b was proposed as Rel-18 in Figure-1 [4], and also it can be reused in Rel-19.

Figure-1. Reference UE architecture

2.3 UE RF requirements for Type 4a/4b

Considering that above working assumption and reference UE architecture for Type 2 (4Rx, separated Rx chains), Type 4a (6Rx, separated Rx chains) and Type 4b (8Rx, separated Rx chains),  Type 2 requirements and mechanism can be reused for Type 4a/4b. Additionally, the following WF on power imbalance requirement for Type 4a/4b is captured in [5].

< Issue 4-2-1: Whether UE Type 2 power imbalance is re-used for Type 4a/4b > Way forward: - In case of that RAN4 discuss Type 4a/4b in future release, Type 2 requirement will be a baseline.

Proposal 1: Reuse 25dB power imbalance requirements on Type 2 for Type 4a/4b

Proposal 2: Reuse other UE RF requirements except on Type 2 for Type 4a/4b

Proposal 3: Add new UE capability for Type 4a/4b support indication and new network signaling to configure UE behavior between Type 4a/4b (Non-collocated) and Type 1 (Collocated). Details up to RAN2 decision.

- 3. Conclusion

Observation 1: There are still needs from operators to deploy non-collocated EN-DC/NR-CA.

Observation 2: The working assumption of RP-240101 can be reused in RAN4.

Proposal 1: Reuse 25dB power imbalance requirements on Type 2 for Type 4a/4b

Proposal 2: Reuse other UE RF requirements except on Type 2 for Type 4a/4b

Proposal 3: Add new UE capability for Type 4a/4b support indication and add new network signaling to configure UE behavior between Type 4a/4b (Non-collocated) and Type 1 (Collocated). Details up to RAN2 decision.

- 4. Reference

[1] RP-230838 New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), Apple, RAN#103

[2] RP-240101 Motivation on Non-collocated ENDC,NRCA in Release 19, KDDI, Docomo, KT, LG Uplus, SoftBank, Telecom Italia, Samsung, RAN#103

[3] R4-2303696 WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI, RAN4#106

[4] R4-2215736, Views on UE RF aspect for non-collocated EN-DC, NR-CA deployment, Samsung, RAN4#104-bis-e

[5] R4-2306605 WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI, RAN4#106-bis-e