3GPP TSG-RAN WG4 Meeting # 110bis                                                                     R4-2404202

Changsha, China, 15 April – 19 April 2024

Title: 	Test tolerance for MIMO OTA FR1

Source: 	Huawei, HiSilicon

Agenda item:	6.10.2

Document for:	discussion

# Introduction

TS 38.161 [2] for TRP and TRS has the following statement in section 4.1

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification in RAN5 will define final test tolerances for FR1 TRP TRS. The test tolerances are used to relax the minimum requirements in this specification to create test requirements.

TS 38.151 [3] for MIMO OTA has a similar statement in section 4.1

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification in RAN5 will define test tolerances for FR1 and FR2 MIMO OTA. The test tolerances are used to relax the minimum requirements in this specification to create test requirements.

Both of the above statements indicate that RAN5 will provide Test Tolerance (TT) values. However, LS [1] from RAN5 requests RAN4 to provide TT value for TRP and TRS.

In anticipation of a similar request for MIMO OTA, this contribution proposes TT value for FR1 MIMO OTA. Similar approach could be taken for MIMO OTA FR2 regarding its TT value.

Besides, revised WID [9] states that RAN4 is to provide TT value, with a qualifier that “the final MU and test tolerances (TT) will be decided by RAN5”.

# Discussion on TT values

The TT value for TRP and TRS [4] has been agreed to be 0.62*MU, which is option 4 out of the available options listed below [6].

- Option 1: Define TT=0.5* Preliminary MU budget, i.e. 0.9dB for TRP, and 1.1 dB for TRS. [Orange, TIM, AT&T]

- Option 2: TT values are not directly driven from assessed MU budget. Propose TT= 1.1 dB for TRP, and 1.3 dB for TRS. [Samsung, vivo, AT&T, Apple, Xiaomi]

- Option 3: Define TT values as the same as lab alignment pass/fail limit [0.75*MU], i.e. 1.34dB for TRP, and 1.65dB for TRS. [Xiaomi]

- Option 4: 0.62 * MU as TT recommendation (1.1 dB TT for TRP and 1.36 dB TT for TRS) in line with actual RAN4 3GPP lab alignment framework and measurements [Apple]

In fact, option 4 comes from the worst-case deviation in lab alignment tests [8], using the principle of option 3, namely TT should be the same as the lab alignment pass/fail limit.

RAN4 has agreed the LTE MIMO OTA TRMS TT value [5] to be 1dB based on shared risk principle, where TT ≈ ½ MU [7]. However, it would be consistent for MIMO NR OTA FR1 to follow the same approach as NR TRP and TRS to use lab alignment pass/fail limit or its worst-case deviation.

Proposal 1: consider the following options for MIMO OTA NR FR1 test tolerance value:

Option 1: lab alignment pass/fail limit

Option 2: the worst-case deviation in lab alignment tests

# Conclusions

This contribution makes the following proposal.

Proposal 1: consider the following options for MIMO OTA NR FR1 test tolerance value:

Option 1: lab alignment pass/fail limit

Option 2: the worst-case deviation in lab alignment tests

# References

[1] R4-2404119 LS to RAN4 on TT work for Rel-18 NR FR1 TRP TRS

[2] TS 38.161 v18.0.0 User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements; Range 1 Standalone and Range 1 Interworking operation with other radios

[3] TS 38.151 v17.6 Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements for NR UEs

[4] R4-2214797 Reply LS on TT and requirements, RAN4#104e, 15-25 August, 2022

[5] R4-1714204 LS on MIMO OTA test tolerance

[6] R4-2214321 Email discussion summary for [104-e][332] FR1_TRP_TRS_Part1

[7] R4-1710390 Test tolerance for TRMS

[8] R4-2212405 On TRP and TRS performance requirement phase

[9] RP-233742 Revised WID: Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs