3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404212

# Changsha, China, 15th – 19th April, 2024

Agenda item:	6.20.3

Source:	Qualcomm Incorporated

Title:	On NR SL enhancement RRM Requirement Performance

Document for:	Discussion

# Introduction

In this contribution, we present our view on the scope of R18 SL enhancement.

# Discussion

We have the following observations on the potential RAN4 impact from different SL procedures discussed in RAN1.

## RRM performance requirement for SL-U

For the accuracy requirement for LBT, since it’s identical to NR-U LBT, we have the following proposal:

Proposal 1: Reuse RSSI accuracy requirements from NR-U for SL-U LBT measurement.

## RRM performance requirement for SL-CA

In the LTE CA test procedure, the following is specified:

For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X sidelink communication both on RF channel 1 and on RF channel 2. RF channel 1 and on RF channel 2 are both included in syncFreqList.

However, in order to enable the UE to transmit on RF channel 1 and 2, a carrier has to be added to channel 2 if UE initial SL transmission is on RF channel 1. However, RAN4 concluded the following requirement in RAN4#109:
Agreement: Introduce constraints on interruptions to WAN at SL carrier addition/release ONLY due to paging and SIB reception for UE in RRC_IDLE/INACTIVE mode.

Based on the understanding that the carrier can be configured at time t, but added by SL UE at an arbitrary time t’, and therefore carrier addition can happen in idle mode. Based on this understanding, when TE configured carriers on RF channels 1 and 2, UE may not add the additional carrier before the test period (T1, T2, T3 in the LTE test) starts.

Observation 1: Based on this understanding from RAN4#109 WF, when TE configured carriers on RF channels 1 and 2 as in LTE corresponding test case, UE may not add the additional carrier before the test period (T1, T2, T3 in the LTE test) starts.

Therefore, we can’t properly test the UE search requirement under CA, when we don’t know when the DUT UE will add the additional carrier and transmit on that carrier. Therefore, we propose not to have the agreed test on “Selection/Reselection of synchronization reference source under NR SL CA operation” due to the testability issue.

Proposal 2: Do not to introduce the agreed test on “Selection/Reselection of synchronization reference source under NR SL CA operation” due to the testability issue.

# Conclusions

Proposal 1: Reuse RSSI accuracy requirements from NR-U for SL-U LBT measurement.

Observation 1: Based on this understanding from RAN4#109 WF, when TE configured carriers on RF channels 1 and 2 as in LTE corresponding test case, UE may not add the additional carrier before the test period (T1, T2, T3 in the LTE test) starts.

Proposal 2: Do not to introduce the agreed test on “Selection/Reselection of synchronization reference source under NR SL CA operation” due to the testability issue.