# 3GPP TSG-RAN WG4 Meeting #110bis                                                           R4-2404229

# Changsha, China, 15th - 19th April 2024

# Title:	TP for TR38.718-03-01 Support of CA_n1-n77-n79

# Source:	Softbank

# Agenda Item:	5.11.2

# Document for:	Approval

- Introduction

This contribution is a text proposal for TR 38.718-03-01[5] to include the following NRCA combinations as requested in RAN4#109.

- CA_n1-n77-n79

- Reference

[1] 3GPP TR 21.905 Vocabulary for 3GPP Specifications (Release 17) V17.1.0

[5] TR38.718-03-01, Rel-18 NR inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL(x=1,2) V0.10.0

- Text Proposal

- -- Start of TP –

-- Unaffected parts omitted –

- 5.x	CA_n1- n77-n79

- 5.x.1	Common for 1 band UL and 2 bands UL CA

- 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n77-n79 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

- 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n1+n40+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n77(2A)-n79 | CA_ n77(2A) | n1 | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 70, 80, 90, 100 |  |

- 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n40-n77, the TIB,c and RIB,c values are reused from CA_n1-n40-n78 and are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n1-n77-n79 | 0.6 | 0.8 | 0.5 |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n1-n40-n77 | 0.2 | 0.5 | - |

- 5.x.2	Specific for 2 bands UL CA

- 5.x.2.1	UE co-existence studies

UL n77-n79 gives IMD into DL n1.

- 5.x.2.2	REFSENS requirements

Based on the co-existence studies there are a need to define MSD values. MSD values from CA_n1-n77-n79 are reused.

Table 5.x.2.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n40-n77 CA_1A-n77(2A)-n79A | n1 | N/A | 5 | N/A | 2140 | 6.0 | FDD | IMD1, 2 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| NOTE 1:This band is subject to IMD3 also which MSD is not specified. NOTE 2:For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified |  |  |  |  |  |  |  |  |

-- Unaffected parts omitted --

-- End of TP --