3GPP TSG-RAN WG4 # 110-bis	R4-2404235

Changsha, China, April 15 – April 19, 2024

Agenda item:	6.3.4.3

Source:	MediaTek Inc.

Title:	Simulation results of PMI requirements of FR2 Multi-Rx DL Demod

Document for:	Discussion

# Introduction

In RAN4#110, the Way Forward document was approved [1]. We provide final simulation results of PMI reporting requirements for simulation alignment for the Rel-18 work in this contribution.

# Simulation results

In this chapter we provide final simulation result of agreed PMI reporting requirement test for final result alignment.

We have simulated Single-DCI SDM transmission scheme by using separate processing receiver assumption as agreed.

Channel model is TDLA30-35 for 100 MHz/120 kHz. The correlation matrix model follows agreement from the previous meeting [1]. MCS13 is simulated as agreed. TX EVM is 6% as agreed. For phase noise modelling we are using Example 1 model from TR38.803 [2] assuming carrier frequency parameters for 29.55 GHz. Time and frequency offsets between TRPs are set to zero.

The simulated cross-talk power ratios ρ is -12dB as agreed. Here, ρ in [dB] is defined as 10·log10(ρ). In these simulations we have not assumed any additional scaling to TRPs as agreed in the previous meetings.

Table 1 collects agreed test configurations using Multi-TRP Single-DCI SDM scheme with MCS13 Rank 2 using separate processing receiver assumption with single and dual port PTRS configurations.

|  | ρ=-12dB |
| --- | --- |
| MCS13 / separate processing / 90% tput / single port PT-RS | 1.29 |
| MCS13 / separate processing / 90% tput / dual port PT-RS | 1.29 |

Table 1: Simulation result collection of Single-DCI SDM scenario with MCS13
showing (throughput ratio) γ in SNR of follow PMI in 90% of maximum throughput
with single and dual port PT-RS configurations.

From simulation result in Table 1, we can do following observation:

Observation #1: In our simulations γ value without margins is 1.29 for both PT-RS configurations.

# Conclusion

In this paper we provided simulation results on the PMI reporting requirements of FR2 multipanel RX downlink for performance alignment. The following observation is made:

Observation #1: In our simulations γ value without margins is 1.29 for both PT-RS configurations.

# Reference

- R4-2402859, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm

- 3GPP TR 38.803 V14.3.0 (2022-03), “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”