3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2404239

# Changsha, China, April 15 - April 19

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-4: PDSCH requirements of MU-MIMO advanced receiver in 4Rx TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-Perf |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PDSCH requirements for MU-MIMO advanced receiver agreed to be defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new tests • Add new Chapter 5.2.3.2.22 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements for MU-MIMO advanced receiver will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38-521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

##### 5.2.3.2.[22]	Minimum requirements for PDSCH with intra-cell inter-user interference

The performance requirements are specified in Table 5.2.3.2.[22]-3 and and Table 5.2.3.2.[22]-4, with the addition of test parameters in Table 5.2.3.2.[22]-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.[22]-1.

Table 5.2.3.2.[22]-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions, when the PDSCH transmission of target UE is interfered by co-scheduled UE. | 1-1, 1-2, 2-1, 2-2 |

Table 5.2.3.2.[22]-2: Test parameters

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration (Note 1) | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | {1000} for test 1-1 and test 1-2 {1000, 1001} for test 2-1 and test 2-2 | {1001} for test 1-1 and test 1-2 {1002, 1003} for test 2-1 and test 2-2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for test 1-1 and test 1-2 2 for test 2-1 and test 2-2 | 1 for test 1-1 and test 1-2 2 for test 2-1 and test 2-2 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. [Any column of precoder matrix is not equal to any column of precoder matrix of Target UE for test 1-1 and test 1-2 Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH for test 2-1 and 2-2] |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 |  |
| Number of HARQ Processes |  |  | 8 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 | N/A |
| Co-scheduled UE information (Note 2) |  |  | [1] for test 1-1 6 for test 1-2 [1] for test 2-1 6 for test 2-2 |  |
| Note 1: DMRS scrambling ID is the same for both target and co-shceduled UEs. Note 2: Modulation order information of co-scheduled UE, see Table 7.3.1.2.2-12 of TS38.212. |  |  |  |  |

Table5.2.3.1.[22]-3: Minimum performance for target UE with Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [R.PDSCH.7-1.1 TDD] | 40 / 30 | [16QAM, 0.48] | Random [16QAM] symbols | FR1.30-1 | [TDLC300-100] | 2x4, [ULA Low] | 70 | TBD |
| 1-2 | [R.PDSCH.7-1.1 TDD] | 40 / 30 | [16QAM, 0.48] | Random [16QAM] symbols | FR1.30-1 | [TDLC300-100] | 2x4, [ULA Low] | 70 | TBD |

Table 5.2.3.2.[22]-4: Minimum performance for target UE with Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | [R.PDSCH.7-1.2 TDD] | 40 / 30 | [16QAM, 0.48] | Random [16QAM] symbols | FR1.30-1 | [TDLA30-10] | 4x4, [ULA Low] | 70 | TBD |
| 2-2 | [R.PDSCH.7-1.2 TDD] | 40 / 30 | [16QAM, 0.48] | Random [16QAM] symbols | FR1.30-1 | [TDLA30-10] | 4x4, [ULA Low] | 70 | TBD |

END OF CHANGE 1