3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404285

Changsha, China, 15 – 19 April 2024

Agenda Item:	9.3.2

Source: 	Nokia

Title: 	LS reply for NR in 4400 to 4800 MHz

Document for:	Approval

# 1.	Introduction

The SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#103 [1]. One of the objectives of this SI is to send LS reply to ITU-R WP 5D (CC to TSG RAN) regarding NR in 4400 to 4800 MHz.

It is stated in the ‘Justification’ of the approved SID that:

For the frequency range 4400 to 4 800 MHz RF parameters can be extracted from specifications TS 38.104 and TS 38.101-1 and BS antenna parameters can be extracted from the extended array antenna model in R4-2108080 applicable for 1710 to 4990 MHz.

This contribution discusses the BS antenna parameters extracted from the extended array antenna model in R4-2108080 [2] and proposes some updates according to better represent actual deployment scenarios.

# 2.	Discussion

The BS antenna parameters from the extended array antenna model in [2] are extracted below.

Table 1-2: Beamforming antenna characteristics supporting subarray structures for IMT in 1710 – 4990 MHz Macro Rural; Macro suburban; Macro urban. 1; Base station Antenna Characteristics. 1.1; Antenna pattern; Refer to Table 1-1. 1.2; Element gain (dBi) (Note 2); 6.4; 6.4; 6.4. 1.3; Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 65º for V; 90º for H 65º for V; 90º for H 65º for V. 1.4; Horizontal/vertical front-to-back ratio (dB); 30 for both H/V; 30 for both H/V; 30 for both H/V. 1.5; Antenna polarization; Linear ±45º; Linear ±45º; Linear ±45º. 1.6; Antenna sub-array configuration (Row × Column) (Note 4); 4 × 8 elements; 4 × 8 elements; 4 × 8 elements. 1.7; Horizontal/Vertical radiating sub-array spacing; 0.5 of wavelength for H, 2.1 of wavelength for V; 0.5 of wavelength for H, 2.1 of wavelength for V; 0.5 of wavelength for H, 2.1 of wavelength for V. 1.7a; Number of element rows in sub-array; 3; 3; 3. 1.7b; Vertical element separation in sub-array (dv,sub); 0.7 of wavelength of V; 0.7 of wavelength of V; 0.7 of wavelength of V. 1.7c; Pre-set sub-array down-tilt (degrees); 3; 3; 3. 1.8; Array Ohmic loss (dB) (Note 2); 2; 2; 2. 1.9; Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3); 28; 28; 28. 1.10; Base station horizontal coverage range (degrees); +/-60; +/-60; +/-60. 1.11; Base station vertical coverage range (degrees) (Note 1); 90-100; 90-100; 90-100. 1.12; Mechanical down-tilt (degrees); 3; 6; 6. Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as in ITU-R M.2101. Note 2: The element gain in row 1.2 includes the loss given in row 1.8 and is per polarization. Note 3: The conducted power per sub-array assumes 4x8x2 sub-arrays (i.e. power per H/V polarized element). Note 4: 4 × 8 means there are 4 vertical and 8 horizontal radiating sub-arrays. Note 5: For the case of 3 elements per sub array, dv will be 2.1 wavelengths.

Firstly, there is an ambiguity which antenna polarization is used, as ‘Linear ±45º’ is stated in Row 1.5 but ‘H/V polarized element’ is stated in Note 3. It is proposed to clarify that ‘Linear ±45º’ antenna polarization is used, as this better represents the antenna polarization in actual deployment scenarios than ‘H/V polarized element’. Also, the word ‘element’ in Note 3 may be misinterpreted as ‘antenna element’ when the scope of Note 3 is ‘sub-array’.

Proposal 1: Update the term ‘H/V polarized element’ to ‘linear ±45º polarized sub-array’ in Note 3.

Secondly, the word ‘elements’ in Row 1.6 may be misinterpreted as ‘antenna elements’ when it means ‘sub-arrays’ as stated in Note 4.

Proposal 2: Remove the word ‘elements’ from Row 1.6.

Thirdly, the pre-set sub-array down-tilt is typically set to 6 degrees instead of 3 degrees in actual deployment scenarios. It is proposed to increase the pre-set sub-array down-tilt (degrees) from ‘3’ to ‘6’ in Row 1.7c and reduce the mechanical down-tilt in Row 1.12 accordingly to keep the current overall down-tilt.

Proposal 3: Increase the pre-set sub-array down-tilt (degrees) from ‘3’ to ‘6’ in Row 1.7c and reduce the mechanical down-tilt in Row 1.12 accordingly to keep the current overall down-tilt.

Moreover, the antenna characteristics for AAS base stations for bands between 1710 and 4990 MHz in small cell cases (micro urban and indoor urban) were provided to ITU-R WP5D in R4-2008924 [3], they are extracted below.

TABLE 3 Beamforming antenna characteristics for IMT in 1710 – 4990 MHz Rural; Macro suburban; Macro urban; Small cell outdoor/ Micro urban; Small cell indoor/ Indoor urban. 1; Base station Antenna Characteristics. 1.1; Antenna pattern; Refer to Recommendation ITU-R M.2101; N/A. 1.2; Element gain (dBi) (Note 2); 7.1; 7.1; 6.4; 6.4; N/A. 1.3; Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 54º for V; 90º for H 54º for V; 90º for H 65º for V; 90º for H 65º for V; N/A. 1.4; Horizontal/vertical front-to-back ratio (dB); 30 for both H/V; 30 for both H/V; 30 for both H/V; 30 for both H/V; N/A. 1.5; Antenna polarization; Linear ±45º; Linear ±45º; Linear ±45º; Linear ±45º; N/A. 1.6; Antenna array configuration (Row × Column) (Note 4); 8 × 8 elements; 8 × 8 elements; 8 × 8 elements; 8 × 8 elements; N/A. 1.7; Horizontal/Vertical radiating element spacing; 0.5 of wavelength for H, 0.9 of wavelength for V; 0.5 of wavelength for H, 0.9 of wavelength for V; 0.5 of wavelength for H, 0.7 of wavelength for V; 0.5 of wavelength for H, 0.7 of wavelength for V; N/A. 1.8; Array Ohmic loss (dB) (Note 2); 2; 2; 2; ## 2; N/A. 1.9; Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3); 25; 25; 25; 16; N/A. 1.10; Base station maximum coverage angle in the horizontal plane (degrees); 120; 120; 120; 120; N/A. 1.11; Base station vertical coverage range (degrees) (Note 1); 90-100; 90-100; 90-120; 90-120; N/A. 1.12; Mechanical downtilt (degrees); 3; 6; 10; N/A; N/A. Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as in ITU-R M.2101. Note 2: The element gain in row 1.2 includes the loss given in row 1.8. Note 3: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 4: 8 × 8 means there are 8 vertical and 8 horizontal radiating elements. In the sub-array case, one implementation is 2 vertical radiating elements combined in a 2x1 sub-array.

Note that antenna sub-array was not included in [3]. Since it is not clear whether antenna sub-array would be commonly used in small cell (micro-urban and indoor urban) cases in actual deployment scenarios in 4400 to 4800 MHz, it is proposed to reuse the antenna characteristics in [3] for small cell (micro-urban and indoor urban) cases.

Proposal 4: Reuse the antenna characteristics in R4-2008924 for small cell (micro-urban and indoor urban) cases.

# 3.	Conclusion

This contribution has discussed discusses the BS antenna parameters extracted from the extended array antenna model in [2] and proposed some updates according to actual deployment scenarios.

Proposal 1: Update the term ‘H/V polarized element’ to ‘linear ±45º polarized sub-array’ in Note 3.

Proposal 2: Remove the word ‘elements’ from Row 1.6.

Proposal 3: Increase the pre-set sub-array down-tilt (degrees) from ‘3’ to ‘6’ in Row 1.7c and reduce the mechanical down-tilt in Row 1.12 accordingly to keep the current overall down-tilt.

Proposal 4: Reuse the antenna characteristics in R4-2008924 for small cell (micro-urban and indoor urban) cases.

The proposals above are implemented in the table below using the template in [1], where the updates compared to [2] and [3] are highlighted in yellow.

TABLE 2

Beamforming antenna characteristics for IMT-2030 in 4400-4800 MHz

|  |  | Rural macro (If it’s available) | Suburban macro | Urban macro | Urban small cell (outdoor)/Micro cell | Indoor (small cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station antenna characteristics |  |  |  |  |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  | N/A |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 | 6.4 | N/A |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V | N/A |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V | 30 for both H/V | N/A |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º | Linear ±45º | N/A |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 4 × 8 | 4 × 8 | 4 × 8 | 8 × 8 | N/A |
| 1.7 | Horizontal/Vertical radiating element/sub-array spacing, dh /dv (Note 5) | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V | N/A |
| 1.7a | Number of element rows in sub-array, Msub | 3 | 3 | 3 | N/A | N/A |
| 1.7b | Vertical radiating element spacing in sub-array, dv,sub | 0.7 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V | N/A | N/A |
| 1.7c | Pre-set sub-array down-tilt, θsubtilt (degrees) | 6 | 6 | 6 | N/A | N/A |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 | 2 | N/A |
| 1.9 | Conducted power (before Ohmic loss) per antenna element/sub-array (dBm) (Note 3) | 28 | 28 | 28 | 16 | N/A |
| 1.10 | Base station horizontal coverage range (degrees) | +/-60 | +/-60 | +/-60 | +/-60 | N/A |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 | 90-120 | N/A |
| 1.12 | Mechanical downtilt (degrees) | 0 | 3 | 3 | N/A | N/A |
| 1.13 | Maximum base station output power/sector (e.i.r.p.) (dBm) | 72.28 | 72.28 | 72.28 | 61.53 | N/A |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as in ITU-R M.2101. Note 2: The element gain in row 1.2 includes the loss given in row 1.8 and is per polarization. Note 3: The conducted power per sub-array assumes 4x8x2 sub-arrays (i.e. power per linear ±45º polarized sub-array). Note 4: 4 × 8 means there are 4 vertical and 8 horizontal radiating sub-arrays. Note 5: For the case of 3 elements per sub-array, dv will be 2.1 wavelengths. |  |  |  |  |  |  |

# References

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	R4-2108080, “LS to ITU-R and CEPT on extension of IMT array antenna model to support sub-array structures”, Ericsson.

[3]	R4-2008924, “LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (below 5 GHz)”, Ericsson.