3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404287

Changsha, China, 15 – 19 April 2024

Agenda Item:	9.3.4

Source: 	Nokia

Title: 	IMT parameters for 14800 to 15350 MHz

Document for:	Discussion

# 1.	Introduction

The SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#103 [1]. One of the objectives of this SI is to study the IMT parameters relevant for sharing and compatibility for 14800 to 15350 MHz frequency range.

It is stated in the ‘Justification’ of the approved SID that:

Previous work e.g., SI outcomes on 7 to 24 GHz as captured in TR 38.820 or 6 to 10 GHz parameters as captured in TR 38.921 may be taken to account, as needed.

This contribution discusses the BS antenna parameters from the SI outcomes on 7 to 24 GHz as captured in TR 38.820 [2] and the propagation loss model in TR 36.942 [3].

# 2.	Discussion

The examples of BS antenna topologies in [2] for 7 to 24 GHz are extracted below.

Figure 7.2.3-1: Example RDN mappings

Note that different antenna array size with different number of RDN ports are used as examples. On the other hand, the propagation loss model in TR 36.942 [3] in urban macro area is extracted below.

### 4.5.2 Macro cell propagation model – Urban Area Macro cell propagation model for urban area is applicable for scenarios in urban and suburban areas outside the high rise core where the buildings are of nearly uniform height [3]: where: R is the base station-UE separation in kilometres f is the carrier frequency in MHz Dhb is the base station antenna height in metres, measured from the average rooftop level

Since the carrier frequency in 14800 to 15350 MHz is about double of that in 7125 to 8400 MHz, the propagation loss in 14800 to 15350 MHz in urban macro area will be around 6 dB higher than that in 7125 to 8400 MHz. Therefore, the antenna characteristics in 14800 to 15350 MHz will need to cover around 6 dB propagation loss difference if the same cell coverage as in 7125 to 8400 MHz is targeted. This can be achieved by, e.g., doubling the antenna array size in both horizontal and vertical dimensions, or doubling the antenna array size in one dimension and doubling the TRX chains (each of which connected to one RDN port).

# 3.	Conclusion

This contribution discusses the BS antenna parameters from the SI outcomes on 7 to 24 GHz as captured in TR 38.820 [2] and the propagation loss model in TR 36.942 [3].

It is observed that the antenna characteristics in 14800 to 15350 MHz will need to cover around 6 dB propagation loss difference if the same cell coverage as in 7125 to 8400 MHz is targeted. This can be achieved by, e.g., doubling the antenna array size in both horizontal and vertical dimensions, or doubling the antenna array size in one dimension and doubling the TRX chains (each of which connected to one RDN port).

# References

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	3GPP TR 38.820-1 v16.1.0: “NR; 7 - 24 GHz frequency range”.

[3]	3GPP TR 36.942 v18.0.0: “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.