3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404289

Changsha, China, 15 – 19 April 2024

Agenda Item:	9.14.3

Source: 	Nokia

Title: 	Required changes for BS RF requirements for low-power wake-up signal for NR

Document for:	Discussion

# 1.	Introduction

The revised WI for low-power wake-up signal and receiver for NR was approved at TSG RAN#103 [1]. One of the objectives of this WI is to:

- Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline

This contribution discusses the current NR BS requirements on dynamic range in TS 38.104 [2] and the required changes to support the transmission of LP-WUS/LP-SS for NR.

# 2.	Discussion

Currently, the RE power control dynamic range requirements for NR BS are specified in TS 38.104 as follows:

### 6.3.2 RE power control dynamic range #### 6.3.2.1 General The RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,AC or Pmax,c,TABC) for a specified reference condition. For BS type 1-C this requirement shall apply at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band. #### 6.3.2.2 Minimum requirement for BS type 1-C and BS type 1-H RE power control dynamic range: Table 6.3.2.2-1: RE power control dynamic range Modulation scheme used; RE power control dynamic range (dB). on the RE; (down); (up). QPSK (PDCCH); -6; +4. QPSK (PDSCH); -6; +3. 16QAM (PDSCH); -3; +3. 64QAM (PDSCH); 0; 0. 256QAM (PDSCH); 0; 0. 1024QAM (PDSCH); 0; 0. NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station..

In table 6.3.2.2-1 above, the RE power control dynamic range requirements are specified depending on the modulation and physical channels. Since LP-WUS/LP-SS is a new physical channel using On-Off Keying (OOK) modulation, a new requirement shall be added for the dynamic range for LP-WUS/LP-SS.

It is agreed and recorded in the SI TR 38.869 [3] that:

BS manufacturer could declare if power boosting for WUS signal is supported and the boosting level in the range of [0] to [6] dB is considered. Final power boosting level and the condition of power boosting will be decided based on further analysis in WI phase.

Understandably, it is desirable to specify a higher (up) requirement for LP-WUS/LP-SS as it is used to wake-up the UE in sleep (DRX off) mode. On the other hand, it is undesirable to significantly increase the implementation complexity and energy usage of the NR BS for LP-WUS/LP-SS transmission as the main objective of this new feature is indeed energy saving.

Considering the other NR BS requirements for output power dynamics, currently, the requirement for NB-IoT RB power dynamic range for NB-IoT operation in NR in-band is specified in TS 38.104 as follows:

### 6.3.4 NB-IoT RB power dynamic range for NB-IoT operation in NR in-band #### 6.3.4.1 General The NB-IoT RB power dynamic range (or NB-IoT power boosting) is the difference between the average power of NB-IoT REs (which occupy certain REs within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel) and the average power over all REs (from both NB-IoT and the NR carrier containing the NB-IoT REs). #### 6.3.4.2 Minimum Requirement NB-IoT RB power dynamic range for NB-IoT operation in NR in-band shall be larger than or equal to the level specified in Table 6.3.4.2-1. This power dynamic range level is only required for one NB-IoT RB. Table 6.3.4.2-1: NB-IoT RB power dynamic range for NB-IoT operation in NR in-band BS channel bandwidth (MHz); NB-IoT RB frequency position; NB-IoT RB power dynamic range (dB). 5, 10; Any; +6. 15; Within center 77*180kHz+15kHz at each edge; +6. Other; +3. 20; Within center 102*180kHz+15kHz at each edge; +6. Other; +3. 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; Within center 90% of BS channel bandwidth; +6. Other; +3.

In table 6.3.4.2-1, the minimum NB-IoT RB power dynamic range is specified to be +6 dB when the NB-IoT RB frequency position is within the center part of the BS channel bandwidth, and +3 dB otherwise.

Since conditions such as LP-WUS bandwidth and supported M value (OOK-1 and/or OOK-4) will have a big impact on the new dynamic range requirement value(s) for LP-WUS/LP-SS, the new requirement should be specified after RAN1 has decided those conditions.

# 3.	Conclusion

This contribution has discussed the current NR BS requirements on dynamic range in TS 38.104 [2]. A new dynamic range requirement shall be added in TS 38.104 for LP-WUS/LP-SS, where the requirement value(s) should be specified after conditions such as LP-WUS/LP-SS transmission bandwidth and supported M value (OOK-1 and/or OOK-4) have been decided in RAN1.

# References

[1]	RP-240801, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony.

[2]	3GPP TS 38.104 v18.5.0: “NR; Base Station (BS) radio transmission and reception”.

[3]	3GPP TR 38.869 v18.0.0: “Study on low-power wake-up signal and receiver for NR”.