3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2404294

Changsha, China, Apr 15-19, 2024

Agenda item:	6.11.2.2

Source:	Apple

Title:	Simulation results for MU-MIMO with advanced receiver

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#108 RAN4 concluded the Phase 1/ study on advanced receiver for MU-MIMO and the details of the study were captured in TR 38.878. In RAN4#110 the UE capability signaling and parameters for defining requirements in phase 2 were discussed and WF [1] was agreed. In this contribution we present our simulation results for Phase 2 of this WI.

# 2. Simulation Results

In [1] we agreed on some simulation assumptions for further evaluation for defining requirements for MU-MIMO with advanced receiver.

## Simulation results when MO is signaled (DCI index 1~5)

For the agreed simulation assumptions in [1], in Tables 1 and 2 we provide the simulation results with co-scheduled UE modulation order signaled for FDD and TDD modes respectively.

Table 1: Results with MO signaled for FDD

| Case Number | Rank Combo | Precoder selection | MIMO | Channel Model | Antenna config/ corr | MCS | MO of co- UE | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | R-ML | MMSE-IRC |
| 1 | 1+1 | Random | 2T2R | TDLC300-100 | 2x2 ULA medium | MCS 13 | QPSK | 14.3 | 25.8 |
| 2 |  | Orthogonal |  |  |  |  |  | 13.5 | 20.5 |
| 3 |  | Random | 2T4R |  |  |  |  | 12.7 | 24.6 |
| 4 |  | Orthogonal |  |  |  |  |  | 11.8 | 19.8 |
| 5 | 2+2 | Orthogonal | 4T4R | TDLA30-10 | ULA Low |  |  | 10.5 | 13.2 |
| 6 |  |  |  |  | XP medium |  |  | 11.7 | 14.8 |
| 7 |  |  |  |  | ULA Low | MCS 17 | 16QAM | 16.3 | 17.8 |
| 8 |  |  |  |  | XP medium |  |  | 17.9 | 19.7 |
| 9 |  |  |  |  | XP medium |  | QPSK | 15.3 | 19.7 |

Table 2: Results with MO signaled for TDD

| Case Number | Rank Combo | Precoder selection | MIMO | Channel Model | Antenna config/ corr | MCS | MO of co- UE | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | R-ML | MMSE-IRC |
| 10 | 1+1 | Random | 2T2R | TDLC300-100 | ULA medium | MCS 13 | QPSK | 14.6 | 29.5 |
| 11 |  | Orthogonal |  |  |  |  |  | 13.7 | 21.3 |
| 12 |  | Random | 2T4R |  |  |  |  | 13.2 | 27.5 |
| 13 |  | Orthogonal |  |  |  |  |  | 12.1 | 20.7 |
| 14 | 2+2 | Orthogonal | 4T4R | TDLA30-10 | ULA Low |  |  | 10.4 | 13.2 |
| 15 |  |  |  |  | XP medium |  |  | 11.6 | 14.8 |
| 16 |  |  |  |  | ULA Low | MCS 17 | 16QAM | 16.2 | 17.7 |
| 17 |  |  |  |  | XP medium |  |  | 17.8 | 19.7 |
| 18 |  |  |  |  | XP medium |  | QPSK | 15.1 | 19.7 |

Observations based on the results:

- For all cases evaluated R-ML performance is better than baseline MMSE-IRC

- For 4x4 cases the performance gain of R-ML for ULA -Low and XP medium antenna correlation are comparable

- The performance is improved by ~ 1dB with orthogonal precoder compared to random precoder for 1+1 with medium ant correlation.

For all cases evaluated R-ML performance is better than baseline MMSE-IRC

For 4x4 cases the performance gain of R-ML for ULA -Low and XP medium antenna correlation are comparable

The performance is improved by ~ 1dB with orthogonal precoder compared to random precoder for 1+1 with medium ant correlation.

## Simulation results when MO is not signaled (DCI index 6)

For the simulation assumptions agreed in [1] we provide results with blind modulation order detection for FDD and TDD in tables below.

Table 3: Results with blind detection of MO for FDD

| Case Number | Rank Combo | Precoder selection | MIMO | Channel Model | Antenna correlation | MCS | MO of co- UE | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | R-ML | MMSE-IRC |
| 19 | 1+1 | Random | 2T2R | TDLC300-100 | ULA medium | MCS 13 | QPSK | 19.9 | 25.8 |
| 20 |  | Orthogonal |  |  |  |  |  | 17.9 | 20.5 |
| 21 |  | Random |  |  | ULA Low |  |  | 14.1 | 15.0 |
| 22 |  | Random | 2T4R |  | ULA medium |  |  | 19.6 | 24.6 |
| 23 |  | Orthogonal |  |  |  |  |  | 17.6 | 19.8 |
| 24 |  | Random |  | TDLA30-10 | ULA Low |  |  | 7.3 | 7.3 |
| 25 |  | Random | 2T2R | TDLC300-100 | ULA medium | MCS 17 | 16QAM | 24.8 | N/A |
| 26 |  | Orthogonal |  |  |  |  |  | 22.6 | N/A |
| 27 |  | Random |  |  | ULA Low |  |  | 18.3 | N/A |
| 28 |  | Random | 2T4R |  | ULA medium |  |  | 24.3 | N/A |
| 29 |  | Orthogonal |  |  |  |  |  | 22.0 | N/A |
| 30 |  | Random |  | TDLA30-10 | ULA Low |  |  | 10.9 | 10.9 |
| 31 | 2+2 | Orthogonal | 4T4R |  |  | MCS 13 | QPSK | 12.6 | 13.2 |
| 32 |  |  |  |  | XP medium |  |  | 14.2 | 14.8 |

Table 4: Results with blind detection of MO for TDD

| Case Number | Rank Combo | Precoder selection | MIMO | Channel Model | Antenna correlation | MCS | MO of co- UE | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | R-ML | MMSE-IRC |
| 33 | 1+1 | Random | 2T2R | TDLC300-100 | ULA medium | MCS 13 | QPSK | 20.1 | 29.5 |
| 34 |  | Orthogonal |  |  |  |  |  | 18.1 | 21.3 |
| 35 |  | Random |  |  | ULA Low |  |  | 14.2 | 15.4 |
| 36 |  | Random | 2T4R |  | ULA medium |  |  | 19.9 | 27.5 |
| 37 |  | Orthogonal |  |  |  |  |  | 17.8 | 20.7 |
| 38 |  | Random |  | TDLA30-10 | ULA Low |  |  | 7.2 | 7.2 |
| 39 |  | Random | 2T2R | TDLC300-100 | ULA medium | MCS 17 | 16QAM | 25.3 | N/A |
| 40 |  | Orthogonal |  |  |  |  |  | 22.9 | N/A |
| 41 |  | Random |  |  | ULA Low |  |  | 18.8 | N/A |
| 42 |  | Random | 2T4R |  | ULA medium |  |  | 25.0 | N/A |
| 43 |  | Orthogonal |  |  |  |  |  | 22.3 | N/A |
| 44 |  | Random |  | TDLA30-10 | ULA Low |  |  | 10.9 | 10.9 |
| 45 | 2+2 | Orthogonal | 4T4R |  |  | MCS 13 | QPSK | 12.5 | 13.2 |
| 46 |  |  |  |  | XP medium |  |  | 14.1 | 14.8 |

Observations:

- For 1+1 with TDLA channel, Low antenna correlation, the performance with R-ML is very close to MMSE-IRC

- For 1+1 with either TDLC channel or Medium antenna correlation the performance of R-ML is better than MMSE-IRC across both MCS13+QPSK and MCS17+16QAM

- Performance with orthogonal precoder is significantly better than random precoder

- For 2+2 performance with R-ML with BD-MO is not significantly better than MMSE-IRC

For 1+1 with TDLA channel, Low antenna correlation, the performance with R-ML is very close to MMSE-IRC

For 1+1 with either TDLC channel or Medium antenna correlation the performance of R-ML is better than MMSE-IRC across both MCS13+QPSK and MCS17+16QAM

Performance with orthogonal precoder is significantly better than random precoder

For 2+2 performance with R-ML with BD-MO is not significantly better than MMSE-IRC

# 3. Conclusion

In this paper, we provide simulation results for requirements with advanced receiver for MU-MIMO. Our observations are captured below:

- For all cases evaluated R-ML performance is better than baseline MMSE-IRC

- For 4x4 cases the performance gain of R-ML for ULA -Low and XP medium antenna correlation are comparable

- The performance is improved by ~ 1dB with orthogonal precoder compared to random precoder for 1+1 with medium ant correlation.

- For 1+1 with TDLA channel, Low antenna correlation, the performance with R-ML is very close to MMSE-IRC

- For 1+1 with either TDLC channel or Medium antenna correlation the performance of R-ML is better than MMSE-IRC across both MCS13+QPSK and MCS17+16QAM

- Performance with orthogonal precoder is significantly better than random precoder

- For 2+2 performance with R-ML with BD-MO is not significantly better than MMSE-IRC

# Reference

- R4-2402864, “Way forward on [110][319] NR_demod_enh3_Part1”, China Telecom.