3GPP TSG-RAN4 Meeting #110Bis	R4-2404301

# Changsha, China, Apr 15 - Apr 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for active TCI state list update requirements for eUTCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In RAN4#110 we agreed on CR R4-2400451 for updating the active TCI state list update for UTCI. The same update is needed for eUTCI requirements. - For some case there are requirements for all TCI states are unknown – this is not agreed; Based on R17 UTCI requirements, they need to be removed - For mDCI UL active TCI state list the requirements account for serving cell and cell with differnet PCI. In case f mDCI requirements they are specified per TRP, so its either serving cell or cell with different PCI, cannot be a combination of both |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated active TCI list switching delay requirements inline with CR R4-2400451 - Deleted requirements when all TCI states are unknown. - For mDCI UL active TCI state list deleted any delay depending on combination of serving cell and cell with diff PCI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eUTCI requirements for active TCI state list update will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.21.5, 8.22.5, 8.23.5, 8.24.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change 1

### 8.21.5	Active Downlink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates more than one target separate TCIs, and at least one DL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates more than one target joint TCI.

If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n + THARQ +3Nslotsubframe,µ +  TOk*(Tfirst-SSB_List + TSSB-proc) / NR slot length.

If a subset of the target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP_list +TOuk*(Tfirst-SSB_List+ TSSB-proc)) / NR slot length.

If all target TCI states in the active TCI state list are unknown, the requirements specified in this clause are not applicable.

Where

-	TL1-RSRP_List is the longest L1 measurement time (TL1-RSRP) of the source RS among the unknown target TCI states, where TL1-RSRP is specified in clause 8.21.3

-	Tfirst-SSB_List is the Tfirst-SSB from serving cell

-	THARQ, Tfirst-SSB, TSSB-proc , TOk, TOuk are defined in clause 8.21.3. TSSB is the SSB periodicity.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state list update.

Change 2

### 8.22.5	Active Downlink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates more than one target separate TCIs, and at least one DL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates more than one target joint TCI.

If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n + THARQ +3Nslotsubframe,µ +  TOk*(Tfirst-SSB_List + TSSB-proc) / NR slot length.

If a subset of the target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP_list +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length.

If all target TCI states in the active TCI state list are unknown, the requirements specified in this clause are not applicable.

Editor’s note: FFS how to take into account overlapping MAC-CE based switches from two TRPs where the SSB associated to each TCI state is overlapping or adjacent to each other.

Where

-	TL1-RSRP_List is the longest L1 measurement time (TL1-RSRP) of the source RS among the unknown target TCI states, where TL1-RSRP is specified in clause 8.22.3

Editor’s note: FFS the L1-RSRP measurement requirement when RTD>CP.

-	THARQ, Tfirst-SSB, TSSB-proc, TOk, TOuk are defined in clause 8.22.3. TSSB is the SSB periodicity.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state list update.

Change 3

### 8.23.5	Active uplink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates more than one target separate TCIs, and at least one DL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates more than one target joint TCI.

If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n + THARQ + 3Nslotsubframe,µ + NM * (Tfirst_target-PL-RS + 4 * Ttarget_PL-RS + 2ms) / NR slot length.

If a subset of the target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n+THARQ + 3Nslotsubframe,µ+ (TL1-RSRP_List + Tfirst_target-PL-RS + 4*Ttarget_PL-RS+ 2ms) / NR slot length.

If all target TCI states in the active TCI state list are unknown, the requirements specified in this clause are not applicable.

Where

-	TL1-RSRP_List is the longest L1 measurement time (TL1-RSRP) of the source RS among the unknown target TCI states, where TL1-RSRP is specified in clause 8.23.2

-	THARQ, Tfirst_target-PL-RS, Ttarget-PL-RS are defined in clause 8.23.3.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to transmit on UL based on the target TCI before UE completes the DL and UL TCI list update.

The requirements in this clause are applicable when the source RS of the active UL TCI state is a DL-RS and this DL-RS is included as one of the source RSs in the DL active TCI list.

Change 4

### 8.24.5	Active Uplink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates more than one target separate TCIs, and at least one UL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates more than one target joint TCI.

If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n + THARQ + 3Nslotsubframe,µ + NM * (Tfirst_target-PL-RS_List + 4 * Ttarget_PL-RS_List + 2ms) / NR slot length,

FFS on additional time tracking of DL Ref RS for 2TA

If a subset of target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n+THARQ + 3Nslotsubframe,µ+ (TL1-RSRP_List + Tfirst_target-PL-RS_List + 4*Ttarget_PL-RS_List + 2ms) / NR slot length

FFS on additional time tracking of DL Ref RS for 2TA

If all target TCI states in the active TCI state list are unknown, the requirements specified in this clause are not applicable.

Where

-	If all TCIs are known,

-	if the target PL-RS associated with or included in any UL TCI is not maintained, NM = 1;  Where maintained PL-RS is defined in clause 8.16.3

-	if a subset of target TCI states in the active TCI state list are unknown,

-	TL1-RSRP_List = is the longest L1 measurement time (TL1-RSRP) of the source RS among the unknown target TCI states , where TL1-RSRP is specified in clause 8.16.3

-	Otherwise,

-	Tfirst_target-PL-RS_List = Tfirst_target-PL-RS_SC

-	Ttarget-PL-RS_List = Ttarget-PL-RS_SC.

-	Tfirst_target-PL-RS_SC is Tfirst_target-PL-RS for serving cell

-	Tfirst_target-PL-RS_CDP is Tfirst_target-PL-RS for cell with different PCI from serving cell

-	Ttarget-PL-RS_SC is Ttarget-PL-RS for serving cell

-		THARQ, Tfirst_target-PL-RS, Ttarget-PL-RS are defined in clause 8.16.3.

The requirements specified in this clause do not apply if more than 4 different RSs are activated as PL-RS per serving cell among all active UL (or joint) TCI states.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to transmit on UL based on the target TCI before UE completes the DL and UL TCI list update.

The requirements in this clause are applicable when the source RS of the active UL TCI state is a DL-RS and this DL-RS is included as one of the source RSs in the DL active TCI list.