3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2404307

Changsha, China, Apr 15-19, 2024

Agenda item:	6.25.1

Source:	Apple

Title:	UE demodulation performance requirements for NR dynamic spectrum sharing

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#110 requirements for enhanced DSS were discussed and way forward [1] were agreed.  In this contribution we present our views on demodulation requirements for eDSS.

# 2. Discussion

In RAN4#110 the demodulation requirements for eDSS were discussed and the following agreements were reached [1]:

Antenna configuration Agreement: - 2x4, 4x2 for both FDD and TDD Channel bandwidth Agreement: - For TDD, consider 10MHz/15kHz Aggregation level Agreement: - 4, 8 Channel model Agreement: - TDLA30-10 and TDLC300-100 Payload Agreement: - Remain 39 bits

Based on the simulation assumptions and evaluation cases agreed in [1] we provide simulation results for performance evaluation.

Table 1: PDCCH performance evaluation results in eDSS

| Test number | Duplex | BW (MHz) | CORESET RB | CORESET duration | AL | Propagation Condition | Antenna configuration and correlation Matrix | SNR @ 1% Pm-dsg |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Alignment | Impairment |
| 1 | FDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | -0.9 | 2.1 |
| 2 | FDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | -3.0 | -1.0 |
| 3 | TDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | -0.9 | 2.1 |
| 4 | TDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | -3.0 | -1.0 |

# 3. Conclusion

In this paper, we provide simulation requirements for UE demodulation requirements for DSS enhancements.

# Reference

[1].  R4-2402873,	 “Way forward on [110][328] NR_DSS_enh”, Ericsson.