3GPP TSG-RAN WG4 Meeting #110bis                                                          R4-2404332

# Changsha, China, April 15 – April 19, 2024

Agenda item:	6.24.4

Source:	Apple Inc.

Title:	Summary of Simulation Results for Network Energy Savings for NR

Release:	Rel-18

Document for:	Discussion

# 1. Simulation Results

The focus of this simulation exercise is to analyze 	“the performance of SSB-less SCells CA with Rel-15 compliant demodulator implementation (without TRS based TOE) and decide, if the observed performance constitutes a practical operating point of the feature”

Below are the simulation results obtained, and in the next section, we remind the simulation assumptions used for this exercise.

The simulation duration is 1000 slots and the time offset compensation converges quickly in a few tenths of slots. Regardless of a fixed RTD between PCell and Scell of {-0.9, -0.4, 0, +0.4, +0.9} of the length of the cyclic prefix, the results are as expected.

Despite an initial TO, the system is able to fully correct TO hence virtually no loss in throughput at steady state.

# 2. Simulation Assumptions

Our observations and proposals are summarized below:

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| Channel model |  |  | {TDLA30-10} |
| Antenna configuration |  |  | 2x2, ULA Low |
| Fraction of maximum throughput (%) |  |  | 70 (Only SCell measured) |
| SCS/CBW |  |  | 30kHz/40MHz (106 PRB) |
| MCS |  |  | {16QAM, 0.48 (MCS13)} |
| Channel estimation |  |  | DM-RS based: non-ideal/practical |
| TO Estimation & Compensation |  |  | As per implementation |
| TO |  |  | {-0.9, -0.4, 0, 0.4, 0.9} CP |
| FFT window shift target |  |  | {As per implementation} |
| SSB/TRS |  |  | PCell: SSB and TRS (optional) SCell: TRS only |
| Precoder |  |  | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Fill remaining DL symbols in the slot. 12 in normal DL slot 4 in special slot |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  | Antenna ports |  | {1000} |
| Number of HARQ Processes |  |  | 8 |
| Maximum number of HARQ transmissions |  |  | 4 |
| TDD UL-DL pattern |  |  | [30kHz SCS: FR1.30-1] |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  | 'No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity | Slots | 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |

# References

- R4-2402872	, “WF on [109][330] Netw_Energy_NR_demod”, Huawei.

- R4-2318353, “Discussion on Network energy saving for NR UE demodulation performance and CSI requirements”, Nokia, Nokia Shanghai Bell.