3GPP TSG-RAN WG4 Meeting #110bis 	R4-2404334

# Changsha, China, February 26 – March 1, 2024

Agenda item:	6.8.7.1

Source:	Apple Inc.

Title:	Summary of Simulation Results for NR support for dedicated spectrum less than 5MHz for FR1

Release:	Rel-18

Document for:	Discussion

# 1. Summary

In this contribution we present the simulation results obtained of the performance of Dedicated Spectrum less than 5MHz, more specifically, considering a 3MHz channel bandwidth. With a 3MHz channel bandwidth, the SSB is reduced to 12 PRBs, while CORESET#0 is reduced to 15PRBs. In both cases, puncturing of the original signals takes place so only minimal standardization modifications are needed.

# 2. PBCH Results

For During the last RAN4#110 meeting the following agreements were confirmed as per the previous WF [1]. Effectively, these agreements closed the PDSCH requirements agenda item, and no new requirements were added.

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low, | 1 | -6.3 |
|  |  |  |  | 1 x 4 Low | 1 | -8.6 |

# 3. PDCCH Requirements

For PDCCH, the following agreement was captured in the previous WF [1]:

| Number of Tx | BW (MHz) | CORESET RB | CORESET duration | Aggr. Level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLA30-10 | 1x2 Low | 1 | [TBA] |
| 2 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLC300-100 | 2x2 Low | 1 | [TBA] |
| 1 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLA30-10 | 1x4 Low | 1 | [TBA] |
| 1 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLC300-100 | 2x4 Low | 1 | [TBA] |

# References

- R4-2321064, “WF on NR_FR1_lessthan_5MHz_BW_demod”, Nokia.