3GPP TSG-RAN WG4 Meeting #110bis	                                                      R4-2404400

Changsha, China, 15 April – 19 April, 2024

Title:	Discussion on the maintenance issues for the support for bandwidth part operation without restriction

Source:	CATT

Agenda Item:	6.6.1

Document for:	Discussion

1 Introduction

In last meeting, the RRM requirements for bandwidth part operation without restriction were discussed and the conclusions were captured in the approved WF [1]. In this paper, we further discuss the maintenance issue and present our views.

2 Discussion

The remaining issues for core part maintenance are as below:

#### Issue 1-1: Requirements/UE behaviour for UE supporting both option B-1-1 and A - FFS following options - Option 1a: - For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour. - Option 1b: - For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario. - Option 2: - For UE supporting both option B-1-1 and A, if CSI-RS is configured within the active BWP for L1 measurements, then UE is NOT expected to perform RLM/BFD/BM based on CD-SSB outside active BWP. - It is clarified in the SSB based L1 measurement requirements that if UE supports both option B-1-1 and option A then the UE is not required to measure the SSB if it is outside active BWP. #### Issue 1-3: Requirements/UE behaviour for UE supporting both option C and B-1-1 - FFS following options - Option 1: - For UE supporting both option B-1-1 and C, if both CD-SSB and NCD-SSB are configured outside active BWP, then UE is expected to perform RLM/BFD/BM following NW configuration based on TS 38.331. No additional requirements will be defined for such scenario. - Option 2: - For UE supporting both option B-1-1 and C, if the active BWP that does not contain CD- or NCD-SSB, UE should perform L1 measurement on CD-SSB. No additional requirements will be defined for such scenario. #### Issue 1-4: Applicability of NCD-SSB based L1 and L3 intra-frequency measurement requirements - FFS: NCD-SSB related requirements also apply for PSCell when SCG deactivation is not enabled for the UE. - FFS requirements are specified for EN-DC only or for both EN-DC and NR-DC. #### Issue 1-6: Requirements for L3 intra-f measurement without gaps for option C - FFS following option - Option 1: - Add one bullet in section 9.2.1 that the target measurement SSB is completely contained in the active downlink BWP of the UE provided the UE supports [FG53-3] and servingCellMO is present in the corresponding BWP-DownlinkDedicated.

For UE behavior/requirements when supporting both option A and option B-1-1, we think in such case, UE can perform both measurement with CD-SSB outside active BWP and CSI-RS measurement within active BWP. The existing requirements still apply and no additional requirements need to be defined, i.e., option 1b is reasonable. But considering that similar as the case UE supporting both option A and option C, performing one of the measurement is enough, leveraging the conclusion of issue 1-2 i.e., option 1a is also acceptable.

For UE supporting both option B-1-1 and option C, the conclusion of issue 1-2 can also be leveraged.

Proposal 1: For UE supporting both option A and B-1-1, option 1a and 1b are both acceptable.

Proposal 2: For UE supporting both option C and B-1-1, UE shall perform L1 measurement according to network configuration, and there is no need to define additional requirements/ UE behaviour.

Regarding the applicability of NCD-SSB based L1 and L3 intra-frequency measurement requirements, since we already had agreements and operator comments in last meeting that DC is not priority use case, we think we should respect the previous agreements and not to apply the NCD-SSB based requirements to PSCell.

Proposal 3: NCD-SSB related requirements are not applicable to PSCell.

3 Summary

In this paper, we have some further discussions on the remaining issues of bandwidth part operation without restriction, and the following proposals are given:

Proposal 1: For UE supporting both option A and B-1-1, option 1a and 1b are both acceptable.

Proposal 2: For UE supporting both option C and B-1-1, UE shall perform L1 measurement according to network configuration, and there is no need to define additional requirements/ UE behaviour.

Proposal 3: NCD-SSB related requirements are not applicable to PSCell.

4 References

- R4-2403544, WF on core maintenance and performance part for BWP operation without restriction, vivo, Vodafone, RAN4#110

5 Annex

3GPP TSG-RAN WG4 Meeting #110bis	                                                       R4-24xxxxx

Changsha, China, 15 April – 19 April, 2024

Title:	Reply LS on BWP operation without restriction

Response to:	R4-2400021(R2-2313951)

Release:	Rel-18

Work Item:	NR_BWP_wor-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name:                  Qiuge Guo

E-mail Address:   guoqiuge@CATT.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 thanks RAN2 for the reply LS R4-2400021(R2-2313951) on BWP with restriction.

Regarding RAN2 question that whether NCD-SSB based requirements are applicable for PSCell for UE supporting option C and configured with CA, RAN4 conclude that it is not applicable for PSCell.

RAN4 kindly ask RAN2 to take the above information into account.

2. Actions:

To RAN WG2:

RAN4 kindly ask RAN2 to take the above conclusion into account.

3. Date of Next TSG-RAN Meetings:

TSG RAN WG4 Meeting #111		May 20 – May 24, 2024		Fukuoka, Japan

TSG RAN WG4 Meeting #112		August 19 –August 23, 2024		Maastricht, Netherlands