# 3GPP TSG-RAN WG4 Meeting #110bis	                                                       R4-2404401

# Changsha, China, 15 April – 19 April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.133 | CR | DraftCR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TC set 1&2) DraftCR on test cases for L1 and intra-frequency measurement without gap for option B-1-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BWP_wor-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The feature for the support of bandwidth part operation without restriction is introduced in Rel-18 and the corresponding test cases should be clarified for option B-1-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the test cases for L1 and intra-frequency measurement without gap for option B-1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case for L1 and intra-frequency measurement without gap for option B-1-1 is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.3.x	Testing principle for UE supporting option B-1-1 for bandwidth part operation without restriction

A.3.x.1	Introduction

For the test cases listed in table A.3.x.1-1, if UE supports bwpOperationMeasWithoutInterrupt-r18, the starting PRB index of the SSB can be any possible PRB index of the RF channel BW occurring after the last PRB of the DL active BWP.

Table A.3.x.1-1: Test cases applicable to UE supporting B-1-1 for bandwidth part operation without restriction

| Test category | Section | Test case |
| --- | --- | --- |
| RLM measurements | A.4.5.1.1 | Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode |
|  | A.4.5.1.2 | Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode |
|  | A.4.5.1.3 | Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode |
|  | A.4.5.1.4 | Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode |
|  | A.5.5.1.1 | Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode |
|  | A.5.5.1.2 | Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode |
|  | A.5.5.1.3 | Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode |
|  | A.5.5.1.4 | Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode |
|  | A.6.5.1.1 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode |
|  | A.6.5.1.2 | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode |
|  | A.6.5.1.3 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode |
|  | A.6.5.1.4 | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode |
|  | A.7.5.1.1 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode |
|  | A.7.5.1.2 | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode |
|  | A.7.5.1.3 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode |
|  | A.7.5.1.4 | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode |
| BFD measurements | A.4.5.5.1 | EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode |
|  | A.4.5.5.2 | EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode |
|  | A.5.5.5.1 | EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode |
|  | A.5.5.5.2 | EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode |
|  | A.6.5.5.1 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode |
|  | A.6.5.5.2 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode |
|  | A.7.5.5.1 | EN-DC Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode |
|  | A.7.5.5.2 | EN-DC Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode |
| L1-RSRP measurements | A.4.6.4.1 | SSB based L1-RSRP measurement when DRX is not used |
|  | A.4.6.4.2 | SSB based L1-RSRP measurement when DRX is used |
|  | A.5.6.3.1 | SSB based L1-RSRP measurement when DRX is not used |
|  | A.5.6.3.2 | SSB based L1-RSRP measurement when DRX is used |
|  | A.6.6.4.1 | SSB based L1-RSRP measurement when DRX is not used |
|  | A.6.6.4.2 | SSB based L1-RSRP measurement when DRX is used |
|  | A.7.6.3.1 | SSB based L1-RSRP measurement when DRX is not used |
|  | A.7.6.3.2 | SSB based L1-RSRP measurement when DRX is used |
| L1-SINR measurements | A.4.6.7.2 | L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used |
|  | A.5.6.6.2 | L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used |
|  | A.6.6.8.2 | L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used |
|  | A.7.6.6.2 | L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used |
| Intra-frequency measurements without gaps | A.4.6.1.1 | EN-DC event triggered reporting tests without gap under non-DRX |
|  | A.4.6.1.2 | EN-DC event triggered reporting tests without gap under DRX |
|  | A.4.6.1.5 | EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading |
|  | A.5.6.1.1 | EN-DC event triggered reporting test without gap under non-DRX |
|  | A.5.6.1.2 | EN-DC event triggered reporting test without gap under DRX |
|  | A.6.6.1.1 | SA event triggered reporting tests without gap under non-DRX |
|  | A.6.6.1.2 | SA event triggered reporting tests without gap under DRX |
|  | A.6.6.1.5 | SA event triggered reporting tests without gap under non-DRX with SSB index reading |
|  | A.7.6.1.1 | SA event triggered reporting test without gap under non-DRX |
|  | A.7.6.1.2 | SA event triggered reporting test without gap under DRX |

<End of Change 1>