3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2404412

# Changsha, CN, 15th Apr – 19st Apr, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PDCCH ordered based RACH for LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In PDCCH ordered RACH delay requirements defined in RAN1, TSSB, TBWPswitchDelay and ∆RF/BB preparation are referred to TS 38.133. However, they haven’t been specified in TS 38.133 yet. RAN4 has already agreed to introduced them into TS 38.133 in RAN4#110 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introudce TSSB and ∆RF/BB preparation for PDCCH ordered RACH delay requirements in TS 38.133. - Introduce the definition of TSSB and further clarification for the value of TSSB in clause 8.X.2. - Not to introduce the term of TBWPswitchDelay as the previous agreement in clause 8.X.2. - Introduce the definition of ∆RF/BB_preparation in clause 8.X.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDCCH ordered RACH delay requirements for LTM would still be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 8.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.X	PDCCH ordered based RACH delay for LTM candidate cell

### 8.X.1	Introduction

The requirements in this clause apply for a UE capable of [FG 45-5: RACH-based early TA acquisition] configured to trigger RACH procedure toward target LTM candidate cell before receiving cell switch command MAC-CE.

### 8.X.2	PDCCH ordered based RACH delay

UE shall complete the RACH transmission within the delay defined in TS38.213 [3] clause 8.1, in which TSSB and ∆RF/BB preparation are defined as the following:

-	TSSB:

-	For target LTM candidate cell is in FR1, TSSB = 0 if the following conditions are met.

- UE has received MAC-CE to activate TCI of target candicate cell and the time between receiving the MAC-CE activating the target TCI state and the PDCCH order for RACH is at least [THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length, where THARQ, TOk, Tfirst-SSB and TSSB-proc are as stated in section 8.15.3], and

- The L1-RSRP measurement period defined in clause 9.14 and 9.15 respectively for intra-frequency and inter-frequency is less than or equal to 160ms.

Otherwise,

- TSSB is the time to first SSB transmission after PDCCH-order RACH command is decoded by the UE when SSB is within active BWP

- TSSB is the time to first SSB transmission overlapped with MGL after PDCCH-order RACH command is decoded by the UE when SSB is outside active BWP.

-	For target LTM candidate cell is in FR2, TSSB is always as following:

- TSSB is the time to first SSB transmission after PDCCH-order RACH command is decoded by the UE when SSB is within active BWP

- TSSB is the time to first SSB transmission overlapped with MGL after PDCCH-order RACH command is decoded by the UE when SSB is outside active BWP.

-	∆RF/BB preparation: If PRACH bandwidth is outside active UL BWP but within one of configured UL BWPs of any active serving cell, ∆RF/BB preparation is the DCI based BWP switching delay specified in clause 8.6. If PRACH bandwidth is not within any of the configured UL BWPs of any active serving cell, ∆RF/BB preparation equals to [UE capability of [1ms, 3ms, 5ms, 10ms]]. Otherwise, ∆RF/BB preparation = 0.

<End of Change 1>