3GPP TSG-RAN WG4 Meeting #110bis                                                                                        R4-2404445

Changsha, China, April 15 – April 19, 2024

Source: 	CATT

Title: 	Simulation results for SAN demodulation requirements for above 10 GHz bands

Agenda item:	6.16.8.1

Document for:	Approval

- Introduction

In RAN4#110 meeting, the simulation assumption on demodulation requirements for NR NTN enhancements was agreed [1].

This contribution provides initial simulation results for SAN demodulation requirements for alignment.

- Discussion

Based on the agreed simulation assumptions [1], presented below are our preliminary simulation outcomes for PUSCH and PRACH concerning SAN demodulation:

2.1 Ka-band PUSCH

PUSCH with DFT-S-OFM

The simulation results for PUSCH with DFT-s-OFDM are shown in Table2.1-1.

Table 2.1-1: Simulation results for PUSCH with DFT-s-OFDM, Type B, 50 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-3000 | 70 % | 2 | pos1 | -2.8 |
|  | 2 | Normal | NTN-TDLC5-3000 | 70 % | 2 | pos1 | [TBD] |

2.2 Ka-band PRACH

The simulation results for PRACH missed detection are shown in Table 2.2-1.

Table 2.2-1: Simulation results for PRACH missed detection, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex G) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format B4 | Burst format C2 |
| 1 | 1 | AWGN | 0 | -15.9 | -13.0 |
|  |  | NTN-TDLC5 Low | 3000 Hz | -12.6 | -10.3 |
|  | 2 | AWGN | 0 | [TBD] | [TBD] |
|  |  | NTN-TDLC5 Low | 3000 Hz | [TBD] | [TBD] |

- Conclusion

This contribution provides our simulation results for SAN demodulation requirement for above 10GHz bands for alignment.

- References

- R4-2401719, Simulation assumption on demodulation requirements for NR NTN enhancements, Huawei, HiSilicon, RAN4#110