3GPP TSG-RAN WG4 Meeting #110bis	R4-2404450

Changsha, China, April 15th ‒ 19th, 2024

Title: 	Further discussion on power class applicability

Agenda Item:	11.2

Source: 	CATT

Document for:	Approval

# 1 Introduction

The persistent concerns regarding the ambiguity surrounding power class specifications have been a topic of discussion in numerous meetings, including the most recent RAN4 meeting [1]. The ambiguity leads to a structural incompleteness both in RAN2 and RAN4 specs. Despite efforts, only a limited consensus was reached during the discussion [2]. The root cause of this confusion lies in the existence of multiple power class capabilities that a UE can convey through either single-band or band combination signalling. Therefore, there is a need for clarification regarding the applicability of these power class capabilities.

Generally, it is beneficial to elucidate the applicable power class concerning the following scenarios:

- Case #A: UL comprises a single CC, which may include the following sub-cases:

  - Case #A-1: (1CC DL + 1CC UL, signalling based on band combination reporting): DL consists of only one CC, yet a UE may convey band-related capabilities via band combination-based signalling where the band combination contains only one band.

  - Case #A-2: (intra-band multiple CCs DL + 1CC UL, i.e., intra-band DL-only CA): DL contains multiple CCs within the same band as UL CC, which may be contiguous or non-contiguous.

  - Case #A-3: (inter-band multiple CCs DL + 1CC UL, i.e., inter-band DL-only CA): DL comprises multiple CCs across two or more bands.

- Case #B: (intra or inter-band multiple CCs DL + intra-band 2CCs UL)

  - Case #B-1: 2 UL CCs are contiguous, i.e., intra-band contiguous UL CA + intra or inter-band DL CA.

  - Case #B-2: 2 UL CCs are non-contiguous, i.e., intra-band non-contiguous UL CA + intra- or inter-band DL CA.

- Case #C: (inter-band multiple CCs DL + inter-band 2CCs UL)

Furthermore, it is necessary to clarify the fallback behaviour as well. These matters are addressed in this contribution.

# 2 Discussion

For Case #A-1, RAN4#110 has made the following agreement [2]:

- For UE that is configured in the single carrier mode (1 DL + 1 UL on this band), the power class is determined by ue-PowerClass for this NR band.

And accordingly, we propose the revision reflecting this agreement in our companion draft CR submitted to this meeting [3].

Proposal 1: RAN4 to endorse the draft CR reflecting the agreement on Case #A-1.

For Case #A-2, though there is only one UL CC, however, power class capability for this UL CC can be reported either via single band based signaling, i.e., ue-PowerClass, or band combination based signaling, i.e., PowerClass, or ue-PowerClassPerBandPerBC-r17. In this case, the overriding order if all of them are reported should be: ue-PowerClassPerBandPerBC-r17 > PowerClass > ue-PowerClass.

Proposal 2: In Case #A-2 (intra-band DL only CA), the power class of the sole uplink component carrier is determined according to the priority sequence of reported ue-PowerClassPerBandPerBC-r17, PowerClass, and ue-PowerClass.

For Case #A-3, where DL is aggregated by multiple CCs from different bands but there is still only one UL CC, similar to Case #A-2, the same principle can be applied since all of three power class capabilities can be reported.

Proposal 3: In Case #A-3 (inter-band DL only CA), the power class of the sole uplink component carrier is determined according to the priority sequence of reported ue-PowerClassPerBandPerBC-r17, PowerClass, and ue-PowerClass.

For Case #B-1 where UL comprises two contiguous CCs, and there could be only one ue-PowerClassPerBandPerBC-r17 reported, and only one PowerClass in the band combination. Both of them can be different from the power class ue-PowerClass in the single band operation, and still the same principle similar to Case #A-2 may apply in this case.

Proposal 4: In Case #B-1 (intra/inter-band DL CA, intra-band contiguous UL CA), the power class of the two uplink component carriers is determined according to the priority sequence of reported ue-PowerClassPerBandPerBC-r17, PowerClass, and ue-PowerClass.

For Case #B-2 where UL comprises two non-contiguous CCs, and there could be two ue-PowerClassPerBandPerBC-r17 reported, and of course only maximum one PowerClass in the band combination. The two shall be identical, i.e., either both not reported, or the same value reported. However, if a different power class from ue-PowerClass is required in this case, band combination power class i.e., PowerClass can be reported, hence a simple way is not to allow a UE to report ue-PowerClassPerBandPerBC-r17. Besides the restriction, the similar principle applies.

Proposal 5: In Case #B-2 (intra/inter-band DL CA, intra-band non-contiguous UL CA), ue-PowerClassPerBandPerBC-r17 is not applicable, and the power class of the two uplink component carriers is determined according to the priority sequence of reported PowerClass, and ue-PowerClass.

Finally for Case #C, where DL comprises multiple CCs from different bands, and UL comprises two UL CCs from different bands also, it is quite clear that the same principle applies.

Proposal 6: In Case #B-3 (inter-band DL CA, inter-band UL CA), the power class of each uplink component carriers is determined according to the priority sequence of ue-PowerClassPerBandPerBC-r17, PowerClass, and ue-PowerClass reported for the corresponding band.

# 3 Conclusion

In this contribution, we have the following proposals for power class applicability:

Proposal 1: RAN4 to endorse the draft CR reflecting the agreement on Case #A-1.

Proposal 2: In Case #A-2 (intra-band DL only CA), the power class of the sole uplink component carrier is determined according to the priority sequence of reported ue-PowerClassPerBandPerBC-r17, PowerClass, and ue-PowerClass.

Proposal 3: In Case #A-3 (inter-band DL only CA), the power class of the sole uplink component carrier is determined according to the priority sequence of reported ue-PowerClassPerBandPerBC-r17, PowerClass, and ue-PowerClass.

Proposal 4: In Case #B-1 (intra/inter-band DL CA, intra-band contiguous UL CA), the power class of the two uplink component carriers is determined according to the priority sequence of reported ue-PowerClassPerBandPerBC-r17, PowerClass, and ue-PowerClass.

Proposal 5: In Case #B-2 (intra/inter-band DL CA, intra-band non-contiguous UL CA), ue-PowerClassPerBandPerBC-r17 is not applicable, and the power class of the two uplink component carriers is determined according to the priority sequence of reported PowerClass, and ue-PowerClass.

Proposal 6: In Case #B-3 (inter-band DL CA, inter-band UL CA), the power class of each uplink component carriers is determined according to the priority sequence of ue-PowerClassPerBandPerBC-r17, PowerClass, and ue-PowerClass reported for the corresponding band.

# Reference

- R4-2401102, Topic summary for [110][143] NR_power_class, Moderator(Samsung)

- R4-2403891, WF on NR UE power classes, Huawei

- R4-240xxxx, draftCR on power class applicability, CATT