3GPP TSG-RAN WG4 Meeting #110bis                                                                                        R4-2404461

Changsha, China, April 15 – April 19, 2024

Source: 	CATT

Title: 	Discussion on BS RF requirement for expected EIRP mask for upper 6GHz

Agenda item:	9.2.2

Document for:	Approval

- Introduction

New WI on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved in RAN#103. This contribution discusses the BS RF requirement for expected EIRP mask for upper 6GHz.

- Discussion

The following expected EIRP requirements are copied from [2], the requirements are the expected e.i.r.p requirements for the base station operating in the frequency range 6425-7125 MHz to protect the FSS in 6425-7 075 MHz.

| Vertical angle range θL ≤ θ < θH (vertical angle θ above horizon) | Expected e.i.r.p. (dBm/MHz) (See NOTES 1, 2 and 3) |
| --- | --- |
| 0 ≤ θ < 5 | 27 |
| 5 ≤ θ < 10 | 23 |
| 10≤ θ < 15 | 19 |
| 15≤ θ < 20 | 18 |
| 20≤ θ <30 | 16 |
| 30≤ θ < 60 | 15 |
| 60≤ θ ≤ 90 | 15 |
| NOTE 1: The expected e.i.r.p. is defined as the average value of the e.i.r.p., with the averaging being performed: ‒ over horizontal angles from −180 to +180, with the IMT base station beamforming in a specific direction within its horizontal and vertical steering range, ‒ over different beamforming directions within the IMT base station horizontal and vertical steering range, and ‒ over the specified vertical angle range θL ≤ θ < θH. NOTE 2: An IMT base station shall comply with the specified limits on expected e.i.r.p. spectral density for all mechanical tilts with which it can be deployed, taking into account considering m). NOTE 3: See the Annex to this Resolution for additional details on how the expected e.i.r.p. can be calculated for this frequency band. |  |

The followings need to be discussed to define the corresponding BS RF requirements definition.

Table 1: Issues to define the BS RF requirement for expected EIRP mask for upper 6GHz

| Issues | CATT view |
| --- | --- |
| How to introduce the e.i.r.p limitation concept in specification | Introduce the concept in the table of the requirements |
| What’s the frequency range for this requirement | 6425-7 075 MHz |
| Which section is used to add the requirements | Create a new clause 9.2.2.1 A |
| What’s the tolerance | Reuse ±2.2 dB for EIRP requirement |
| How to handle the note 2 “for all mechanical tilts” in the specification | FFS how to relate to the 3GPP directional requirement |

- Conclusion

This contribution provides our preliminary views on how to define the BS RF requirement for expected EIRP mask for upper 6GHz. We provide our views in Table 1.

- References

[1] RP-240829, “New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, RAN#103

[2] WRC-23, Resolution COM4/7 (WRC-23), Terrestrial component of International Mobile Telecommunications (IMT)
 within the frequency band 6 425-7 125 MHz.