3GPP TSG-RAN4 Meeting #110-bis	R4-2404468

# Changsha, China, 15th Apr 2024 – 19th Apr 2024

Title:	TP to TR 36.718-02-01 Addition of CA_3-7-40 and CA_3-7-40-40

Source:	Samsung, Spark NZ Ltd

Agenda item:	7.1.3.1

Document for:	Approval

# 1	Introduction

This TP is to add CA_3A-7A-40A, CA_3A-7A-40C, CA_3A-7A-40D, and CA_3A-7A-40A-40A with dual UL. CA_3A-7A-40A, CA_3A-7A-40C, CA_3A-7A-40D, and CA_3A-7A-40A-40A with single UL have been specified. All fallbacks are included in the latest spec or big CR (R4-2402620) agreed in RAN4#110.

************************************* Start of TP*****************************************

### 5.4.x	CA_3-7-40, CA_3-7-40-40

#### 5.4.x.1	Channel bandwidths per operating band for CA

Table 5.4.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 5.4.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-7A-40A | CA_3A-7A CA_3A-40A CA_7A-40A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-7A-40A-40A | CA_3A-7A CA_3A-40A CA_7A-40A | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_3A-7A-40C | CA_3A-7A CA_3A-40A CA_7A-40A | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-7A-40D | CA_3A-7A CA_3A-40A CA_7A-40A | 3 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 | See CA_40D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

#### 5.4.x.2	Co-existence studies

Based on the co-existence studies, it can be observed:

No IMD issue caused by 3+7 falls into band 40 Rx;

No IMD issue caused by 3+40 falls into band 7 Rx;

IMD5 issue caused by 7+40 falls into band 3 Rx;

#### 5.4.x.3	∆TIB and ∆RIB values

Already included in TS 36.101.

#### 5.4.x.4	REFSENS Requirements

Based on the co-existence studies there is a need to define additional REFSENS requirements. MSD value and test points are taken from DC_3A-7A_n40A.

Table 5.3.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_3A-7A-40A CA_3A-7A-40C CA_3A-7A-40D CA_3A-7A-40A-40A | 3 | N/A | 5 | N/A | 1866.6 | 3.4 | FDD | IMD5 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | FDD | N/A |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |

************************************* End of TP*****************************************

# 2	References

[1] RP-240414_Revised WID: Rel-18 LTE-Advanced Carrier Aggregation for x bands (2<=x<= 6) DL with y bands (y=1, 2) UL