3GPP TSG-RAN4 Meeting #110-bis	R4-2404475

# Changsha, China, 15th Apr 2024 – 19th Apr 2024

Source:	Samsung, TELUS, Bell Mobility

Title:	TP for TR 38.718-03-01 to include CA_n7-n66-n71

Agenda item:			5.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-03-01 to include CA_n7-n66-n71.

All, except CA_n7A-n71A (BCS4 and 5) and CA_n7A-n66A (BCS4 and 5), PC3 fallbacks have been specified. TP to add PC3 CA_n7A-n71A is proposed in this meeting as R4-2404472. Draft CR to add PC3 CA_n7A-n66A is proposed in this meeting as R4-2404471.

# 2. Text Proposal

# ---Start of changes---

## 5.x	CA_n7-n66-n71

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: 3DL Inter-band CA operating bands

| NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n7+n66+n71

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n66A-n71A | CA_n7A-n66A CA_n7A-n71A CA_n66A-n71A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c of CA_n7-n66-n71 are already defined in the spec TS 38.101-1.

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

Based on the co-existence studies, it can be observed:

No IMD issue caused by n7+n66 falls into band n71 Rx;

No IMD issue caused by n7+n71 falls into band n66 Rx;

No IMD issue caused by n66+n71 falls into band n7 Rx;

#### 5.x.2.2	REFSENS requirements

Based on co-existence studies on 5.x.2.1, no need to define exceptional REFSENS requirements.

---End of changes---

# Reference

[1] RP-240166, Revised WID: Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2), ZTE