3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404487

Changsha, CN, April 15 – April 19, 2024

Agenda Item: 11.1.2

Source: MediaTek Inc.

Title: Views on spec quality improvement for TS38.133

Document for: Discussion

# Introduction

In RP#103 meeting, one WF [1] was endorsed for RAN4 spec quality improvement. Details of the WF are captured as below

- The RAN4 Rel-19 specifications are expected to be available by December 2024. - RAN4 will organize the discussions for improving the specifications in Q2 and Q3 2024 in RAN4 meeting(s), and report to RAN#104 and RAN#105 - Focus on 38.133 and 38.101-1/38.101-2/38.101-3, not covering other specifications in this RAN task - Motivation of the work: - Try to improve the above specifications for Rel-19 for 5G in the short term - Try to conclude on guidance including the structure, drafting rule to ensure the quality of specifications for UE RF and RRM. - Set up one dedicated agenda to collect the input from companies for specification improvement - Companies are expected to point out the key issues and also provide the concrete solutions. - No corresponding CR is expected before September - Schedule the specific time slot for the single discussions on the specification improvement in RAN4 main session starting from April - Identify the key issues and root reasons behind - Summarize the candidate solutions for the next action - Further discuss and decide how to capture the outcome of this RAN task in RAN#105

In this paper, we provide our view to TS 38.133. In all the discussions below, we take version 18.4.0 as the example. We try to point out the issues we found in current spec and the potential solutions. Nevertheless, we are open to all possible solutions that can help to improve spec quality.

# Discussion

## 2.1 Hierarchy of indent

The first issue we want to raise is the hierarchy of indent. Let’s take “Section 8.3.2 SCell Activation Delay Requirement for Deactivated SCell” as an example. Part of the current spec paragraphs are captured as below.

Our understanding of the structure should be,

- The yellow and green highlighted parts should be at the same level.

- The blue part is a condition under the green.

- The purple part is very ambiguous. The red part already has its otherwise counterpart in gray. However, the purple part should not be the otherwise counterpart of the blue, because they have essentially the same condition.

Similar issues also happen in section 8.1.2.2, 9.2.1, 9.3.1, 9.5.4, … etc. In our understanding, this issues usually happen when a certain scenario has too many conditions. Some of its conditions may be shared to other scenarios and some are not. It is very easy to make mistakes, especially when people tend to use ‘otherwise’ to skip mentioning the conditions again. For some scenarios come in the later paragraphs of a section, people need to trace back all previous paragraphs to understand the overall conditions. This is not straightforward and easily leads to misunderstanding.

Observation 1: In current TS38.133, the hierarchy of indent for some requirements are wrong and require improvement.

In our view, to improve the spec quality, there are some approaches RAN4 can consider.

- Avoid specify requirements with too many conditions, e.g., avoid optimizing for corner cases.

- Learn from the RRC procedure spec 38.331 by using similar hierarchy as 1>, 2>, 3>, … (An example is provided below by copying from Section 5.2.2.2.1 of TS38.331)

Proposal 1: RAN4 to avoid defining requirement with a length list of conditions.

Proposal 2: RAN4 to develop a similar hierarchy as RRC procedure in TS38.331 to clarify the hierarchy structure of the requirement.

## 2.2 Positive list or Negative list?

In current TS38.133 spec, sometimes we use a positive list to indicate that the requirement is only applicable in a certain scenario. This implicitly means other scenarios that are not mentioned should have no requirement. However, sometimes we also use negative list to indicate that the requirement is not applicable to a certain scenario. This means requirements are automatically applicable to those scenarios that are not mentioned.

There are also some requirements which have both positive and negative lists in the same section. (For example: Section 8.1.7.3, as captured below) It is a bit confusing about how one can interpret those scenarios that are not mentioned. Particularly for scheduling availability, it is even more confusing to see Section 3.6.9 adding some ‘assumptions’ for the requirements. E.g., if those assumptions are not met, whether we allow scheduling restriction or not?

Observation 2: In current TS38.133, some requirements which have both positive and negative lists in the same section.

To improve the spec quality, we would prefer to only capture requirements with positive list. In other words, those scenarios not mentioned in the applicability rule have no requirement. This can ensure that RAN4 only introduces requirements that are discussed and avoid automatically including those requirements that never discussed.

Proposal 3: RAN4 to capture the applicability of scenarios only with a positive list.

## 2.3 Requirement duplication

One of a very classic example of requirements duplication regarding the IDLE mode relaxation for Redcap. The navigation page is captured below for reference. Although Sections 4.2B.2.9, 4.2B.2.10 and 4.2B.2.11 are for intra-freq, inter-freq and inter-RAT respectively, their sub-section titles are almost the same.

After checking into individual sections, we understand that some sub-sections are almost the same or with a slight difference, e.g., Section 4.2B.2.10.4 is almost the same as 4.2B.2.11.4, except the referred section for requirements. Section 4.2B.2.9.4 is only with a slight difference in the condition of the 2nd bullet.

Similar issues also happen in the calculation of Kp for all L1 measurement requirements (RLM, BFD, CBD, L1-RSRP, ICBM, LTM measurements), scheduling restriction, measurement restrictions. There are more than 95% duplications with perhaps some tiny differences which are very difficult to be identified for people not involved in introducing the requirements.

We understand the advantage of duplication is that for people who care only a particular scenario, he/she only needs to check one section without bothered by other irrelevant scenarios. However, this brings a huge maintenance effort. For example, it is not very easy to tell whether a difference is made by an agreement or by mistake. Also, when companies identify one error in one requirement (e.g., in Rel-15), companies also need to bring not only cate A CRs but also other Cate F CRs for later releases if the requirements were duplicated from Rel-15 (e.g., Rel-16 NR-U, Rel-17 Redcap, Rel-18 LTM all have the Kp calculation copied and modified from Rel-15.)

Observation 3: Current 38.133 has a lot of duplication of requirements, which is difficult to tell whether/where the difference is and also increases the effort of maintenance

Perhaps it is too late to re-structure the spec to resolve the duplication issue at this stage. Nevertheless, we would prefer to document the difference somewhere (e.g., the introduction section) when people duplicate and modify the legacy requirements to a new section.

Proposal 4: When duplicating and modifying the legacy requirements to a new section, document the key difference to the legacy requirements [in the introduction section].

## 2.4 Approaches to ensure spec consistency

So far new changes to TS38.133 is triggered by company CRs. This is an efficient approach to distribute the work to multiple delegates and avoid adding huge workload only to a spec editor. However, this also leads to the problem of spec consistency because different sections are now drafted by different companies. It is not easy to guarantee the wording, indent, order of paragraphs mathematic symbols/equations and abbreviations are consistent across all sections.

Also, due to the current high workload of RRM session, delegates have no sufficient time to check CRs. E.g., the agreement may be achieved on Friday afternoon, leaving very little time for CR implementation and checking. In the online discussion, there are too many documents to be returned to on Friday. The time we can spent on a CR becomes very limited. Even for offline discussions, because one RRM delegate usually needs to handle multiple topics, there is sometime no sufficient time to sit down together with a group of people to work on CRs.

Observation 4: The spec quality is low due to insufficient time for CR checking during the meeting week.

One way to resolve this is to invite some section editors to share the load of spec editor. The section editor can check all CRs which brings changes to his/her section. However, this may not work because one CR may contain changes across different sections. The section editor has to scan all CRs first. The workload of the section editor is still very high. And this may also mean we need longer post-meeting period for consistency checking, which seems difficult to meet the typical 2-week gap between WG and plenary meetings.

In our view, one possible approach to provide more time for CR checking is to preserve a certain time for CR preparation on Friday. Take one example. This means all open issues need to be concluded (or decided to comeback in next meeting) by Thursday. Then on Friday, delegates can focus on how to implement the agreement into CRs.

Proposal 5: Preserve sufficient time for delegates to review CRs on Friday.

# Summary

In this paper, we provided our view on the current issue of TS38.133 and how to resolve it. We have the following observations and proposals. We are also open to another solutions.

Observation 1: In current TS38.133, the hierarchy of indent for some requirements are wrong and require improvement.

Proposal 1: RAN4 to avoid defining requirement with a length list of conditions.

Proposal 2: RAN4 to develop a similar hierarchy as RRC procedure in TS38.331 to clarify the hierarchy structure of the requirement.

Observation 2: In current TS38.133, some requirements which have both positive and negative lists in the same section.

Proposal 3: RAN4 to capture the applicability of scenarios only with a positive list.

Observation 3: Current 38.133 has a lot of duplication of requirements, which is difficult to tell whether/where the difference is and also increases the effort of maintenance

Proposal 4: When duplicating and modifying the legacy requirements to a new section, document the key difference to the legacy requirements [in the introduction section].

Observation 4: The spec quality is low due to insufficient time for CR checking during the meeting week.

Proposal 5: Preserve sufficient time for delegates to review CRs on Friday.

# 4 Reference

RP-240782, “Moderator's summary on RAN4 spec quality”, RAN4 chair (Huawei)