3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2404490

Changsha, Hunan Province, CN, Apr 15 – Apr 19, 2024

Agenda Item: 6.3.3.2

Source: MediaTek Inc.

Title: Discussion on accuracy requirement for multi-Rx UEs

Document for: Discussion

# Introduction

According to WF [1], there is still an open issue about measurement accuracy for multi-RX. In this paper, we provide our views.

# Discussion

## 2.1 Accuracy requirements for multi-RX

### Accuracy requirements Issue 2-10: Accuracy requirements for multi-Rx in Rel-18 FFS following options - Option 1a: - The legacy accuracy requirements for L1-RSRP measurement apply for L1-RSRP measurements with group-based beam reporting. - Option 1b: - The legacy accuracy requirements in section 10.1.20 of TS 38.133 apply for L1-RSRP measurements under multi-rx operation, with a clarification that multi-rx chain L1-RSRP accuracy requirements apply for FR2-1. - No new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation. - Option 1c: - Under multi-Rx chain DL reception scenario, the legacy accuracy requirements for L1-RSRP measurement should be satisfied for both Rx chains. - In this sense, no new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation - Option 2: - Do not define group-based L1-RSRP measurement accuracy requirements unless the requirements are applicable only when there are negligible mutual interferences across different pair of Tx-Rx beams.

For issue 2-10, we support option 1a, 1b and 1c to reuse the legacy accuracy requirement in clause 10.1.20 of TS 38.133. In our understanding, UE’s RF components and baseband RSRP estimation algorithm are the same regardless UE is operating under single or multiple Rx model. The only extra thing to do is probably adding a clarification for multi-RX chain L1-RSRP accuracy requirement and apply for FR2-1.

Proposal 1: Legacy accuracy requirements in the clause 10.1.20 of TS 38.133 apply to multi-Rx UEs.

# Summary

In this paper, the discussion of R18 multi-Rx chains is provided. We have the following proposal:

Proposal 1: Legacy accuracy requirements in the clause 10.1.20 of TS 38.133 apply to multi-Rx UEs.

# Reference

- R4-2403560 WF on core maintenance and performance part for multi-RX, Vivo, Ericsson RAN4 #110